3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2206080

Electronic Meeting, February 21st- March 3rd, 2022

Agenda Item: 10.20.3.1.3

Source: MediaTek Inc.

Title: UE complexity reduction impact on timing requirements

Document for: Discussion

# Introduction

In this contribution paper, we discuss the remaining open issues on timing requirements for RedCap from the way forward (WF) of the meeting RAN4#101bis-e [1]:

Condition for meeting UE transmit timing requirements Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms is agreeable: - Option 1 (E///, HW, CMCC, NOkia ): SSB refers to CD-SSB or any of CS- and NCD-SSB - Option 2 (Apple, QC): SSB has to be in active BWP - Option 3 (MTK): SSB refers to both CD-SSB and NCD-SSB and SSB shall be in the active BWP. Whether CSI-RS/TRS can be used for meeting the timing requirements - Option 1 (Xiaomi, MTK, OPPO, Nokia, QC, vivo, Xiaomi, Apple, MTK, E///): CSI-RS/TRS is not needed to acquire the reference cell timing in Rel-17. - Option 2 (HW, CMCC): - CSI-RS can be used for RedCap UEs to acquire the reference cell timing depending on UE capability. - No CSI-RS based timing requirements are defined the CSI-RS in R17.

# Discussion on timing requirements

This section provides discussion on the remaining open issues from timing requirements.

# Condition for meeting UE transmit timing requirements

Given that NCD-SSB is supported by RedCap UE, hence the RAN4 requirements should be adapted to capture this new feature. Furthermore, it is necessary to capture that the SSB (either CD-SSB or NCD-SSB) has to be in the active BWP. Now, the provided options 1 and 2 in the above open issues are not contradicting each other and in fact these two options shall be combined in a single option as given in option 3. Therefore, we support option 3 and option 2 because these options specify that the SSB has to be in active BWP.

- Option 1 and 2 are not contradicting each other and both shall be captured as in option 3.

- Support Option 2: SSB has to be in active BWP, and Option 3: SSB refers to both CD-SSB and NCD-SSB and SSB shall be in the active BWP.

# Whether CSI-RS/TRS can be used for meeting the timing requirements

From the WF of the previous meeting [1], there is an open issue regarding whether to use CSI-RS when no SSB in active BWP to acquire timing. To our understanding this proposal is not needed because this feature was not implemented in general 5G NR devices. In addition, CSI-RS is not a standalone signal as described in clause 9.10.2.5 (excerpt below) and hence it can’t be used for acquiring timing for neighbouring cells. Besides, the TRS signal is sent after the connection is established so it is feasible for fine tracking, however, it can’t be used to acquire initial timing. There fore, to our understanding that using CSI-RS/TRS to acquire timing is not feasible. Thus, we support option 1 from the open issue.

‘for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, or if CSI-RS is configured with associated SSB but not QCL-ed to the associated SSB, the UE is not required to monitor the corresponding CSI-RS resource’

- Don’t support the use of CSI-RS/TRS to acquire the reference cell timing in RedCap UEs.

- Support Option 1: CSI-RS/TRS is not needed to acquire the reference cell timing in Rel-17.

# Summary

In this contribution, discussion on timing requirement in RedCap UEs are provided and we have the following observations:

Observation 1: Option 1 and 2 are not contradicting each other and both shall be captured as in option 3.

Also, we have the following proposals:

Proposal 1: Support Option 2: SSB has to be in active BWP, and Option 3: SSB refers to both CD-SSB and NCD-SSB and SSB shall be in the active BWP.

Proposal 2: Don’t support the use of CSI-RS/TRS to acquire the reference cell timing in RedCap UEs.

Proposal 3: Support Option 1: CSI-RS/TRS is not needed to acquire the reference cell timing in Rel-17.

# References

- RP-2202670, “WF on RedCap RRM requirements”, Ericsson.