3GPP TSG RAN WG4 Meeting #102-e		R4-2206101

Electronic meeting

February 21st ‒ March 3rd, 2022

Title:	Reply LS on configuration of p-MaxEUTRA and p-NR-FR1

Response to:	R4-2200011

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	[Qualcomm Incorporated]

To:	TSG RAN WG1, TSG RAN WG2, TSG RAN WG5

Cc:

Contact person:	Gene Fong

gfong[at]qti[dot]qualcomm[dot]com

Attachments:	N/A

# 1	Overall description

RAN4 appreciates the opportunity to provide its view on the optionality of IEs p-MaxEUTRA and p-NR-FR1 when the UE is configured for EN-DC.  In the RAN4 specification TS 38.101-1 sub-clause 6.2B.4, the IEs p-MaxEUTRA and p-NR-FR1 appear in the configured maximum output power upper and lower limits for dual connectivity.  These IEs may limit the configured maximum output power for the UE when configured by the network.  It is RAN4’s understanding that these IE’s may be configured in specific network deployment scenarios where the maximum output power of the UE must be limited below what it may otherwise be capable of transmitting.  Hence, it is RAN4’s view that the IEs are optional and configured by the network as needed.  In case the IEs are not configured, it is assumed that no such limit applies and the configured maximum output power is determined by the remainder of the parameters from TS 38.101-3 sub-clause 6.2B.4.  In other words, if the IEs are not configured, it is assumed that they take on the value of “infinity” in the configured maximum output power equations.

# 2	Actions

To TSG RAN WG1, TSG RAN WG2, TSG RAN WG5

ACTION: 	RAN4 respectfully requests RAN1, RAN2, and RAN5 to take the above into consideration and inform RAN4 if it has a different understanding

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #103-e		16st – 27th, May 2022			Electronic meeting

TSG RAN WG4 Meeting #104		22nd – 26th, August 2022			Toulouse, FR