3GPP TSG-RAN WG4 #102-e	R4-2206102

February 21st ‒ March 3rd, 2022

Electronic Meeting

Agenda item:	9.3.2

Source: 	Qualcomm Incorporated

Title: 	Channel raster and sync raster for the 6 GHz licensed band

Document for:	Discussion

# Introduction

The technical work to define a 6 GHz licensed band has been initiated in [1] based on an input on regulatory requirements in [2].  In [3], two options were presented for channel raster to either follow the legacy numbering and resolution of NR-ARFCN or to specify the raster based on 5 MHz frequency blocks.

# Discussion

## Channel raster

According to the information provided in [2], the frequency plan in the 6425 – 7125 MHz band consists of 35 reference frequency blocks of 20 MHz each, but the bandwidth for each block could be extended or reduced by 10 MHz.  Therefore, frequency blocks can be 10, 20, or 30 MHz wide with center frequency addressable in 5 MHz multiples.  On the other hand, the global frequency raster for NR provides a resolution ΔFGlobal of 15 kHz.  Since 5 MHz is not evenly divisible by 15 kHz, it will not be possible to exactly align the NR channel raster with the channelization indicated in [2].   The closest ARFCN raster that aligns with 5 MHz increments is given by the following sequence

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXYZ | 15 | 828667 – <333, 334> – 874667 | 828667 – <333, 334> – 874667 |

The ARFCN available for this band as follows

ARFCN0 = 828667

ARFCNn+1 = ARFCNn + 333 if n mod 3 ≠ 0

ARFCNn+1 = ARFCNn + 334 if n mod 3 = 0

ARFCN138 = 8874667

Where n = {0, 1, 2, … 138}.

In this manner, the first ARFCN for the band corresponds to 6430 MHz center frequency since this is the first possible 10 MHz frequency block according to the rules provided in [2].  Similarly, the last ARFCN corresponds to 7120 MHz center frequency which is the last possible 10 MHz frequency block.  Intermediate values are spaced by 5 MHz as closely as possible with a ΔFRaster of 15 kHz.  The largest error in center frequency is ± 5 kHz which is smaller than a single subcarrier and there is no bias in the error over the set of channel numbers.

## Sync raster

It is proposed to specify a 30 kHz SCS for the SSB with block pattern C for this band since the minimum channel bandwidth has been agreed to be 10 MHz and is therefore capable of supporting 30 kHz SCS.  In principle the resolution of the sync raster should be aligned with the channelization since the SSB should reside inside the channel, but there should also be flexibility in locating the SSB within the channel.  Additionally, the sync raster should be minimized in order to reduce the impact on UE search.  Since the band extends over 700 MHz, the search range can be extensive.  Therefore, it is proposed to downsample the sync raster in a similar manner with the channel raster to 5 MHz resolution.  If the SSB is located off this raster, the UE search may not be able to locate it; instead, its location should be explicitly signaled.  The raster entries are FFS.

# Conclusion

The channel raster for the 6 GHz licensed band should conform to the channelization described in [2].  In other words, the channel raster should be specified at 5 MHz resolution to accommodate frequency blocks of 10, 20, and 30 MHz which may be aggregated together for wider channels.  The sync raster should also be compatible with this channelization, especially considering the extent of this band and the impact on UE cell search.

# Reference

- RP-213696, “Revised WID: Introduction of 6GHz NR licensed bands,” Huawei, HiSilicon

- RP-213605, “Liaison statement to 3GPP TSG RAN on the inclusion of the 6425-7125 MHz frequency range in the 3GPP specification for 5G-NR/IMT-2020 systems,” Regional Commonwealth in the Field of Communications.

- R4-2202266, “WF on system parameters for 6GHz licensed band,” ZTE