3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2206127

# Electronic Meeting, 21st February – 3rd March, 2022

Title:	6GHz licensed band system parameters

Agenda Item:	9.3.2

Source:	MediaTek Inc.

Document for:		Discussion

# Introduction

This document considers some of the design requirements for operation in the 6425-7125MHz range, in line with the RCC recommendations in [1].

# Channel and sync raster options

Frequency plan

- 35 reference frequency blocks of 20 MHz each

- The bandwidth of a frequency block could be increased in relation to the reference block by multiples of 10 MHz

- The central frequency of a frequency block can be shifted in relation to the central frequency of a reference block by multiples of 5 MHz

- Minimum channel bandwidth of 20MHz, as agreed at R4#101bis-e as 20MHz

Below, we indicate the potential channel and sync raster if we consider the above parameters as the minimum for the band.

Channel raster minimum requirements from RCC

- Channel raster: 5MHz is required (closest is: 4995kHz for 15kHz SCS, 4980kHz for 30kHz SCS)

- However it needs to be considered as to whether more flexibility is required.

Sync raster minimum requirements from RCC

- GSCN raster based on 5MHz channel raster:

  - SCS = 30kHz or 15kHz

  - Spectrum utilization: 51 RB (30kHz), 106 RB (15kHz)

  - SSB = 20 RBs

  - CORESET#0 of 48 RBs with offset values defined in TS38.213 for 10MHz bandwidth (4320kHz from edge)

  - GSCN spacing: 7.5MHz (30kHz SCS) ideal

    - As multiples of current 1.44MHz raster => 7.2MHz (30kHz SCS).

Reduction in cell search effort: 1 – (97/486 GSCNs) = 0.8 => ~80% reduction compared to 1.44MHz sync raster with step size of 1.

Other considerations for sync raster

If more flexibility in the channel raster is required, this may lead to a more granular sync raster. Also if in future more flexibility was required in other regions, this may lead to more flexibility being needed too.

# Conclusion

Proposal: Consider the sync and channel raster aspects in this document for further discussion.

# References

[1] LS from RCC on 6GHz band