3GPP TSG-RAN WG4 Meeting #102-e                                                    	R4-2206263

Online, 21st February – 3rd March, 2022

Source:	Vodafone

Title:	TP for TR 37.717-51-11: DC_1-3-7-8-32_n78

Agenda item:			9.18.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-51-11 to include DC_1-3-7-8-32_n78.

The TIB,c and RIB values provided are derived from the relevant subsets already present in 36101 and 38101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.X	DC_1-3-7-8-32_n78

#### 5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (six bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-7A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |

#### 5.1.X.2	∆TIB and ∆RIB values

<Editor’s note: rows in ΔTIB,c andΔRIB tables shall be deleted if ΔTIB,c andΔRIB,c values are zero. >

Table 5.1.X.2-1: ΔTIB,c due to EN-DC (six bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-8-32_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.X.2-2: ΔRIB,c due to EN-DC (six bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-8-32_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |

<End of Text Proposal>