3GPP TSG-RAN WG4 Meeting # 102-e												     R4-2206394

Electronic Meeting, 1st February – 3rd March, 2022

Source: 	Skyworks Solutions Inc., Qualcomm Incorporated.

Title: 	WF on capturing triple beat MSD in 38.101-1 and 38.101-3

Agenda Item:	9.6

Document for:	Approval

# Background – Motivation for Way Forward

- Band combinations with 3 ULCCs were analysed in [1,2,3]

- 3 active UL CCs create triple beat distortion and can lead to RX de-sensitization.

- The table below captures the uplink configurations for which triple beat IMD products are generated.

  - Eligible REL-17 band combinations from [1][2] that were detected to create triple beat distortion are highlighted in yellow in table below.

  - Combinations highlighted in orange are not eligible uplink configurations for REL-17 and should not be requested or specified [4].

  - Triple beat MSD test point for uplink 25A_n41C is not needed since this is configuration is not requested in REL-17.

| 3A_n7B | 3A_n41C | 3A_n78(2A) | 7C_n28A | 41C_n3A |
| --- | --- | --- | --- | --- |
| 28A_n7B | 3C_n1A | 3C_n77A | 7C_n78A | 41C_n77A |
| 1A_n7B | 3C_n5A | 3C_n78A | 8A_n79C | 41C_n78A |
| 1C_n3A | 3C_n7A | 3C_n79A | 25A_n41C | 41C_n79A |
| 2C_n41A | 3C_n28A | 7C_n1A | 30_n77(2A) | 42C_n28A |
| 2C_n71A | 3C_n41A | 7C_n3A | 39C_n41A | 42C_n3A |
| 2A_n77(2A) | 3A_n77(2A) | 7C_n5A | 40C_n78A | 66_n77(2A) |

- This WF brings triple MSD test points for two-band CA or EN-DC combinations. It should be noted however that there may be several three-band combinations for which a 1st order triple beat product may fall into the third DL band. Companies are therefore encouraged to further explore eligible three-band REL-17 combination candidates.

- Background on Detection, Analysis, Previous Framework for Specification can be found in [5]

- Way Forward on Test point guidelines can be found in [5].

# WF on Capturing Triple Beat MSD in 38.101-1 and 38.101-3

- ULCA band (TDD or FDD) Parameters

    - Must have non-contiguous RB allocation in UL LCRB (ie. 1RB + 1RB)

    - Duplex mode (i.e.. FDD or TDD)

    - Spacing between RB allocations in each CC must equal the FDD Victim band duplex offset

    - UL CC1 BW and UL CC1 Fc

    - UL CC2 BW and UL CC2 Fc

- Non-ULCA FDD band Parameters

    - DL Fc and UL Fc

    - TX LCRB

    - RX BW

    - TX BW

- Triple Beat Order

    - 1st order triple beat is for 3rd order non-linearity

    - In Release-17, capture only MSD test point due to 1st order triple beat interference

- Capturing MSD test points into 38.101-3

    - New tables are not needed: Triple beat MSD test points to be captured in existing tables for MSD for intermodulation interference due to dual uplink operation, e.g., capture in Table 7.3B.2.3.5.1-1 for two-band PC3 combinations, etc...

    - Since only 1st order triple beat are considered, the MSD test points should be captured as “IMD3” in column entitled “IMD order”,

    - Since this only two companies have made contributions, the values in [3] are proposed in brackets to allow companies to further check – see table below. The DC_3A_n41C MSD test point is captured in CR [6].

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_3A_n41C | 3 | 1782.5 | 5 | 25 | 1877.5 | [13.4] | FDD | IMD3 |
|  | n41C | 2555 2635 | [80] [80] | [1 (RBstart=88)] [1 (RBstart=128)] | 2555 2635 | N/A | TDD | N/A |
| DC_8A_n79C | 8 | 912.5 | 5 | 25 | 957.5 | [0] | FDD | IMD3 |
|  | n79C | 4545 4645 | 100 100 | 1 (RBstart=212) 1 (RBstart=60) | 4545 4645 | N/A | TDD | N/A |

- Capturing MSD test points into 38.101-1

    - New tables are not needed: Triple Beat MSD test points to to be captured in existing tables for Reference sensitivity exceptions due to intermodulation interference due to 2UL CA, e.g., capture in Table 7.3A.5-1 for 2UL/2DL PC3 combinations, etc...

    - Since only 1st order triple beat are considered, the MSD test points should be captured as “IMD3” in column entitled “IMD order”

    - The table below is an example of this WF agreement for eligible 2UL/2DL PC3 combinations.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_nX-nYC | nX | Fc_UL_nX | nX_CBW | xx | Fc_DL_nX | [XX] | FDD | IMD3 |
|  | nYC | Fc_nY_UL_CC1 Fc_nY_UL_CC2 | [nY-CC1_CBW] [nY-CC2_CBW] | LCRB_CC1_nY (RBstart=CC1_nY) LCRB_CC2_nY (RBstart=CC2_nY) | Fc_nY_DL_CC1 Fc_nY_DL_CC2 | N/A | TDD | N/A |

# References

[1] R4-2107627, Triple beat and 3ULCC MSD, Qualcomm Incorporated, R4#99-e

[2] R4-2202154 rev of R4-2202034 Triple beat B3 MSD evaluation for DC_3A_n41C, Skyworks Solutions Inc., R4#101Bis-e

[3] R4-2204217, Triple Beat MSD update, Qualcomm Incorporated, R4#102-e

[4] RP-210639, Guidelines for FR1 CA/DC band combinations not subject to block approval, 3GPP TSG-RAN #91-e , Apple Inc., Skyworks Solutions Inc.

[5] R4-2202279, WF on triple beat evaluation and specification framework_v5, Qualcomm Incorporated, Skyworks Solutions Inc.,R4#101-bis-e

[6] R4-22xxxxx, CR_for_DC_3_n41C Triple_Beat_MSD, Qualcomm Incorporated, Skyworks Incorporated, R4#102e.