3GPP TSG-RAN WG4 Meeting #102-e                                          R4-2206584

Electronic Meeting, 21 Feb – 3 Mar 2022

Source: 	MediaTek, Skyworks, Qualcomm, Samsung, KDDI

Title: 	WF on CA_n18- n28 and DC_18-n28 low MSD second test point

Agenda Item:	9.8.2

Document for:	Approval

Background

- In WF R4-2202275 it was agreed to review band n28 MSD levels for CA_n18-n28 as well as DC_18_n28 using proposals in the WF

- The n28 MSD specifications need to be re-evaluated since the n28A range may be impacted by n18 UL 5th order IMD products, while the n28B range may be impacted by partial overlap of n18 UL third-order IMD products

- [1][2] analyzed the n28 MSD and these are further discussed during the meeting

Proposed solution:

| 18 | n28X | 31.3 | 28.7 | 26.9 | 24.3 |  | 12.4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | n28 XX | 4.5 | 3 | [2.2] | [1.4] |  | [0.1] |
| NOTE X: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. NOTE XX: The MSD exceptions are applicable to the case that interference of UL band 5th order IMD product falls into the affected DL channels. Minimum requirements apply when the upper edge of DL carrier bandwidth is less than 788MHz. |  |  |  |  |  |  |  |

Concerns :

If we do not have an issue with having a test point that is representative of the Japan spectrum that overlaps 28A and 28B, there is no way this should imply the use of the 28A filter. The spectrum assigned in Japan is 28B and a UE implementing 28B or 28F filter needs to be able to pass the two test points (we cannot force the implementation)

WF if agreeable

Agree on test point that address concern and UE shall pass the test point irrespective of UE implementation

- Proposal for CA_n18-n28

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n18 | n28x | 31.3 | 28.7 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE X: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Proposal for DC_18_n28

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 18 | n28x | 31.3 | 28.7 | 26.9 | 24.3 |  | 12.4 |  |  |  |  |  |  |  |
| NOTE X: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Companies can always bring contributions next meeting for a second test point once they find a way

- References

[1] R4-2204213 CA_n18-n28 and DC_18_n28 LB_LB_MSD, RAN4#102-e

[2] R4-2206140 Corrections for CA_n18-n28, DC_18_n18 MSD, RAN4#102-e

[3] R4-2204480 Discussion on CA_n18_n28, RAN4#102-e

[4] R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28, RAN4#102-e