3GPP TSG-RAN4 Meeting #102-e	R4-2206791

# Electronic Meeting, 21 February – 3 March, 2022

Agenda item:			4.1.6, 4.1.7

Source:	Huawei, HiSilicon

Title:	WF on remaining issues in Rel-15 NR RRM

Document for:	Approval

Introduction

This document captures the agreements as well as open issues and corresponding options for Rel-15 NR RRM core and performance requirements maintenance.

Way Forward for core requirements

- Issue 1-1-1: Applicable DRX cycle for measurement in NE-DC and NR-DC

Candidate options:

  - Option 1

    - For both NE-DC and NR-DC mode, the applicable DRX cycle for the inter-frequency measurement requirement follows the maximum of configured MCG DRX cycle and SCG DRX cycle

  - Option 2

    - For both NE-DC and NR-DC mode, no clarification in 38.133 is needed.

      - For the case where the MCG and the SCG configure an inter-frequency or an inter-RAT measurement on a different ssbFrequency, follow DRX cycle of the CG that configures the measurement.

      - For the case where the MCG and the SCG configure an inter-frequency or an inter-RAT measurement on a same non-serving ssbFrequency, follow the shortest DRX cycle between MCG and SCG.

      - For the case where the MCG and the SCG configure an inter-frequency or an inter-RAT measurement on a same serving ssbFrequency, follow the shortest DRX cycle of the CG that is “in use”.

  - Option 3

    - DRX cycle for NR-DC inter-frequency case shall follow the principles agreed for intra-frequency measurements.

    - DRX cycle for NE-DC shall be follow the principles mentioned in below table.

      - For inter-frequency NR measurement configured by MCG, follow MCG DRX cycle

      - For inter-RAT NR LTE measurement configured by MCG, follow SCG DRX cycle

Conclusion (from GTW on 28th Feb.):

Session chair: No consensus can be reached. Do not recommend to continue discussion.

- Issue 1-2-1: FR2 cell reselection in Idle mode

Current spec: If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1].

Agreement (from GTW on 28th Feb.):

Keep 10s in Rel-15 and Rel-16, and FFS for Rel-17

Way Forward for performance requirements

- Issue 2-1-1: additional margins to the lower bound for FR2 inter-frequency relative RSRP accuracy test requirements

- D: margin due to mis-alignment between fine beam and rough beam

- Ginter: margin due to different antenna gain on different bands

- E: margin due to difference between Y’ and Z’

  - Y’: actual gain difference between fine and rough beam at peak direction

  - Z’: actual gain difference between fine and rough beam at spherical coverage direction

Agreements:

- Add the following margin to the lower bound when two cells are in different bands

  - D + Ginter

Open issue and Candidate options:

- Add the following margin to the lower bound when two cells are in same band

  - Option 1 (Apple, MTK, HW, Ericsson, Anritsu)

    - D + Ginter

  - Option 2 (QC, MTK, Ericsson)

    - D

  - Option 3 (Anritsu)

    - FFS, Ginter may be different for inter-band and intra-band cases

- Whether the conclusion on the need for Ginter for intra-band case can apply to upper bound (Issue 2-1-2)

  - Option 1

    - Yes

  - Option 2

    - No

- Issue 2-1-2: additional margins to the upper bound for FR2 inter-frequency relative RSRP accuracy test requirements

Candidate options:

  - Option 1 (Anritsu, QC, Ericsson)

    - Ginter

  - Option 2 (Anritsu, Apple, HW)

    - Ginter + E

  - Note: Need for Ginter for intra-band case is addressed in Issue 2-1-1.

- Issue 2-1-3: exact values for different margins for FR2 inter-frequency relative RSRP accuracy test requirements

Agreements:

- D = [5.5] dB

- Ginter = [3] dB

Open issue and Candidate options:

- The value for E (if option 2 in Issue 2-1-2 can be agreed)

  - Option 1 (Apple, HW)

    - Same value as Y

  - Option 2a (Anritsu)

    - Y-Y’

  - Option 2b (HW)

    - Y’-Z’

  - Note: Need for E for is addressed in Issue 2-1-2.

  - Y’: actual gain difference between fine and rough beam at peak direction

  - Z’: actual gain difference between fine and rough beam at spherical coverage direction