3GPP TSG-RAN WG4 Meeting #102-e	R4- 2207066

E-meeting, February 21 – March 03, 2022

Agenda item:			10.16.8.3, 10.16.8.4, 10.16.8.5, 10.16.8.6

Source:	Moderator (Intel)

Title:	Email discussion summary for [102-e][225] NR_ext_to_71GHz_RRM_2

Document for:	Information

# Introduction

In RAN #89e meeting a new WI on Extending current NR operation to 71GHz was approved. During the RAN4 #99 the initial scope of RRM work for NR_ext_to_71GHz WI was defined and captured in the way forward R4-2108354. Further discussion was split into two email threads. The discussion in the first email thread can be retraced through R4-2115405 with corresponding WF R4-2115351 for RAN4 #100-e, through R4-2120370 with corresponding WF R4-2120316 for RAN4 #101-e and through R4-2202733 with corresponding WF R4-2202657 for RAN4 #101-bis-e. For the second email thread the discussion can be retraced through R4-2115406 with corresponding WF R4-2115352 for RAN4 #100-e, through R4-2120371 with corresponding WF R4-2120317 for RAN4 #101-e and through R4-2202734 with corresponding WF R4-2202659 for RAN4 #101-bis-e.

Current email discussion document covers four agenda items:

10.16.8.3	Interruption requirements

10.16.8.4	Active BWP switching delay requirements

10.16.8.5	Measurement gap interruption requirements

10.16.8.6	LBT impacts on RRM requirements

For three of them all the essential agreements have already been made and a very limited number of tdocs were submitted for current meeting. The discussion will be mostly focused on the last agenda item in the list - LBT impacts on RRM requirements.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Companies are expected to provide views and/or comments on the listed open issues.

- 2nd round: Continue discussion on the open issues. Focus on draft CRs review.

For the Email discussion guidelines please refer to the Meeting Arrangements document provided by RAN4 chair before the meeting.

# Topic #1: Interruption and active BWP switching requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204876 | Huawei, Hisilicon | Proposal 1: Interruption requirements of CBW change for NR-CA can apply to NR-DC. |

## Open issues summary

### Sub-topic 1-1. Interruption requirements

Sub-topic description: In this sub-topic the interruption requirements of CBW change are discussed

Open issues and candidate options before e-meeting:

Issue 1-1-1: Interruption requirements of CBW change for NR-DC.

- Proposals

  - Proposal 1 (Huawei): Interruption requirements of CBW change for NR-CA can apply to NR-DC.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We are fine with Proposal 1, considering that it was already agreed for maintenance. |
| Intel | Ok with the proposal. |
| vivo | OK with the proposal. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204877 Draft CR on interruption requirements for FR2-2 | Nokia: We are fine with the CR. Please notice that you have changed over changes at: The interruption is only allowed within the delay TRRCprocessingDelay + TCBWchangeDelayRRC defined in clause 8.713. |
|  | Huawei: Response to Nokia: Thanks for spotting. This is made on purpose to show the changes based on the endorsed version. We think it can be merged in the final big CR. |
|  | Intel: Fine with the CR |
|  | vivo: Fine with the CR. |
| R4-2204541 Draft CR - Correction on BWP switch delay for dormant BWP in FR2-2 | Intel: The corrections are ok for us. |
|  | vivo: Fine with the CR. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 Interruption requirements | Issue 1-1-1: Interruption requirements of CBW change for NR-DC. Companies’ views: All companies agreed with the Proposal Agreement: Interruption requirements of CBW change for NR-CA can apply to NR-DC Recommendations for 2nd round: No further discussion is needed in the second round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2204877 Draft CR on interruption requirements for FR2-2 | Based on 1st round of comments collection moderator recommends the “agreeable” status for this draft CR |
| R4-2204541 Draft CR - Correction on BWP switch delay for dormant BWP in FR2-2 | Based on 1st round of comments collection moderator recommends the “agreeable” status for this draft CR |

## Discussion on 2nd round (if applicable)

No further discussion is needed on this topic

# Topic #2: LBT impacts on RRM requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203808 | Apple | Proposal 1: Agree on Option 1: N is equal to the RX beams sweeping scaling factor. Proposal 2: Options 1 and 2 can be considered for scaling the maximum number of allowed measurement occasions to be missed due to LBT failure. Two options on how to scale the maximum number of allowed measurement occasions: Option 1: Scaling the number of rounds of beam sweeping needed, e.g., Mpss/sss_sync_w/o_gaps_CCA= Mpss/sss_sync_w/o_gaps + N*SRpss/sss = (3+ SRpss/sss)*N, where N is the FR2-2 RX beam sweeping factor, and SRpss/sss is the additional rounds of RX beam sweeping required to compensate for the lost SMTC/SSB occasions due to LBT failures. A limit can be placed on SRpss/sss. Option 2: Using an absolute number, e.g., Mpss/sss_sync_w/o_gaps_CCA= Mpss/sss_sync_w/o_gaps + Lpss/sss, where Lpss/sss is the number of SMTC/SSB occasions not available at the UE due to LBT failures. A limit can be placed on Lpss/sss, similar to the case of NR-U. Proposal 3: Further discuss if some limit should be set on the time gap between two successful measurement samples to guarantee performance. |
| R4-2204192 | MediaTek inc. | Proposal 1: N is equal to the RX beams sweeping scaling factor (Option 1), as the starting point. Proposal 2: Reuse the legacy maximum number of allowed measurement occasions to be missed due to LBT failure, Lmax. No need to scale the Lmax. |
| R4-2204543 | Nokia, Nokia Shanghai Bell | Observation 1: At 60 GHz narrow beams are expected, which, combined with a stronger signal attenuation, results in a LBT failure probability which is lower than what is experienced at 5 GHz band. Observation 2: At 60GHz, the performance of directional LBT and omni-directional LBT is similar to the performance with no-LBT. Observation 3: The LBT procedure in FR2-2 is simplified when compared to FR1. There is no adaptation of the contention window based on collision detection, the deferral times are also reduced, and the discovery burst transmissions by the gNB might be exempt from LBT. Proposal 1: N is equal to the RX beam sweeping scaling factor. Proposal 2: There is no need to apply scaling to the maximum number of SMTC occasions not available at the UE. Observation 4: N is already used in clauses 9.2A and 9.3A, to refer to the number of candidate positions for the same SSB index. Proposal 3: In TS 38.133, use NRxBeam instead of N, to refer to RX beam sweeping scaling factor. Observation 5: The number of samples considered in intra-frequency and inter-frequency measurements in FR2 depends on the UE power classes. Observation 6: There is no consensus yet on the power classes to be supported by UEs in FR2-2 unlicensed. Proposal 4: Wait for decision on the power classes to be supported in FR2-2 before agreeing on the number of samples Mpss/sss_sync_w/o_gaps_CCA , Mmeas_period_w/o_gaps_CCA, Mpss/sss_sync_with_gaps_CCA, Mmeas_period_ with_gaps_CCA, Mpss/sss_sync_inter_CCA, MSSB_index_inter_CCA, Mmeas_period_inter_CCA for unlicensed operation in FR2-2. Observation 7: In NR-U in FR1, it was agreed that the UE is required to be capable of performing measurements in the same number of cells and SSBs as in licensed FR1. Proposal 5: For FR2-2 intra-frequency measurement requirements with LBT, reuse the FR2 requirements on the minimum number of cells and number of SSBs that the UE shall be capable of performing measurements in each intra-frequency layer. Proposal 6: For FR2-2 inter-frequency measurement requirements with LBT, reuse the FR2 requirements on the minimum number of cells and number of SSBs that the UE shall be capable of performing measurements in each inter-frequency layer. Proposal 7: Define the time for intra-frequency PSS/SSS detection without measurement gaps in FR2-2 with LBT as: DRX cycle; TPSS/SSS_sync_intra_CCA. No DRX; max(600ms, ceil((Mpss/sss_sync_w/o_gaps_CCA + [NRxBeam] x LPSS/SSS) x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x (Mpss/sss_sync_w/o_gaps_CCA + [NRxBeam] x LPSS/SSS) x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil((Mpss/sss_sync_w/o_gaps_CCA + [NRxBeam] x LPSS/SSS) x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, LPSS/SSS is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When DRX is configured, LPSS/SSS is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 3: LPSS/SSS,max =7 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,max =5 for 40ms<Max(DRX cycle,SMTC period)≤320ms, LPSS/SSS,max = 3 for DRX cycle>320ms. NOTE 4: Upon exceeding LPSS/SSS,max, the UE is not required to meet the requirements for PSS/SSS detection.. Proposal 8: Define the time period for intra-frequency PSS/SSS detection without measurement gaps for deactivated SCell in FR2-2 with LBT as: DRX cycle; TPSS/SSS_sync_intra_CCA. No DRX; Ceil((Mpss/sss_sync_w/o_gaps + [NRxBeam] x LPSS/SSS,deact) x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil((Mpss/sss_sync_w/o_gaps + [NRxBeam] x LPSS/SSS,deact) x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil((Mpss/sss_sync_w/o_gaps + [NRxBeam] x LPSS/SSS,deact) x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. NOTE 1: When DRX is not configured, LPSS/SSS, deact is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max. When DRX is configured, LPSS/SSS, deact is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max.When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. NOTE 2: LPSS/SSS, deact,max, = 7 for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS, deact,max = 5 for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, LPSS/SSS, deact,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding LPSS/SSS, deact,max,, the UE is not required to meet the requirements for PSS/SSS detection.. Proposal 9: Use the tables proposed for intra-frequency measurement requirements without measurement gaps as baseline for the definition of intra-frequency measurement requirements with measurement gaps and inter-frequency measurement requirements in FR2-2 with LBT. Observation 8: deriveSSB-IndexfromCell is optional for NR-U in FR1, and it is up to network configuration whether to enable it or not in FR2-2, for SCS equal to 960 kHz. Proposal 10: It is up to network configuration whether to enable deriveSSB-IndexfromCell or not for unlicensed operation in FR2-2. Proposal 11: RAN4 to postpone the definition of requirements for intra-frequency time index detection in FR2-2 unlicensed, until there is an agreement for intra-frequency time index detection in FR2-2 licensed. Proposal 12: Develop new handover requirements for FR2-2 with CCA, at least for the following scenarios: NR FR2-2 – NR FR2-2 Handover and NR FR1 – NR FR2-2 Handover. Proposal 13: Extend the applicability of the requirements in clause 6.1.1.3 in TS 38.133 to support handover from NR FR2-2 with CCA to NR FR1. Proposal 14: For handover to cells in FR2-2 with LBT, no extension of TIU is needed for Type 3 channel access, defined in TS 37.213: L3 = 0. Proposal 15: Update the work split for FR2-2 LBT support in Measurement procedures as follows: Draft CR; Company. DraftCR for FR2-2 LBT support in Intra-Frequency measurements; Nokia. DraftCR for FR2-2 LBT support in Inter-Frequency measurements. DraftCR for FR2-2 LBT support in L1-RSRP measurements for reporting.. |
| R4-2204650 | vivo | Proposal 1: For LBT operation in the new unlicensed band in FR2-2, further scale of NR-U extension in Rel-16 is discussed on case-by-case basis. Proposal 2: Whether and how to scale the maximum number of allowed measurement occasions to be missed due to LBT failure is discussed on case-by-case basis. |
| R4-2204727 | Ericsson | Proposal 1: Support Option 1. N is equal to the RX beams sweeping scaling factor Proposal 2: RAN4 to introduce the max function for timer T = max(10s, M1*( P1s +K1*N1)*DRX cycles) for FR2-2, where - K1 is 16 if DRX cycle is 0.32s, 8 if DRX cycle is 0.64s, otherwise, K1 = 4. - In FR2-2, for UE support power class 2&3&4, N1 = 8 for TDRX =0.32s, 5 for TDRX = 0.64s, 4 for TDRX = 1.28s and 3 for TDRX = 2.56s. In FR2, for UE support power class 1, N1= 8. - P1s is the number of DRX cycles each with at least one SMTC occasion not available during the TPLMN and P1s ≤ P1s,max. - P1s,max = 16 if DRX cycle is 0.32s; 8 if DRX cycle is 0.64s, otherwise, P1s,max = 4. |
| R4-2204878 | Huawei, Hisilicon | Proposal 1: N is equal to the RX beams sweeping scaling factor Proposal 2: L* and L*, max is defined as the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one SSB/SMTC occasions is not available in the group. |
| R4-2205404 | ZTE Corporation | Proposal 1: RAN4 will reflect the LBT failures by extending the RRM requirements by N*SMTC/SSB occasions when there is at least one SMTC/SSB occasion not available at UE within N*SMTC/SSB occasions, where N equals to the RX beams sweeping scaling factor is adopted as the baseline. |
| R4-2206006 | Intel Corporation | Proposal 1: RAN4 to focus on defining the CCA related requirements for single carrier case deprioritizing the following CCA related requirements definition: SCell activation and deactivation delay in Carriers with CCA PSCell addition and release delay PSCell Change Conditional PSCell change Proposal 2: RAN4 to agree how to define the number of additional Rx beam sweeping rounds when there are the SMTC/SSB occasions not available at the UE: Option 1: One additional Rx beam sweeping round for each Rx beam sweeping round which contains at least one SMTC/SSB occasion missed due to LBT failure Option 2: Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period. If there are no SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period, then only one additional Rx beam sweeping round is needed for measurement. Proposal 3: For requirements on Measurement and evaluation when subject to CCA on the serving cell RAN4 to scale up the parameters Mn, Mp, Mq and the periodicity of the cell selection criterion evaluation by Rx beam scaling factor Proposal 4: If Option 1 in Proposal 2 will be agreed then Table 4.2A.2.2-1 should be changed as follows Table 4.2A.2.2-1: Nserv_CCA DRX cycle length [s]; Scaling Factor (N1); Nserv_CCA [number of DRX cycles]. FR1; FR2. 0.32; 1; [8]; N1*M1*(4+Ms). 0.64; [5]; N1*M1*(4+Ms). 1.28; [4]; N1*(2+Ms). 2.56; [3]; N1*(2+Ms). Note 1: Ms is the number of (N1 DRX cycles) each with at least one SMTC occasion not available at the UE during Nserv_CCA, and Ms< Ms,max Note2: Ms,max=8 for DRX cycle length < 1.28 s, Ms,max= 4 for DRX cycle length ≥ 1.28 s.. If Option 2 in Proposal 2 will be agreed then Table 4.2A.2.2-1 should be changed as follows Table 4.2A.2.2-1: Nserv_CCA DRX cycle length [s]; Scaling Factor (N1); Nserv_CCA [number of DRX cycles]. FR1; FR2. 0.32; 1; [8]; N1*M1*(4+Ms). 0.64; [5]; N1*M1*(4+Ms). 1.28; [4]; N1*(2+Ms). 2.56; [3]; N1*(2+Ms). Note 1: For FR1 Ms is the number of DRX cycles each with at least one SMTC occasion not available at the UE during Nserv_CCA, For FR2 if there are at least two DRX cycles each with at least one SMTC occasion not available at the UE which are spaced by N1 DRX cycles, then Ms is equal to the number of DRX cycles each with at least one SMTC occasion not available at the UE consequently spaced by N1 DRX cycles during Nserv_CCA, otherwise if there is at least one DRX cycle with at least one SMTC occasion not available at the UE during Nserv_CCA, then Ms is equal to 1, otherwise Ms is equal to 0 Ms< Ms,max Note2: Ms,max=8 for DRX cycle length < 1.28 s, Ms,max= 4 for DRX cycle length ≥ 1.28 s.. Proposal 5: If Option 1 in Proposal 2 will be agreed then Table 4.2A.2.3-1 and Table 4.2A.2.4-1 should be changed as follows: DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra_CCA [s] (number of DRX cycles); Tmeasure,NR_Intra_CCA [s] (number of DRX cycles); Tevaluate,NR_Intra_CCA [s] (number of DRX cycles). FR1; FR2. 0.32; 1; [8]; 0.32xN1x(36+Md)xM2 {(36+Md)xN1xM2}; 0.32xN1x(4+Mm)xM2 {(4+Mm)xN1xM2; 0.32xN1x(16+Me) x M2 {(16+Me)xN1xM2}. 0.64; [5]; 0.64xN1x(28+Md) {(28+Md)xN1}; 0.64xN1x(2+Mm) {(2+Mm)xN1}; 0.64xN1x(8+Me) {(8+Me)xN1}. 1.28; [4]; 1.28xN1x(25+Md) {(25+Md)xN1}; 1.28xN1x(1+Mm) {(1+Mm)xN1}; 1.28xN1x(5+Me) {(5+Me)xN1}. 2.56; [3]; 2.56xN1x(23+Md) {(23+Md)xN1}; 2.56xN1x(1+Mm) {(1+Mm)xN1}; 2.56xN1x(3+Me) {(3+Me)xN1}. Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: Md, Mm, Me are the number of (N1 DRX cycles) each with at least one SMTC occasion not available during the Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, Note 3: Mm,max = 16 for DRX cycle length = 0.32s; Mm,max = 8 for DRX cycle length = 0.64s; Mm,max = 4 for DRX cycle length = 1.28s; Mm,max = 4 for DRX cycle length = 2.56s. Note 4: Md,max = 4*Mm,max, Me,max = 2*Mm,max.. If Option 2 in Proposal 2 will be agreed then Table 4.2A.2.3-1 and Table 4.2A.2.4-1 should be changed as follows DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra_CCA [s] (number of DRX cycles); Tmeasure,NR_Intra_CCA [s] (number of DRX cycles); Tevaluate,NR_Intra_CCA [s] (number of DRX cycles). FR1; FR2. 0.32; 1; [8]; 0.32xN1x(36+Md)xM2 {(36+Md)xN1xM2}; 0.32xN1x(4+Mm)xM2 {(4+Mm)xN1xM2; 0.32xN1x(16+Me) x M2 {(16+Me)xN1xM2}. 0.64; [5]; 0.64xN1x(28+Md) {(28+Md)xN1}; 0.64xN1x(2+Mm) {(2+Mm)xN1}; 0.64xN1x(8+Me) {(8+Me)xN1}. 1.28; [4]; 1.28xN1x(25+Md) {(25+Md)xN1}; 1.28xN1x(1+Mm) {(1+Mm)xN1}; 1.28xN1x(5+Me) {(5+Me)xN1}. 2.56; [3]; 2.56xN1x(23+Md) {(23+Md)xN1}; 2.56xN1x(1+Mm) {(1+Mm)xN1}; 2.56xN1x(3+Me) {(3+Me)xN1}. Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: For FR1 Md, Mm, Me are the number of DRX cycles each with at least one SMTC occasion not available during the Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, For FR2 if there are at least two DRX cycles each with at least one SMTC occasion not available at the UE which are spaced by N1 DRX cycles, then Md, Mm, Me are equal to the number of DRX cycles each with at least one SMTC occasion not available at the UE consequently spaced by N1 DRX cycles during Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, otherwise if there is at least one DRX cycle with at least one SMTC occasion not available at the UE during Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, then Md, Mm, Me are equal to 1, otherwise Ms is equal to 0 Mm ≤ Mm,max, Md ≤ Md,max and Me ≤ Me,max Note 3: Mm,max = 16 for DRX cycle length = 0.32s; Mm,max = 8 for DRX cycle length = 0.64s; Mm,max = 4 for DRX cycle length = 1.28s; Mm,max = 4 for DRX cycle length = 2.56s. Note 4: Md,max = 4*Mm,max, Me,max = 2*Mm,max.. Proposal 6: RAN4 to reuse legacy FR2 margins for reselection criteria for inter-frequency and intra-frequency measurements in RRC_IDLE state mobility Proposal 7: RAN4 to define requirements for Handover to target cell using CCA for FR2 - FR2 and FR1 - FR2 cases by changing regular FR2 - FR2 and FR1 - FR2 requirements in the same way as it is done for FR1 - FR1 case. The definition of L1 and L1´ should follow an agreement on Proposal 2 with N equal to 8. Proposal 8: For RRC re-establishment with CCA and RRC release with redirection to NR carrier subject to CCA RAN4 to reuse NR-U requirements except requirements for Tidentify_intra_NR_CCA , Tidentify_inter_NR_CCA, i Tidentify-NR_CCA. To define requirements for Tidentify_intra_NR_CCA , Tidentify_inter_NR_CCA, i Tidentify-NR_CCA RAN4 to extend the corresponding non-CCA requirements with additional 8xK1, 8xK2,i and 8xL1 SMTC periods, respectively. The definition of K1, K2,i and L1 should follow an agreement on Proposal 2 with N equal to 8. |

## Open issues summary

### Sub-topic 2-1. Scope of the RRM requirements to be defined

Sub-topic description: In this sub-topic the possible LBT-related RRM scope reduction and further DraftCR work split are discussed

Open issues and candidate options before e-meeting:

Issue 2-1-1: RRM scope

- Proposals

  - Proposal 1 (Intel): RAN4 to focus on defining the CCA related requirements for single carrier case deprioritizing the following CCA related requirements definition:

    - SCell activation and deactivation delay in Carriers with CCA

    - PSCell addition and release delay

    - PSCell Change

    - Conditional PSCell change

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Do not agree. Requirements should be defined for the scenarios within the scope of the WID. |
| Qualcomm | We agree with Nokia. We should define requirements for the agreed scenarios in RAN4 - CA and NR-DC with FR2-1 |
| Intel | Our proposal intends to deprioritize only LBT-based requirements for CA and DC. Regular FR2-2 requirements for CA and DC will still be defined. |
| vivo | We share the same view with QC and Nokia. The LBT requirements for CA and NR-DC are also defined. |
| Apple | If time allows, we hope to cover CA/DC. |

Issue 2-1-2: DraftCR Work split

- Proposals

  - Proposal 1 (Nokia): Further split the DraftCR on measurement procedure

- Recommended WF

  - Companies are invited to volunteer for new Draft CRs

| Draft CR | Company |
| --- | --- |
| DraftCR for FR2-2 LBT support in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state mobility requirements | Intel |
| DraftCR for FR2-2 LBT support in Radio Link Monitoring and Link recovery procedures | Huawei |
| DraftCR for FR2-2 LBT support in SCell Activation and Deactivation Delay requirements and Active TCI state switching delay requirements | MTK |
| DraftCR for FR2-2 LBT support in Intra-Frequency measurements | Nokia |
| DraftCR for FR2-2 LBT support in Inter-Frequency measurements | Ericsson |
| DraftCR for FR2-2 LBT support in L1-RSRP measurements for reporting. | Qualcomm |
| DraftCR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change | vivo |

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Qualcomm | We can volunteer for FR2-2 LBT support in L1-RSRP measurements for reporting |
| Ericsson | We voluntarily take DraftCR for FR2-2 LBT support in Inter-Frequency measurements. |
| ZZZ |  |

Issue 2-1-3: Whether to use “FR2” or “FR2-2” term for CCA related requirements in TS38.133 spec

- Moderator’s note:

There was an agreement in R4-2120316: “The designations FR2-1 and FR2-2 should only be used when needed to differentiate the requirements between the two FRs”.

- Options:

  - Option 1: Use “FR2-2” term for all CCA related requirements in the spec since currently there is no CCA operation in FR2-1

  - Option 2: Use “FR2” term in the section naming but “FR2-2” in the section content (e.g. for Handover sections)

  - Other options are not precluded

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Option 1. It is also worth mentioning that TS 37.213, which defines the channel access mechanisms, uses “frequency range 2-2” in clause 4.4. |
| Qualcomm | Support option 1 to maintain consistency. |
| Huawei | Support option 1. |
| MTK | Support option 1. |
| Ericsson | Prefer Option1, because CCA isn’t for FR2-1. |
| Intel | Prefer Option 2. In case if in future any LBT-based operation will be defined for FR2-1, with Option 1 we will need to change the section name which is not preferred. However, we understand that currently there are no such plans and the requirements are defined for FR2-2 only. So, we are also ok with Option 1. |
| vivo | Support option 1. |
| Apple | Option 1 is OK. |
| CATT | Prefer Option1, |

### Sub-topic 2-2. General aspects on the RRM requirements

Sub-topic description: In this sub-topic the general aspects of LBT-related RRM specification changes are discussed mainly focusing on the details of how to take into account the LBT failures on the RRM requirements

Open issues and candidate options before e-meeting:

Issue 2-2-1: The LBT-based FR2-2 RRM requirements extension step.

- Background

During RAN4 #101-bis-e it was agreed that RAN4 will reflect the LBT failures by extending the RRM requirements by N*SMTC/SSB occasions when there is at least one SMTC/SSB occasion not available at UE within N*SMTC/SSB occasions. Value of N needed clarification

- Proposals

  - Proposal 1 (Apple, MTK, Nokia, Ericsson, Huawei, ZTE): N is equal to the RX beam sweeping scaling factor.

  - Proposal 2 (vivo): For LBT operation in the new unlicensed band in FR2-2, further scale of NR-U extension in Rel-16 is discussed on case-by-case basis

- Recommended WF

  - Agree with proposal 1.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Option 1. |
| Qualcomm | Proposal 1 can be a good starting point. |
| Huawei | Support proposal 1 |
| MTK | Support option 1. |
| Ericsson | Ok with WF |
| Intel | Support Proposal 1 |
| vivo | In our understanding, when there is one SMTC/SSB occasion not available at UE, it is possible that the unavailable SSB occasion is not necessary for UE. In other words, the other available SSB occasions is enough for UE to perform measurement. If once there is one SMTC/SSB occasion not available at UE within N*SMTC/SSB occasions, the RRM measurement period need to be expended N*SMTC/SSB occasions. We believe this would cause very long measurement period. As mentioned in Option 3 in Issue 2-2-2, we understand it can avoid the unnecessary delay in some special cases. If other companies can accept the long measurement period, we can compromise to Proposal 1. |
| Apple | Proposal 1. |
| CATT | Support proposal 1 |

Issue 2-2-2: How to take into account the LBT failures on the RRM requirements

- Moderator’s note

To Moderator’s observation there are different interpretations of the RAN4 #101-bis-e agreement on the number of additional N*SMTC/SSB occasions (mentioned above in the background for Issue 2-2-1). This issue is to discuss more details on how many additional Rx beam sweeping rounds are needed.

- Proposals

  - Option 1 (Apple): For every unavailable SMTC occasion, the time is extended by N*SMTC_period.

  - Option 2 (Huawei): The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one SSB/SMTC occasions is not available in the group.

  - Option 3 (Intel): Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period. If there are no SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period, then only one additional Rx beam sweeping round is needed for measurement.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Option 2 and Option 3 are not so different, we can agree to any of them and adjust the wording later. |
| Qualcomm | All three options look similar. We slightly prefer Option 2. |
| Huawei | Slightly prefer option 2. We understand option 3 is try to avoid some unnecessary extension in some cases. N is the used to define the minimum requirements, but the actually beam seeping strategy may depends on UE implementation. For instance, under same LBT pattern, it is possible that for one UE, the LBT failures happen on SMTC with same beam direction, while for another UE, they may happen with different beam directions. |
| Ericsson | If two unavailable SSB occasions within N SSB occasions, Option 1 is extending 2* N SSB occasions: Option 2 is extending 1* N SSB occasions. Regarding Option 3, we think there may be a question derived by‘SMTC occasions not available at the UE are spaced by N SMTC occasions’. It’s not always true that per N SMTC occasion has same spatial angle, how to capture this case? We support Option1 but open to other options. |
| Intel | The intention of Option 3 is to avoid unnecessary extension of RRM requirements. However, we agree that this Option assumes the beam sweeping strategy where UE keeps the same order of beams for different beam sweeping rounds. We understand that beam sweeping strategy is up to UE implementation and it is not good to consider such assumptions for requirement definition. We are ok with Proposal 2. |
| vivo | Prefer Option 3. In our understanding, if the UE keeps the same order of beams for different beam sweeping rounds, when the consequent N SMTC/SSB occasions are available at the UE, it is feasible to increase one additional Rx beam sweeping round. We are fine with Option 2. |
| Apple | We can accept Option 2. |
| CATT | Prefer option 2. We also understand the special case of two LBT failures happen when UE scans the same beam in option 3. However, we believe that when define requirements, it cannot be equivalent to ‘consecutively spaced by N SMTC / SSB occasions’, since the spatial angle of each N SMTC / SSB occasions is not always the same, and whether spaced by N SSB occasions depends on the order of beams for different beam sweeping rounds and UE implementation. Though we think UE may keep the same order of beams and spaced by N SSB occasions in most cases, it is also related to the value of N and the time of beam sweeping. Another option is to revise option3, it is proposed the following option: Option 4:Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and based on the measurement of the same beam during the measurement period. If there are no SMTC/SSB occasions not available at the UE and based on the measurement of the same beam during the measurement period, then only one additional Rx beam sweeping round is needed for measurement. |

Issue 2-2-3: Maximum number of SMTC occasions not available at the UE

- Background

During the RAN4 #101-bis-e the issue was raised on whether the existing maximum numbers of SMTC occasions not available at the UE, which were defined for FR1 CCA related RRM requirements need to be scaled for FR2-2

- Proposals

  - Option 1 (MTK, Nokia): No need to apply scaling to the maximum number of SMTC occasions not available at the UE.

  - Option 2 (vivo): Whether and how to scale the maximum number of allowed measurement occasions to be missed due to LBT failure is discussed on case-by-case basis.

  - Option 3 (Apple): Consider scaling of maximum number of SMTC occasions not available at the UE

    - Option 3a: Scaling the number of rounds of beam sweeping needed, e.g., Mpss/sss_sync_w/o_gaps_CCA= Mpss/sss_sync_w/o_gaps + N*SRpss/sss = (3+ SRpss/sss)*N, where N is the FR2-2 RX beam sweeping factor, and SRpss/sss is the additional rounds of RX beam sweeping required to compensate for the lost SMTC/SSB occasions due to LBT failures. A limit can be placed on SRpss/sss.

    - Option 3b: Using an absolute number, e.g., Mpss/sss_sync_w/o_gaps_CCA= Mpss/sss_sync_w/o_gaps + Lpss/sss, where Lpss/sss is the number of SMTC/SSB occasions not available at the UE due to LBT failures. A limit can be placed on Lpss/sss, similar to the case of NR-U

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Option 1. There is no need to apply scaling. For each SMTC occasion not available at the UE (or SMTC group, depending on the issue above), the requirements will be extended by N*SMTC occasions. Therefore, we can reuse the values from FR1, but the delays in FR2-2 will already be much longer in case of CCA failure. It is also important to stress that the CCA failures in FR2-2 are much less frequent than in FR1. |
| Qualcomm | Extending the delays with N*no of missed SMTCs makes more sense than applying a scaling factor. |
| Huawei | We believe it is related to the discussion in issue 2-2-3 on whether to define the requirements in number of SSB groups/rounds? |
| MTK | Support option 1. The delay is extended with N*no of missed SMTCs, but the maximum delay is not needed to be extended. |
| Ericsson | Prefer Option1. |
| Intel | Support Option 1. We prefer to keep the FR1 values for maximum numbers of SMTC occasions not available at the UE |
| vivo | We understand Option 1 and Option 2 are not opposites. Similar to FR1, for different cases (e.g., PSS/SSS detection and SSB index detection), the maximum number of SMTC occasions is different. We are fine that FR2-2 can reuse the value of FR1 for maximum numbers of SMTC occasions not available at the UE. |
| Apple | With proposal 1 agreed for Issue 2-2-2, we can accept Option 1. Our proposal 3a is essentially the same as Option 1. |
| CATT | Support option 1 and we believe that it is related to the number of SMTC/SSB rounds in issue 2-2-3. |

Issue 2-2-4: Time gap between two successful measurement samples

- Proposals

  - Proposal 1 (Apple): Further discuss if some limit should be set on the time gap between two successful measurement samples to guarantee performance.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We agree to proposal 1. Considering the agreements from last meeting and the discussions above, the time gap between two successful measurements can be much longer in FR2-2 than in FR1, so it makes sense to set a limit between two successful measurement samples. |
| Qualcomm | Proposal 1 makes sense. This can be further discussed. |
| Huawei | We think we have similar discussion in Rel-16 NR-U. It is agreed to use the condition of cell remained detectable to restriction the two available measurement samples. |
| Ericsson | Ok with Proposal 1. |
| Intel | Our understanding is that the limit is already implicitly set by defining maximum numbers of SMTC occasions not available at the UE. |
| Apple | We support proposal 1. |
| CATT | We would like to further discuss proposal 1, and it is proposed to further clarify the relationship between the maximum numbers of SMTC occasions not available at the UE and the time gap limit between two successful measurement samples. |

Issue 2-2-5: Name for RX beam sweeping scaling factor in TS38.133

- Proposals

  - Proposal 1 (Nokia): In TS 38.133, use NRxBeam instead of N, to refer to RX beam sweeping scaling factor.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | This proposal is to avoid confusion with the number of candidate positions within an SMTC/SSB occasion. |
| Qualcomm | We are fine with proposal 1 |
| Huawei | Support option 1 to avoid confusion. |
| Ericsson | Fine with proposal 1 |
| Intel | Ok with Proposal 1 |
| Apple | Proposal 1 is OK. |
| CATT | Fine with proposal 1 |

### Sub-topic 2-3. RRC_IDLE and RRC_CONNECTED state mobility requirements

Sub-topic description: In this sub-topic the requirements on Cell re-selection, Handover, RRC re-establishment and RRC Connection Release with Redirection are discussed

Open issues and candidate options before e-meeting:

#### Cell re-selection

Issue 2-3-1: Time period before cell selection in Idle mode

- Proposals

  - Proposal 1 (Ericsson): RAN4 to introduce the max function for timer T = max(10s, M1*( P1s +K1*N1)*DRX cycles) for FR2-2, where

    - K1 is 16 if DRX cycle is 0.32s, 8 if DRX cycle is 0.64s, otherwise, K1 = 4.

    - In FR2-2, for UE support power class 2&3&4, N1 = 8 for TDRX =0.32s, 5 for TDRX = 0.64s, 4 for TDRX = 1.28s and 3 for TDRX = 2.56s. In FR2, for UE support power class 1, N1= 8.

    - P1s is the number of DRX cycles each with at least one SMTC occasion not available during the TPLMN and P1s ≤ P1s,max.

    - P1s,max = 16 if DRX cycle is 0.32s; 8 if DRX cycle is 0.64s, otherwise, P1s,max = 4.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Do not agree to proposal 1. We would like to point out that this topic is also being discussed in thread 201 (Rel-15 maintenance) and 203 (NR-U maintenance). Our preference is to wait on the decision in the other threads before agreeing to this issue here. |
| Huawei | Should be aligned with discussion in other thread. |
| Intel | Prefer not to discuss this issue in parallel with another thread |

Issue 2-3-2: Requirements on measurement and evaluation when subject to CCA on the serving cell

- Proposals

  - Proposal 1 (Intel): RAN4 to scale up the parameters Mn, Mp, Mq and the periodicity of the cell selection criterion evaluation by Rx beam scaling factor

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We agree to proposal 1. |
| Qualcomm | Fine with the proposal |
| Huawei | Fine with proposal 1. |
| MTK | Support option 1. |
| Ericsson | Agree with Proposal 1 |
| Intel | Agree with Proposal 1 |
| Apple | Proposal 1 is OK. |
| CATT | Fine with proposal 1. |

Issue 2-3-3: Nserv_CCA definition

- Proposals

  - Proposal 1 (Intel): Table 4.2A.2.2-1 should be changed as follows

If Option 2 in Issue 2-2-2 will be agreed

Table 4.2A.2.2-1: Nserv_CCA

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv_CCA [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.32 | 1 | [8] | N1*M1*(4+Ms) |
| 0.64 |  | [5] | N1*M1*(4+Ms) |
| 1.28 |  | [4] | N1*(2+Ms) |
| 2.56 |  | [3] | N1*(2+Ms) |
| Note 1: Ms is the number of (N1 DRX cycles) each with at least one SMTC occasion not available at the UE during Nserv_CCA, and Ms< Ms,max Note2: Ms,max=8 for DRX cycle length < 1.28 s, Ms,max= 4 for DRX cycle length ≥ 1.28 s. |  |  |  |

If Option 3 in Issue 2-2-2 will be agreed

Table 4.2A.2.2-1: Nserv_CCA

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv_CCA [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| 0.32 | 1 | [8] | N1*M1*(4+Ms) |
| 0.64 |  | [5] | N1*M1*(4+Ms) |
| 1.28 |  | [4] | N1*(2+Ms) |
| 2.56 |  | [3] | N1*(2+Ms) |
| Note 1: For FR1 Ms is the number of DRX cycles each with at least one SMTC occasion not available at the UE during Nserv_CCA, For FR2 if there are at least two DRX cycles each with at least one SMTC occasion not available at the UE which are spaced by N1 DRX cycles, then Ms is equal to the number of DRX cycles each with at least one SMTC occasion not available at the UE consequently spaced by N1 DRX cycles during Nserv_CCA, otherwise if there is at least one DRX cycle with at least one SMTC occasion not available at the UE during Nserv_CCA, then Ms is equal to 1, otherwise Ms is equal to 0 Ms< Ms,max Note2: Ms,max=8 for DRX cycle length < 1.28 s, Ms,max= 4 for DRX cycle length ≥ 1.28 s. |  |  |  |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We agree, except for Note 1 which depends on the discussion in issue 2-2-2. |
| Qualcomm | We can try to get some agreement on Issue 2-2-2 first |
| Huawei | We believe N1 is related to the discussion about beam seeping factor. |
| Ericsson | Depends on the discussion in issue 2-2-2. |
| Intel | Since there is majority support for Option 2 in Issue 2-2-2, we are ok with first Table in Proposal 1 |
| CATT | In the view of supporting option 2 in issue 2-2-2, so we correspondingly prefer the first Table in proposal 1. |

Issue 2-3-4: Tdetect, Tmeasure and Tevaluate intra-frequency and inter-frequency NR measurements when subject to CCA

- Proposals

  - Proposal 1 (Intel): Table 4.2A.2.3-1 and Table 4.2A.2.4-1 should be changed as follows

If Option 2 in Issue 2-2-2 will be agreed

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_CCA [s] (number of DRX cycles) | Tmeasure,NR_Intra_CCA [s] (number of DRX cycles) | Tevaluate,NR_Intra_CCA [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2 |  |  |  |
| 0.32 | 1 | [8] | 0.32xN1x(36+Md)xM2 {(36+Md)xN1xM2} | 0.32xN1x(4+Mm)xM2 {(4+Mm)xN1xM2 | 0.32xN1x(16+Me) x M2 {(16+Me)xN1xM2} |
| 0.64 |  | [5] | 0.64xN1x(28+Md) {(28+Md)xN1} | 0.64xN1x(2+Mm) {(2+Mm)xN1} | 0.64xN1x(8+Me) {(8+Me)xN1} |
| 1.28 |  | [4] | 1.28xN1x(25+Md) {(25+Md)xN1} | 1.28xN1x(1+Mm) {(1+Mm)xN1} | 1.28xN1x(5+Me) {(5+Me)xN1} |
| 2.56 |  | [3] | 2.56xN1x(23+Md) {(23+Md)xN1} | 2.56xN1x(1+Mm) {(1+Mm)xN1} | 2.56xN1x(3+Me) {(3+Me)xN1} |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: Md, Mm, Me are the number of (N1 DRX cycles) each with at least one SMTC occasion not available during the Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, Note 3: Mm,max = 16 for DRX cycle length = 0.32s; Mm,max = 8 for DRX cycle length = 0.64s; Mm,max = 4 for DRX cycle length = 1.28s; Mm,max = 4 for DRX cycle length = 2.56s. Note 4: Md,max = 4*Mm,max, Me,max = 2*Mm,max. |  |  |  |  |  |

If Option 3 in Issue 2-2-2 will be agreed

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra_CCA [s] (number of DRX cycles) | Tmeasure,NR_Intra_CCA [s] (number of DRX cycles) | Tevaluate,NR_Intra_CCA [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2 |  |  |  |
| 0.32 | 1 | [8] | 0.32xN1x(36+Md)xM2 {(36+Md)xN1xM2} | 0.32xN1x(4+Mm)xM2 {(4+Mm)xN1xM2 | 0.32xN1x(16+Me) x M2 {(16+Me)xN1xM2} |
| 0.64 |  | [5] | 0.64xN1x(28+Md) {(28+Md)xN1} | 0.64xN1x(2+Mm) {(2+Mm)xN1} | 0.64xN1x(8+Me) {(8+Me)xN1} |
| 1.28 |  | [4] | 1.28xN1x(25+Md) {(25+Md)xN1} | 1.28xN1x(1+Mm) {(1+Mm)xN1} | 1.28xN1x(5+Me) {(5+Me)xN1} |
| 2.56 |  | [3] | 2.56xN1x(23+Md) {(23+Md)xN1} | 2.56xN1x(1+Mm) {(1+Mm)xN1} | 2.56xN1x(3+Me) {(3+Me)xN1} |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: For FR1 Md, Mm, Me are the number of DRX cycles each with at least one SMTC occasion not available during the Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, For FR2 if there are at least two DRX cycles each with at least one SMTC occasion not available at the UE which are spaced by N1 DRX cycles, then Md, Mm, Me are equal to the number of DRX cycles each with at least one SMTC occasion not available at the UE consequently spaced by N1 DRX cycles during Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, otherwise if there is at least one DRX cycle with at least one SMTC occasion not available at the UE during Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, then Md, Mm, Me are equal to 1, otherwise Ms is equal to 0 Mm ≤ Mm,max, Md ≤ Md,max and Me ≤ Me,max Note 3: Mm,max = 16 for DRX cycle length = 0.32s; Mm,max = 8 for DRX cycle length = 0.64s; Mm,max = 4 for DRX cycle length = 1.28s; Mm,max = 4 for DRX cycle length = 2.56s. Note 4: Md,max = 4*Mm,max, Me,max = 2*Mm,max. |  |  |  |  |  |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We agree, except for Note 1, which depends on the discussion in issue 2-2-2. |
| Qualcomm | We can try to get some agreement on Issue 2-2-2 first |
| Intel | Since there is majority support for Option 2 in Issue 2-2-2, we are ok with first Table in Proposal 1 |
| CATT | In the view of supporting option 2 in issue 2-2-2, so we correspondingly prefer the first Table in proposal 1. |

Issue 2-3-5: Margins for reselection criteria for inter-frequency and intra-frequency measurements in RRC_IDLE state mobility

- Proposals

  - Proposal 1 (Intel): RAN4 to reuse legacy FR2 margins for reselection criteria for inter-frequency and intra-frequency measurements in RRC_IDLE state mobility

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Agree with proposal 1. |
| Ericsson | Agree with Proposal 1. |
| Intel | Agree with Proposal 1. |

#### Handover

Issue 2-3-6: Handover requirements

- Proposals

  - Proposal 1 (Nokia, Intel): Develop new handover requirements for FR2-2 with CCA, at least for the following scenarios: NR FR2-2 – NR FR2-2 Handover and NR FR1 – NR FR2-2 Handover

    - Proposal 1a (Intel): RAN4 to define requirements for Handover to target cell using CCA for FR2 - FR2 and FR1 - FR2 cases by changing regular FR2 - FR2 and FR1 - FR2 requirements in the same way as it is done for FR1 - FR1 case. The definition of L1 and L1´ should follow an agreement on Issue 2-2-2 with N equal to 8

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | In general, we agree with the proposals. For proposal 1a, the only part that we do not agree is “N equal to 8”. It is unclear, at this point, whether FR2-2 will reuse the value from FR2 |
| Qualcomm | Fine with proposal 1. |
| Huawei | Fine with option 1 but we believe N is related to the discussion about beam sweeping factor. |
| Intel | Support Proposal 1a and agree with the comments that N should follow the agreement on the beam sweeping scaling factor |

Issue 2-3-7: Handover from NR FR2-2 with CCA to NR FR1

- Proposals

  - Proposal 1 (Nokia): Extend the applicability of the requirements in clause 6.1.1.3 in TS 38.133 to support handover from NR FR2-2 with CCA to NR FR1 (without CCA)

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Huawei | Fine with proposal 1. |
| Intel | Ok with Proposal 1 |
| ZZZ |  |

Issue 2-3-8: TIU is needed for Type 3 channel access

- Proposals

  - Proposal 1 (Nokia): For handover to cells in FR2-2 with LBT, no extension of TIU is needed for Type 3 channel access, defined in TS 37.213: L3 = 0

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | For type 3 channel access in FR2-2, there is no LBT, therefore our proposal is to follow the agreements for NR-U in FR1, and set L3 = 0. |
| YYY |  |
| ZZZ |  |

#### RRC re-establishment and RRC Connection Release with Redirection

Issue 2-3-9: RRC re-establishment with CCA and RRC release with redirection

- Proposals

  - Proposal 1 (Intel): For RRC re-establishment with CCA and RRC release with redirection to NR carrier subject to CCA RAN4 to reuse NR-U requirements except requirements for Tidentify_intra_NR_CCA , Tidentify_inter_NR_CCA, i Tidentify-NR_CCA. To define requirements for Tidentify_intra_NR_CCA , Tidentify_inter_NR_CCA, i Tidentify-NR_CCA RAN4 to extend the corresponding non-CCA requirements with additional 8xK1, 8xK2,i and 8xL1 SMTC periods, respectively. The definition of K1, K2,i and L1 should follow an agreement on Issue 2-2-2 with N equal to 8

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We do not agree that N is equal to 8. It is unclear, at this point, whether FR2-2 will reuse the value from FR2 |
| Huawei | As commented before, N should be based on the conclusion of beam sweeping factor. |
| Intel | Support Proposal 1 and agree with the comments that N should follow the agreement on the beam sweeping scaling factor |

### Sub-topic 2-4. Measurement procedure

Sub-topic description: In this sub-topic different aspects of introducing intra-frequency and inter-frequency measurement requirements with CCA in FR2-2 are discussed

Open issues and candidate options before e-meeting:

Issue 2-4-1: Intra-frequency time index detection in FR2-2 unlicensed

- Proposals

  - Proposal 1 (Nokia): RAN4 to postpone the definition of requirements for intra-frequency time index detection in FR2-2 unlicensed, until there is an agreement for intra-frequency time index detection in FR2-2 licensed

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | This issue depends on the discussion of deriveSSB-IndexFromCell in licensed/ unlicensed. Therefore, we proposed to postpone it. |
| Qualcomm | Agree with proposal 1 |
| Huawei | We think most parts can be discussed only the component related to time index detection can be left as TBD |
| Ericsson | Depends on deriveSSB-IndexFromCell in unlicensed |
| Intel | Ok with Proposal 1 |
| Apple | Proposal 1 is OK. |

Issue 2-4-2: Number of cells and number of SSB for intra-frequency measurements

- Proposals

  - Proposal 1 (Nokia): For FR2-2 intra-frequency measurement requirements with LBT, reuse the FR2 requirements on the minimum number of cells and number of SSBs that the UE shall be capable of performing measurements in each intra-frequency layer.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | This was also done in FR1. Since the working assumption on the number of candidate positions the UE is required to monitor is maintained in FR2-2, we propose to reuse the values from FR2 requirements, in a similar way as it was done in NR-U FR1. |
| Qualcomm | Fine with proposal 1 |
| MTK | Fine with proposal 1 |
| Ericsson | Fine with proposal 1 |
| Intel | Ok with Proposal 1 |
| Apple | Proposal 1 is OK. |

Issue 2-4-3: Number of cells and number of SSB for inter-frequency measurements

- Proposals

  - Proposal 1 (Nokia): For FR2-2 inter-frequency measurement requirements with LBT, reuse the FR2 requirements on the minimum number of cells and number of SSBs that the UE shall be capable of performing measurements in each inter-frequency layer.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | This was also done in FR1. Since the working assumption on the number of candidate positions the UE is required to monitor is maintained in FR2-2, we propose to reuse the values from FR2 requirements, in a similar way as it was done in NR-U FR1. |
| Qualcomm | Fine with the proposal |
| MTK | Fine with proposal 1 |
| Intel | Ok with Proposal 1 |
| Apple | Proposal 1 is OK. |

Issue 2-4-4: Number of samples for intra-frequency and Inter-frequency Measurement Requirements

- Proposals

  - Proposal 1 (Nokia): Wait for decision on the power classes to be supported in FR2-2 before agreeing on the number of samples Mpss/sss_sync_w/o_gaps_CCA , Mmeas_period_w/o_gaps_CCA, Mpss/sss_sync_with_gaps_CCA, Mmeas_period_ with_gaps_CCA, Mpss/sss_sync_inter_CCA,  MSSB_index_inter_CCA, Mmeas_period_inter_CCA for unlicensed operation in FR2-2.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | These values depend on the power classes, therefore we propose to postpone it until there is a decision from the RF group. |
| Qualcomm | Fine with proposal 1 |
| Ericsson | Fine with proposal 1 |
| Intel | In general, we agree with the Proposal. But considering very limited time for Rel-17 completion, we prefer to capture at least some values in brackets in the draftCR. |
| Apple | Proposal 1 is OK. |

Issue 2-4-5: Time for PSS/SSS detection

- Proposals

  - Proposal 1 (Nokia): Define the time for intra-frequency PSS/SSS detection without measurement gaps in FR2-2 with LBT as:

| DRX cycle | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max(600ms, ceil((Mpss/sss_sync_w/o_gaps_CCA + [NRxBeam] x LPSS/SSS) x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x (Mpss/sss_sync_w/o_gaps_CCA + [NRxBeam] x LPSS/SSS) x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil((Mpss/sss_sync_w/o_gaps_CCA + [NRxBeam] x LPSS/SSS) x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, LPSS/SSS is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When DRX is configured, LPSS/SSS is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. NOTE 3: LPSS/SSS,max =7 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,max =5 for 40ms<Max(DRX cycle,SMTC period)≤320ms, LPSS/SSS,max = 3 for DRX cycle>320ms. NOTE 4: Upon exceeding LPSS/SSS,max, the UE is not required to meet the requirements for PSS/SSS detection. |  |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | This table is the implementation of the agreement from RAN4 #101-bis. |
| Qualcomm | Fine with the highlighted changes, but NOTE 2 and 3 may need some revision based on Issue 2-2-2 |
| Huawei | Depends on the conclusion of issue 2-2-2. |
| Intel | Notes 2 and 3 need revision |
| Apple | Agree with Qualcomm, Huawei and Intel. |

Issue 2-4-6: Time for PSS/SSS detection for deactivated SCell

- Proposals

  - Proposal 1 (Nokia): Define the time period for intra-frequency PSS/SSS detection without measurement gaps for deactivated SCell in FR2-2 with LBT as:

| DRX cycle | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | Ceil((Mpss/sss_sync_w/o_gaps + [NRxBeam] x LPSS/SSS,deact) x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil((Mpss/sss_sync_w/o_gaps + [NRxBeam] x LPSS/SSS,deact) x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil((Mpss/sss_sync_w/o_gaps + [NRxBeam] x LPSS/SSS,deact) x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, LPSS/SSS, deact is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max. When DRX is configured, LPSS/SSS, deact is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS, deact< LPSS/SSS, deact,max.When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement cycles, the UE is not required to determine the availability of SMTC occasions more frequent than once per measurement cycle. NOTE 2: LPSS/SSS, deact,max, = 7 for Max(DRX cycle, measCycleSCell)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS, deact,max = 5 for 40ms<Max(DRX cycle, measCycleSCell)≤320ms, LPSS/SSS, deact,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding LPSS/SSS, deact,max,, the UE is not required to meet the requirements for PSS/SSS detection. |  |

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | This table is the implementation of the agreement from RAN4 #101-bis. |
| Qualcomm | Fine with the highlighted changes, but NOTE 1 and 2 may need some revision based on Issue 2-2-2 |
| Huawei | Depends on the conclusion of issue 2-2-2. |
| Intel | Notes 1 and 2 need revision |
| Apple | Agree with Qualcomm, Huawei and Intel. |

Issue 2-4-7: Intra-frequency measurement requirements with measurement gaps and inter-frequency measurement requirements in FR2-2 with LBT

- Proposals

  - Proposal 1 (Nokia): Use the tables proposed for intra-frequency measurement requirements without measurement gaps as baseline for the definition of intra-frequency measurement requirements with measurement gaps and inter-frequency measurement requirements in FR2-2 with LBT.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Huawei | Some clarification is needed. The proposal is not very clear. |
| YYY |  |
| ZZZ |  |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204193 Introduction of SCell activation with CCA for FR2-2 | Nokia: Tactivation_time should be changed to Tactivation_time_withCCA TL1_RSRP_Measurement_Period_SSB should be TL1_RSRP_Measurement_Period_SSB _CCA Fix the reference to clauses with unlicensed RRM requirements, such as 9.5 -> 9.5A There is a mention to TL1_RSRP_Measurement_Period_CSI-RS: RAN4 didn't define requirements for CSI-RS measurements in Rel-16 NR-U. There are no proposals yet on this issue, therefore it might be better to wait before capturing this in the CR. The terminology FR2 or FR2-2 should be the same in all CRs (depends on issue 2-1-3) |
|  | Company B |
| R4-2204194 Introduction of TCI state switch with CCA for FR2-2 | Nokia: Tactivation_time should be changed to Tactivation_time_withCCA TL1_RSRP_Measurement_Period_SSB should be TL1_RSRP_Measurement_Period_SSB _CCA Fix the reference to clauses with unlicensed requirements, such as 9.5 -> 9.5A There is a mention to TL1_RSRP_Measurement_Period_CSI-RS: RAN4 didn't define requirements for CSI-RS measurements in Rel-16 NR-U. There are no proposals yet on this issue, therefore it might be better to wait before capturing this in the CR. The terminology FR2 or FR2-2 should be the same in all CRs (depends on issue 2-1-3) |
|  | Company B |
| R4-2204542 DraftCR for FR2-2 LBT support in Intra-Frequency measurements | Company A |
|  | Company B |
| R4-2204634 Draft CR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change | Nokia: The applicability of the requirements in this clause could be clarified to indicate that the PSCell is in FR2-2. The terminology FR2 or FR2-2 should be the same in all CRs (depends on issue 2-1-3) L1 shall follow the agreements -> L1 x N The delay uncertainty in obtaining the first available PRACH occasion shall also consider the UL LBT failure, like it is considered in the handover clause for NR-U. Fix the references to clauses with CCA requirements 9.3 -> 9.3A. |
|  | Company B |
| R4-2204728 DraftCR on cell reselection in Idle mode for NR_ext_to_71GHz-Core | Nokia: This topic is also being treated in threads 201 (Rel-15 RRM core maintenance) and in thread (Rel-16 NR-U maintenance). Our view is that we should wait for the decision in the other threads. Furthermore, the change proposed in this CR is impacting NR-U FR1 requirements. If any change is agreed, a new paragraph for it should be created for FR2-2, otherwise there will be no requirements for NR-U FR1. This change collides with the draft CR R4-2205523. |
|  | Company B |
| R4-2204879 Draft CR on RLM and link recovery requirements for FR2-2 unlicensed operation | Nokia: There are no agreements on DRX groups or SSB occasion group. Are there any differences between the scheduling restrictions in this clause, and the ones in 8.5.7.3 for example? If not, we suggest to adopt the same solution as in NR-U: just refer to the licensed clause. |
|  | Huawei: Response to Nokia. Regarding the scheduling restriction, yes there was agreed as follows. So the scheduling restriction for FR2-2 is not completely same as licensed clause. |
|  | - “Introduce scheduling restrictions for one symbol before and one symbol after the measurement resources (SSB, CSI-RS etc.) during L1 measurements for 480/960kHz SCS ” |
| R4-2206007 DraftCR for FR2-2 LBT support in RRC_IDLE and RRC_CONNECTED state mobility requirements | nokia: Is there any reason to introduce void clauses to the specification? We suggest to remove clause 6.1B.1.3, and if the group agrees to define requirements for NR FR2 to NR FR1 (with CCA?), we introduce it after the other clauses. Clarification of L3 when the channel access type is 3 is missing (up to discussions). |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 Scope of the RRM requirements to be defined | Issue 2-1-1: RRM scope Companies’ views: Most of the companies have strong preference to define LBT requirements for CA and NR-DC cases. Agreement: RAN4 will define the LBT requirements for all scenarios which were agreed for FR2-2. Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-1-2: DraftCR Work split The list of Draft CRs and responsible companies was updated Draft CR; Company. DraftCR for FR2-2 LBT support in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state mobility requirements; Intel. DraftCR for FR2-2 LBT support in Radio Link Monitoring and Link recovery procedures; Huawei. DraftCR for FR2-2 LBT support in SCell Activation and Deactivation Delay requirements and Active TCI state switching delay requirements; MTK. DraftCR for FR2-2 LBT support in Intra-Frequency measurements; Nokia. DraftCR for FR2-2 LBT support in Inter-Frequency measurements; Ericsson. DraftCR for FR2-2 LBT support in L1-RSRP measurements for reporting.; Qualcomm. DraftCR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change; vivo. |
|  | Issue 2-1-3: Whether to use “FR2” or “FR2-2” term for CCA related requirements in TS38.133 spec Companies’ views: All companies agreed to capture CCA related requirements using only FR2-2 term. Agreement: Use “FR2-2” term for all CCA related requirements in the spec since currently there is no CCA operation in FR2-1. Recommendations for 2nd round: No further discussion is needed in the second round |
| ### Sub-topic #2-2 General aspects on the RRM requirements | Issue 2-2-1: The LBT-based FR2-2 RRM requirements extension step. Companies’ views: Agreement from RAN4 #101-bis-e meeting says: RAN4 will reflect the LBT failures by extending the RRM requirements by N*SMTC/SSB occasions when there is at least one SMTC/SSB occasion not available at UE within N*SMTC/SSB occasions. At this meeting during the first round discussion all companies agreed that N is RX beam sweeping scaling factor which means that the extension of RRM requirements will happen by the units of additional Rx beam sweeping rounds. Agreement: The step of FR2-2 RRM requirements extension due to missed SMTC/SSB occasions is equal to N SMTC/SSB occasions, where N is RX beam sweeping scaling factor. Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-2-2: How to take into account the LBT failures on the RRM requirements Companies’ views: All companies supported distinguishing groups of N SSB/SMTC occasion and extending RRM requirements by the duration of the group if there was one or more LBT failures withing that group. At the same time some companies raised concerns that the above approach will cause very long measurement periods, so they also supported the option which can avoid unnecessary extension of RRM requirements in some cases, but assumes certain Rx beam sweeping strategy. Also, one company proposed the revised version of the latter option, and this version has not been discussed. Moderator would like to double-check companies views for the following candidate options. Candidate options: Option 1 (Nokia, Qualcomm, Huawei, Intel, vivo, Apple, CATT): The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one SSB/SMTC occasions is not available in the group. Option 2 (Nokia, Intel, vivo, CATT): Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period. If there are no SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period, then only one additional Rx beam sweeping round is needed for measurement. Option 3 (CATT): Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and based on the measurement of the same beam during the measurement period. If there are no SMTC/SSB occasions not available at the UE and based on the measurement of the same beam during the measurement period, then only one additional Rx beam sweeping round is needed for measurement. Recommendations for 2nd round: Confirm Option 1 as acceptable way forward. |
|  | Issue 2-2-3: Maximum number of SMTC occasions not available at the UE Companies’ views: Most of the companies agreed that there is no need to apply scaling to the maximum number of SMTC occasions not available at the UE. Moderator found out that the agreement sounds ambiguous, and companies could have different understanding for the Issue. So, further clarification is needed. Candidate options: Option 1: RAN4 will reuse the FR1 value of maximum number of SMTC occasions not available at the UE considering that for FR2-2 it is the maximum number of SMTC/SSB groups with at least one SMTC/SSB occasion not available at the UE. Option 2: RAN4 will reuse the FR1 value of maximum number of SMTC occasions not available at the UE considering that for FR2-2 it is the total maximum number of SMTC/SSB occasion not available at the UE. Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 2-2-4: Time gap between two successful measurement samples Companies’ views: Companies agreed to further discuss if some limit should be set on the time gap between two successful measurement samples to guarantee performance. Some companies also prefer to get clarifications on the relationship between the maximum numbers of SMTC occasions not available at the UE and the time gap limit between two successful measurement samples. Considering limited time for Rel-17 completion, Moderator prefers not to capture that as an agreement but continue discussion in the 2nd round Candidate options: Option 1: Within the set of measurements any two measurements shall not be separated in time by more than X ms. Option 2: No need to set the limit on the time gap between two successful measurement samples since it is already implicitly set by defining maximum numbers of SMTC occasions not available at the UE Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 2-2-5: Name for RX beam sweeping scaling factor in TS38.133 Companies’ views: All companies agree to use NRxBeam instead of N, to refer to RX beam sweeping scaling factor in TS38.133. Moderator finds the words “in TS38.133” too generic. Do they mean that the agreement should be applied to the whole TS38.133. Clarifications to which requirements this agreement should be applied are needed in the 2nd round. Candidate options: Option 1: The 1st round agreement applies to all FR2-2 RRM requirements which refer to RX beam sweeping scaling factor. Option 2: The 1st round agreement applies to all FR2-2 CCA related RRM requirements which refer to RX beam sweeping scaling factor. Option 3: The 1st round agreement applies to Section 9.2A in TS38.133 to avoid confusion with N - number of candidate positions within an SMTC/SSB occasion. Recommendations for 2nd round: Continue discussion in the 2nd round |
| ### Sub-topic #2-3 RRC_IDLE and RRC_CONNECTED state mobility requirements | Issue 2-3-1: Time period before cell selection in Idle mode Companies’ views: Companies prefer to wait for a decision in the other threads Agreement: Do not consider any changes for 10s time period before cell selection in Idle mode until there will be corresponding agreements in the other threads Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-3-2: Requirements on measurement and evaluation when subject to CCA on the serving cell Companies’ views: Companies have a consensus on the issue Agreement: RAN4 to scale up the parameters Mn, Mp, Mq and the periodicity of the cell selection criterion evaluation by Rx beam scaling factor Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-3-3: Nserv_CCA definition Companies’ views: Companies commented that the issue depends on the decision for other issues. Recommendations for 2nd round: Consider further discussion of the issue directly in the corresponding draft CR |
|  | Issue 2-3-4: Tdetect, Tmeasure and Tevaluate intra-frequency and inter-frequency NR measurements when subject to CCA Companies’ views: Companies commented that the issue depends on the decision for other issues. Recommendations for 2nd round: Consider further discussion of the issue directly in the corresponding draft CR |
|  | Issue 2-3-5: Margins for reselection criteria for inter-frequency and intra-frequency measurements in RRC_IDLE state mobility Companies’ views: Companies have a consensus on the issue Agreement: RAN4 to reuse legacy FR2 margins for reselection criteria for inter-frequency and intra-frequency measurements in RRC_IDLE state mobility Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-3-6: Handover requirements Companies’ views: Companies have a consensus on the issue Agreement: Develop new handover requirements for FR2-2 with CCA, at least for the following scenarios: NR FR2-2 – NR FR2-2 Handover and NR FR1 – NR FR2-2 Handover Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-3-7: Handover from NR FR2-2 with CCA to NR FR1 Companies’ views: Companies have a consensus on the issue Agreement: Extend the applicability of the requirements in clause 6.1.1.3 in TS 38.133 to support handover from NR FR2-2 with CCA to NR FR1 (without CCA) Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-3-8: TIU is needed for Type 3 channel access Companies’ views: Only one company commented on this issue. More discussion is needed Candidate options: Proposal 1: For handover to cells in FR2-2 with LBT, no extension of TIU is needed for Type 3 channel access, defined in TS 37.213: L3 = 0 Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 2-3-9: RRC re-establishment with CCA and RRC release with redirection Companies’ views: Companies commented that the revision may be needed based on the decision for other issues. Recommendations for 2nd round: Consider further discussion of the issue directly in the corresponding draft CR |
| ### Sub-topic #2-4 Measurement procedure | Issue 2-4-1: Intra-frequency time index detection in FR2-2 unlicensed Companies’ views: Companies have a consensus on the issue Agreement: RAN4 to postpone the definition of requirements for intra-frequency time index detection in FR2-2 unlicensed, until there is an agreement for intra-frequency time index detection in FR2-2 licensed Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-4-2: Number of cells and number of SSB for intra-frequency measurements Companies’ views: Companies have a consensus on the issue Agreement: For FR2-2 intra-frequency measurement requirements with LBT, reuse the FR2 requirements on the minimum number of cells and number of SSBs that the UE shall be capable of performing measurements in each intra-frequency Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-4-3: Number of cells and number of SSB for inter-frequency measurements Companies’ views: Companies have a consensus on the issue Agreement: For FR2-2 inter-frequency measurement requirements with LBT, reuse the FR2 requirements on the minimum number of cells and number of SSBs that the UE shall be capable of performing measurements in each inter-frequency layer. Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-4-4: Number of samples for intra-frequency and Inter-frequency Measurement Requirements Companies’ views: Companies agree that RAN4 need to wait for decision on the power classes to be supported in FR2-2. At the same time one company prefers to capture at least some values in brackets in the draftCR. Agreement: Wait for decision on the power classes to be supported in FR2-2 before agreeing on the number of samples Mpss/sss_sync_w/o_gaps_CCA , Mmeas_period_w/o_gaps_CCA, Mpss/sss_sync_with_gaps_CCA, Mmeas_period_ with_gaps_CCA, Mpss/sss_sync_inter_CCA, MSSB_index_inter_CCA, Mmeas_period_inter_CCA for unlicensed operation in FR2-2 Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-4-5: Time for PSS/SSS detection Companies’ views: Companies commented that the revision is needed based on the decision for other issues. Recommendations for 2nd round: Consider further discussion of the issue directly in the corresponding draft CR |
|  | Issue 2-4-6: Time for PSS/SSS detection for deactivated SCell Companies’ views: Companies commented that the revision is needed based on the decision for other issues. Recommendations for 2nd round: Consider further discussion of the issue directly in the corresponding draft CR |
|  | Issue 2-4-7: Intra-frequency measurement requirements with measurement gaps and inter-frequency measurement requirements in FR2-2 with LBT Companies’ views: Only one company commented on the Proposal asking for clarification. Moderator finds the proposal very generic and recommends not to spend time on finding an agreement for that but discuss it directly in the corresponding draft CR when it will be available. Recommendations for 2nd round: No further discussion is needed in the second round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2204193 Introduction of SCell activation with CCA for FR2-2 | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |
| R4-2204194 Introduction of TCI state switch with CCA for FR2-2 | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |
| R4-2204542 DraftCR for FR2-2 LBT support in Intra-Frequency measurements | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |
| R4-2204634 Draft CR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |
| R4-2204728 DraftCR on cell reselection in Idle mode for NR_ext_to_71GHz-Core | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |
| R4-2204879 Draft CR on RLM and link recovery requirements for FR2-2 unlicensed operation | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |
| R4-2206007 DraftCR for FR2-2 LBT support in RRC_IDLE and RRC_CONNECTED state mobility requirements | Since the discussion in the 1st round of comments collection was very limited, moderator will recommend “to be revised” status for this CR and encourage companies to leave their comments in the 2nd round. |

## Discussion on 2nd round (if applicable)

### Sub-topic 2-2. General aspects on the RRM requirements

Sub-topic description: In this sub-topic the general aspects of LBT-related RRM specification changes are discussed mainly focusing on the details of how to take into account the LBT failures on the RRM requirements

Open issues and candidate options before e-meeting:

Issue 2-2-2: How to take into account the LBT failures on the RRM requirements

- Proposals

  - Option 1 (Nokia, Qualcomm, Huawei, Intel, vivo, Apple, CATT): The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one SSB/SMTC occasions is not available in the group.

  - Option 2 (Nokia, Intel, vivo, CATT): Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period. If there are no SMTC/SSB occasions not available at the UE and consecutively spaced by N SMTC/SSB occasions during the measurement period, then only one additional Rx beam sweeping round is needed for measurement.

  - Option 3 (CATT): Number of additional Rx beam sweeping rounds is equal to the number of SMTC/SSB occasions not available at the UE and based on the measurement of the same beam during the measurement period. If there are no SMTC/SSB occasions not available at the UE and based on the measurement of the same beam during the measurement period, then only one additional Rx beam sweeping round is needed for measurement.

- Recommended WF

  - Confirm Option 1 as acceptable way forward.

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| MTK | Fine with Option 1. Not much difference between Option 1 and Option 2. |
| CATT | We prefer Option 1 and Option 3. |
| Qualcomm | We support Option 1 in general, but we just noticed that “… when at least one SSB/SMTC occasion is not available…” may not be the correct wording. Since the UE is sweeping it’s Rx beam, it is likely that there will always be some beams which may not receive SSB/SMTC, not because of CCA failure but because of UE beam pointing to a different direction. This would mean that this condition would be always met, and the extension would apply irrespective of whether CCA failure happens or not. Two alternate versions can be considered to handle this: Option 1a: The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one none of the SSB/SMTC occasions is not available in the group. Option 1b: The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one SSB/SMTC occasion is not available in the group is not transmitted by the gNb. Option 1a implies that the UE scanned through all the beams but couldn’t find a single SSB, perhaps because of CCA failure. However, the downside is that if there’s a CCA failure in the main beam, but not in the adjacent beam, the UE may proceed with the sub-optimal beam. Option1b may capture the correct behaviour but the question is how to define the UE behaviour in this case. |
| Huawei | We prefer option 1. As commented in first round, from requirements perspective, we can not defined whether the measurement are from same beam for different beams. But we also recognize what QC commented is valid. The essential question is whether UE can tell whether the low measurement result is due to LBT failure or because the beam is targeting to the wrong direction. Companies had such discussion in Rel-16 NR-U, but no |
| Ericsson | Support Option1. Regarding Qualcomm’s proposal, it is a valid problem but we’re not sure it shall be answered in this issue. |
| Nokia | Option 1 is acceptable, but we understand Qualcomm’s concern. Our view is that defining the requirement using the definition in Option 1b is not possible, because the availability of the SSB/SMTC group is determined at the UE, and the UE does not know if the SSB is not available because of CCA, due to low SINR or because the UE beam is targeting a different direction. Option 1a is confusing, so we propose the following alternative: Option 1c: The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when none of the SSB/SMTC occasions are not available in the group. |
| Apple | Follow Monday’s GTW agreement. |
| Intel | The GTW agreement: - Agreements - The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC occasions group consists of N consecutive SSB/SMTC occasions. An SSB/SMTC occasions group is not available, when at least one SSB/SMTC occasion in the group is not transmitted by the gNb. - The definition of SSB/SMTC occasion follows Rel-16 NR-U definition - FFS how to introduce the test case - FFS if agreement applies to RLM OOS and BFD |

Issue 2-2-3: Maximum number of SMTC occasions not available at the UE

- Proposals

  - Option 1: RAN4 will reuse the FR1 value of maximum number of SMTC occasions not available at the UE considering that for FR2-2 it is the maximum number of SMTC/SSB groups with at least one SMTC/SSB occasion not available at the UE.

  - Option 2: RAN4 will reuse the FR1 value of maximum number of SMTC occasions not available at the UE considering that for FR2-2 it is the total maximum number of SMTC/SSB occasion not available at the UE.

- Recommended WF

  - Continue discussion in the 2nd round

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| MTK | The max. number is derived from the max. acceptable delay in our view. Thus, prefer to Option 1 but ok with Option 2. |
| CATT | We prefer option1 to support limit the number of SMTC/SSB groups. In fact, we believe it is related to issue 2-2-2. In order to ensure measurement periods not be very long, it is sensible to set the maximum number of SMTC/SSB groups. Because it means the number of LBT failures based on the measurement of the same beam has the upper limit, which determines the measurement periods. If we only limit the total maximum number of SMTC/SSB occasion but not have limitation on the number of SMTC/SSB groups, it will also cause long measurement periods in some cases, such as all of LBT failures occur on the measurement of the same beam. |
| Qualcomm | Same comment as in Issue 2-2-2 for Option 1. |
| Ericsson | Support Option 1 |
| Nokia | This issue depends on 2-2-2 and 2-2-4. We can only agree to reusing the FR1 values, if there is a time limit between two successful measurements. |
| Apple | Option 1 is more straightforward. Option 2 could work, but takes more time to decide the maximum number of SMTC/SSB occasions. |
| vivo | Support Option 1. To avoid the long measurement delay, we understand FR2-2 can reuse the value of FR1 for LBT for maximum numbers of SMTC/SSB groups with at least one SMTC/SSB occasion in the group is not transmitted by the gNB. |
| Intel | Support Option 1 |

Issue 2-2-4: Time gap between two successful measurement samples

- Proposals

  - Option 1: Within the set of measurements any two measurements shall not be separated in time by more than [X] ms.

  - Option 2: No need to set the limit on the time gap between two successful measurement samples since it is already implicitly set by defining maximum numbers of SMTC occasions not available at the UE.

- Recommended WF

  - Continue discussion in the 2nd round

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| CATT | We prefer option 1 as a direct approach to set the limit on the time gap. For option 2, it need more clarification to explain how the maximum numbers of SMTC occasions not available at the UE implicitly limit the time gap, and it is also related to the conclusion on issue 2-2-3. |
| Qualcomm | We think Option 1 is a good approach. But may need further consideration. |
| Ericsson | Support Option 1. |
| Nokia | Option 1, but it needs further consideration. Maybe the time gap will need to be defined case by case, since the time between samples will depend on DRX configuration, SMTC period, number of SMTC/SSB or SMTC/SSB groups not available at the UE, etc. |
| Apple | We support Option 1. |
| vivo | Support Option 1. |
| Intel | Ok with Option 1 |

Issue 2-2-5: Name for RX beam sweeping scaling factor in TS38.133

- Proposals

  - Option 1: The 1st round agreement applies to all FR2-2 RRM requirements which refer to RX beam sweeping scaling factor.

  - Option 2: The 1st round agreement applies to all FR2-2 CCA related RRM requirements which refer to RX beam sweeping scaling factor.

  - Option 3: The 1st round agreement applies to Section 9.2A in TS38.133 to avoid confusion with N - number of candidate positions within an SMTC/SSB occasion

- Recommended WF

  - Continue discussion in the 2nd round

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| MTK | Our understanding is Option 1. Fine to update the notation of N to avoid confusion with N - number of candidate positions within an SMTC/SSB occasion, but it seems not only for Section 9.2A in TS38.133. |
| CATT | OK with option 1 |
| Qualcomm | Fine with Option 1 |
| Ericsson | Fine with Option 1. |
| Nokia | Option 2. Our intention with the proposal is to avoid confusion with the number of candidate positions, which is a concept that refers to operation with CCA, therefore, our view is that we should avoid confusion in clauses that refer to operation with CCA, at least in clauses 9.2A and 9.3A. If we agree with Option 1, the specification impact might be huge since many FR2-2 requirements are reusing FR2 requirements. |
| Apple | Option 1 for consistency between licensed and unlicensed operation. |
| vivo | Option 2. Share the same understanding with Nokia. |
| Intel | Support Option 2. With Option 1 there will be huge spec impact since all the FR2 requirements will also need to be changed. |

### Sub-topic 2-3. RRC_IDLE and RRC_CONNECTED state mobility requirements

Issue 2-3-8: TIU is needed for Type 3 channel access

- Proposals

  - Proposal 1: For handover to cells in FR2-2 with LBT, no extension of TIU is needed for Type 3 channel access, defined in TS 37.213: L3 = 0

- Recommended WF

  - Continue the discussion in the 2nd round.

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Qualcomm | Needs further discussion. What should be the correct UE behaviour in this case? |
| MTK | We can support Proposal 1 because to our understanding Type 3 channel cases with no LBT thus the delay due be LBT failures are not needed. |
| Ericsson | Support Proposal 1 for the same reason by MTK. |
| Nokia | Proposal 1 is ok. We didn’t see any objection for this proposal in the first round. To Qualcomm: in this case, for channel access type 3, there is no UL LBT. L3 is the delay caused by UL LBT failures, therefore, we propose it to be set to 0. |
| Intel | Support Proposal 1 |

### Sub-topic 2-5. CRs comment collection

Issue 2-5-1: [R4-2204193-> R4-2206925] Introduction of SCell activation with CCA for FR2-2

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Mediatek | For R4-2204193 Scell activation, the follow revisions are made, based on the received comment - revise Tactivation_time to be Tactivation_time_withCCA - Remove TL1_RSRP_Measurement_Period_CSI-RS and revise TL1_RSRP_Measurement_Period_SSB as TL1_RSRP_Measurement_Period_SSB _CCA, and fix the reference as 9.5A.4.1 - Use FR2-2 rather than FR2 |
| Intel | Thanks for the revisions. We have some further comments: R4-2206925 - N should be replaced with NRxBeam - For L4,1, L5,1, L4,2, L5,2 definition please follow the GTW agreement for Issue 2-2-2 - For L4,1 max, L5,1 max, L4,2 max, L5,2 max FR1 values can be reused based on agreement for Issue 2-2-3 |
| Mediatek | Thanks for the comments. The final draft is available and revised according to the further comments. - R4-2206925 (Revised R4-2204193): - N is replaced with NRxBeam - For L4,1, L5,1, L4,2, L5,2, follow the definition of "SSB/SMTC occasions groups not available at UE". It would be good to capture the detail definition on some common section. - For L4,1 max, L5,1 max, L4,2 max, L5,2 max FR1 values are reused based on agreement for Issue 2-2-3 |

Issue 2-5-2: [R4-2204194-> R4-2206926] Introduction of TCI state switch with CCA for FR2-2

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Mediatek | For R4-2204194 TCI switch, the follow revisions are made, based on the received comment - No use of Tactivation_time - Remove TL1_RSRP_Measurement_Period_CSI-RS and revise TL1_RSRP_Measurement_Period_SSB as TL1_RSRP_Measurement_Period_SSB _CCA, and fix the reference as 9.5A.4.1 - Use FR2-2 rather than FR2 |
| Intel | Thanks for the revisions. We have some further comments: R4-2206926 - Similar comment as in the 1st round – for NR-U we don’t have CSI-RS based L1-RSRP measurement. Better not to consider option for CSI-RS based L1-RSRP measurement in TOuk definition in section 8.10A.3 - For LMAC,known, LMAC,unknown definition for FR2-2 please follow the GTW agreement for Issue 2-2-2. LMAC,known,max LMAC,unknown,max can be kept unchanged according to agreement for Issue 2-2-3 |
| Mediatek | Thanks for the comments. The final draft is available and revised according to the further comments. - R4-2206926 (Revised R4-2204194): - Remove CSI-RS. - For LMAC,known, LMAC,unknown , follow the definition of "SSB/SMTC occasions groups not available at UE". It would be good to capture the detail definition on some common section. - For LMAC,known,max LMAC,unknown,max, FR1 values are reused. (not changed) |

Issue 2-5-3: [R4-2204542-> R4-2206927] DraftCR for FR2-2 LBT support in Intra-Frequency measurements

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Intel | We think that NRxBeam factor is still needed in tables 9.2A.5.1-5, 9.2A.5.1-5, 9.2A.5.2-3, 9.2.5.2-4, 9.2A.6.1-3, 9.2A.6.2-2. Parameters LPSS/SSS, LPSS/SSS,deact etc reflect the number of beam sweeping rounds but not the corresponding duration. The duration is NRxBeam x LPSS/SSS x SMTC period As a simple example let’s take expression for no DRX in Table 9.2A.5.1-5 with removed [NRxBeam]: max(600ms, ceil((Mpss/sss_sync_w/o_gaps_CCA + LPSS/SSS) x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra Here, with LPSS/SSS< LPSS/SSS,max and LPSS/SSS,max =[7] (Notes 2 and 3 from the same table) the maximum possible extension due to LBT is only 7 SMTC periods (assuming Kp = Klayer1_measurement=1) which is less than even one beam sweeping round. |
| Nokia | Thanks for your comment. We agree with your remarks |

Issue 2-5-4: [R4-2204634-> R4-2206928] Draft CR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| vivo | We did following modifications: 1. Based on the GTW agreements, update the wording related to L1 2. Further clarify the applicability of the requirements in all clauses is that PCell is in FR1 and PSCell is in FR2-2. 3. Change the channel access type that has no UL CCA from ‘Type 2C’ to ‘Type 3’ for FR2-2 4. Fix the typo 'Conditional' in the title of clause 8.11.C.2. |
| Intel | Thanks for the revisions. We have some further comments: - Limit for L3 need to be defined in both 8.9A.2 and 8.11C.2 - N should be replaced with NRxBeam - Replace L1=TBD and L2=TBD by L1 max=TBD and L2 max=TBD |
| vivo | Thanks for your comments. We agree with your remarks. |

Issue 2-5-5: [R4-2204728-> R4-2206929] DraftCR on cell reselection in Idle mode for NR_ext_to_71GHz-Core

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |

Issue 2-5-6: [R4-2204879-> R4-2206930] Draft CR on RLM and link recovery requirements for FR2-2 unlicensed operation

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Nokia | Regarding our comment on the 1st round, we assume that these changes will be captured in the FR2-2 requirements, therefore we think that there is no need to capture them again in the requirements with CCA, we can just refer to the clause with the FR2-2 scheduling restrictions. Apart from that: For RLM in sync / CBD: N should be replaced by N_RxBeam. Lin / LCBD is now referring to the SSB occasion groups, which, by definition, already takes into account the Rx beam scaling factor. Therefore, N (or N_RxBeam) should not multiply “Lin / LCBD”. Otherwise N (or N_RxBeam) would be accounted for twice in the evaluation period. The definition of SSB occasion group should also follow the agreements on the GTW. It is not clear whether the agreements apply to RLM OOS and BFD, therefore we suggest to remove these requirements. |
| Huawei | Thanks for the comments. Please find the revision in: The changes are summarized as follows: - The scheduling restriction in link recovery is referred to licensed clause - N is replaced by N_RxBeam, which is also removed from Lin/LCBD - Requirements for RLM and BFD are kept as TBD - The definition of unavailable SSB occasion groups are upadated. |
| Intel | Thanks for new revision. We have the same comment as in other CRs threads. Parameters Lin, LCBD should still be multiplied by NRxBeam, since Lin, LCBD only reflect the number of beam sweeping rounds but not the corresponding duration – the duration is NRxBeam x Lin x TSSB/DRX. |
| Huawei | Thanks for the comments. It makes sense to me |

Issue 2-5-7: [R4-2206007-> R4-2206931] DraftCR for FR2-2 LBT support in RRC_IDLE and RRC_CONNECTED state mobility requirements

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Intel | The changes in relation to the original version follow the agreements/tentative agreements/comments from the 1st round email discussion. |
| Nokia | We have the following comments: In Tables 6.2.1A.2.1-1 and 6.2.1A.2.1-2 we would prefer to keep the definition of K1 and K2,i for FR1 since the concept of “SMTC group” does not exist in FR1. Therefore, in our view, K1 and K2,i should not be used for FR2-2 requirements. Agree. K1 and K2,i were replaced by K3 and K4,i Apart from that, “8” is multiplying K1 in the table and we think it shouldn’t be, since the Rx beam sweeping is already considered in the definition of a SMTC group, if it is also multiplying the number of groups not available at the UE, it will be taken into account twice. Disagree. K1 is only the number of additional groups (or beam sweeping rounds), while we need to reflect the total duration for that groups: total_duration_for_extension = number_of_groups * duration_of_one_group, where number_of_groups = K1 duration_of_one_group = number_of_SMTC_occasion_in_group * SMTC_period, where number_of_SMTC_occasion_in_group = NRxBeam total_duration_for_extension = K1 * NRxBeam * SMTC_period E.g. if we need 1 additional beam sweeping round, then K1=1. 1 * Tsmtc does not reflect one beam sweeping round, but 1 * 8 * Tsmtc does. 8 should be in []. Agree. Moreover, I think it is better to replace 8 by NRxBeam with further clarification that NRxBeam is equal to [8] N should be NRxBeam. Agree Same comments are valid for L1. Agree |
| Intel | Thanks for your review and comments. New revision can be found at Please see my responses on your comments inline in green |
| Nokia | Thanks for addressing our comments. Regarding K1, you are correct. It is only the number of additional groups, but it is not reflecting the duration of a group. I apologize for misinterpreting it. I will also fix it in Nokia’s draft CR. In general, the changes look fine, but we still have some comments. On L1 for FR2-2: I think we cannot directly replace it by L2 for FR2-2, because it is defined in the same clause as: L2 is the consecutive number of SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failures. Maybe you could replace it for L1’? Or L3? We also think that a new table will be needed with L_max for FR2-2. The values could be in [], or TBD. An Editor’s note would also be fine. |
| Intel | Thanks for your comments The new revision was uploaded to L2 was replaced by L1’ Regarding the L_max for FR2-2 I prefer to keep the existing table for now since it is aligned with the agreement for Issue 2-2-3. |

## Summary for 2nd round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| ### Sub-topic #2-2 General aspects on the RRM requirements | Issue 2-2-2: How to take into account the LBT failures on the RRM requirements Companies’ views: Most of the companies supported Option 1. During the discussion the following concern was expressed by Qualcomm and other companies: since the UE is sweeping it’s Rx beam, it is likely that there will always be some beams which may not receive SSB/SMTC, not because of CCA failure but because of UE beam pointing to a different direction. The following options were proposed by Qualcomm: Option 1a: The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one none of the SSB/SMTC occasions is not available in the group. Option 1b: The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC group consists of N SSB/SMTC occasions, and it is not available when at least one SSB/SMTC occasion is not available in the group is not transmitted by the gNB Issue was discussed in GTW session and the following agreement was captured Agreement: - The time is extended by the number of SSB/SMTC occasions groups not available at UE. An SSB/SMTC occasions group consists of N consecutive SSB/SMTC occasions. An SSB/SMTC occasions group is not available, when at least one SSB/SMTC occasion in the group is not transmitted by the gNb. - The definition of SSB/SMTC occasion follows Rel-16 NR-U definition - FFS how to introduce the test case - FFS if agreement applies to RLM OOS and BFD. Recommendations for WF: Capture the agreement in the WF. |
|  | Issue 2-2-3: Maximum number of SMTC occasions not available at the UE Companies’ views: Companies have a consensus on the issue. Agreement: - RAN4 will reuse the FR1 value of maximum number of SMTC occasions not available at the UE considering that for FR2-2 it is the maximum number of SMTC/SSB groups with at least one SMTC/SSB occasion not available at the UE. Recommendations for WF: Capture the agreement in the WF. |
|  | Issue 2-2-4: Time gap between two successful measurement samples Companies’ views: Companies have a consensus on the issue. Agreement: - Within the set of measurements any two measurements shall not be separated in time by more than X ms Recommendations for WF: Capture the agreement in the WF. |
|  | Issue 2-2-5: Name for RX beam sweeping scaling factor in TS38.133 Companies’ views: Companies have different views on the issue. The proponents of Option 1 prefer to have consistency between CCA-based and non-CCA-based operation. The proponents of Option 2 argue that Option 1 will require to change all FR2 requirements, which is a huge spec impact and is beyond the initial intention of avoiding confusion with the number of candidate positions. Candidate options: Option 1: The 1st round agreement applies to all FR2-2 RRM requirements which refer to RX beam sweeping scaling factor. Option 2: The 1st round agreement applies to all FR2-2 CCA related RRM requirements which refer to RX beam sweeping scaling factor. Option 3: The 1st round agreement applies to Section 9.2A in TS38.133 to avoid confusion with N - number of candidate positions within an SMTC/SSB occasion. Recommendations for WF: Continue email discussion in the final round and capture the final agreement in the WF. |
| ### Sub-topic #2-3 RRC_IDLE and RRC_CONNECTED state mobility requirements | Issue 2-3-8: TIU is needed for Type 3 channel access Companies’ views: Companies have a consensus on the issue. Agreement: - For handover to cells in FR2-2 with LBT, no extension of TIU is needed for Type 3 channel access, defined in TS 37.213: L3 = 0 Recommendations for WF: Capture the agreement in the WF |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR extension to 71 GHz RRM requirements (Part 2) | Intel | WF is supposed to capture the agreements and open issues |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2204877 | Draft CR on interruption requirements for FR2-2 | Huawei, HiSilicon | Agreeable |  |
| R4-2204541 | Draft CR - Correction on BWP switch delay for dormant BWP in FR2-2 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2204193 | Introduction of SCell activation with CCA for FR2-2 | MediaTek Inc. | To be revised |  |
| R4-2204194 | Introduction of TCI state switch with CCA for FR2-2 | MediaTek Inc. | To be revised |  |
| R4-2204542 | DraftCR for FR2-2 LBT support in Intra-Frequency measurements | Nokia, Nokia Shanghai Bell | To be revised |  |
| R4-2204634 | Draft CR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change | vivo | To be revised |  |
| R4-2204728 | DraftCR on cell reselection in Idle mode for NR_ext_to_71GHz-Core | Ericsson | To be revised |  |
| R4-2204879 | Draft CR on RLM and link recovery requirements for FR2-2 unlicensed operation | Huawei, HiSilicon | To be revised |  |
| R4-2206007 | DraftCR for FR2-2 LBT support in RRC_IDLE and RRC_CONNECTED state mobility requirements | Intel | To be revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2206925 | Introduction of SCell activation with CCA for FR2-2 | MediaTek Inc. | Agreeable | Changes related to final agreement on Issue 2-2-5 will be captured in Big CR by moderator |
| R4-2206926 | Introduction of TCI state switch with CCA for FR2-2 | MediaTek Inc. | Agreeable |  |
| R4-2206927 | DraftCR for FR2-2 LBT support in Intra-Frequency measurements | Nokia, Nokia Shanghai Bell | Agreeable | Changes related to final agreement on Issue 2-2-5 will be captured in Big CR by moderator |
| R4-2206928 | Draft CR for FR2-2 LBT support in requirements for PSCell addition and release delay, PSCell change and Conditional PSCell change | vivo | Agreeable |  |
| R4-2206929 | DraftCR on cell reselection in Idle mode for NR_ext_to_71GHz-Core | Ericsson | Not Pursued |  |
| R4-2206930 | Draft CR on RLM and link recovery requirements for FR2-2 unlicensed operation | Huawei, HiSilicon | Agreeable | Changes related to final agreement on Issue 2-2-5 will be captured in Big CR by moderator |
| R4-2206931 | DraftCR for FR2-2 LBT support in RRC_IDLE and RRC_CONNECTED state mobility requirements | Intel | Agreeable | Changes related to final agreement on Issue 2-2-5 will be captured in Big CR by moderator |
| R4-2206924 | WF on NR extension to 71 GHz RRM requirements (Part 2) | Intel Corporation | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| Nokia | Erika Almeida | erika.almeida@nokia.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| Ericsson | Ming Li | ming.l.li@ericsson.com |
| Apple | Steven Chen | steven.x.chen@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)