3GPP TSG-RAN WG4 Meeting # 102-e												R4-2207074

Electronic Meeting, 21 February – 03 March 2022

Agenda item:			10.23

Source:	Moderator (Nokia)

Title:	Email discussion summary for [102-e][233] NR_IIOT_URLLC_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

This email discussion cover following agenda items:

  - Enhanced IIoT and URLLC support	[NR_IIOT_URLLC_enh]

    - General	[NR_IIOT_URLLC_enh-Core]

    - RRM core requirements	[NR_IIOT_URLLC_enh-Core]

      - Propagation delay compensation enhancements	[NR_IIOT_URLLC_enh-Core]

      - Reference point for Te requirements	[NR_IIOT_URLLC_enh-Core]

      - Others	[NR_IIOT_URLLC_enh-Core]

3 Main topics have been identified for discussion:

- Propagation Delay Compensation Enhancements

- timing reference point for UE UL timing

- UE features for enhanced IIoT and URLLC

Additionally, a number of companies have contributed with simulation results. They will be collected separately.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: all 3 topics are open for discussion

- 2nd round: TBA

Earlier agreed documents

RAN4#101-bis

- R4-2202783, WF on NR_IIOT_URLLC_enh_RRM

RAN4#101:

- R4-2120335, WF on NR_IIOT_URLLC_enh_RRM

RAN4#100bis:

- R4-2115371, WF on NR_IIOT_URLLC_enh_RRM

RAN4#99:

- R4-2108368, WF on RRM for NR IIoT and URLLC,

# Topic #1: Propagation Delay Compensation Enhancements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203655 | Nokia, Nokia Shanghai Bell | - RAN1 evaluate the PDC RTT error budget with PRS measurement accuracy under fading channel condition. - At least include the AWGN channel and the fading channel condition defined in 10.1.25.2-2 in TS 38.133 v17.3.0 for PDC RTT with PRS. Rel-17 ePos 1 sample PRS measurement accuracy is based on the BW condition “PRS BW (RBs) ≥ 48”. - Reuse the PRS measurement accuracy requirement based on 4 samples defined in 10.1.25 in TS 38.133 v17.3.0 for Rel-17 PDC RTT with PRS. - The PRS measurement period requirement for PDC RTT is TUERxTx, PRS=CSSF*NPRSslotN'Lavailable_PRSN*Nsample-1*Teffect+Tlast - where - Teffect= TTPRS*TPRS The TRS measurement period requirement for PDC RTT is TUERxTx,TRS=Nsample *TTRS, period - where - Nsample is the number of TRS measurement samples for PDC RTT. - TTRS, period is the periodicity of the TRS specific for PDC RTT UE Rx-Tx time difference measurement. The TRS resource number has minor effect on the TUE-RX error. The TRS measurement sample number has minor effect on the TUE-RX error. The TRS TUE-RX error changes significantly between AWGN and TDL-A. Define UE Rx-Tx time difference measurement accuracy for TRS with both AWGN and TDL-A channels. RAN4 should define test cases for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS. [Moderator: Simulation results will be collected separately] |
| R4-2204306 | OPPO | Observation 1: The goal for low latency positioning enhancement in Rel-17 is to keep the existing Rel-16 accuracy requirements. Proposal 1: For PRS-based UE Rx-Tx time difference measurement, accuracy requirements from Rel-16 spec could be reused as baseline for RTT-based PDC. Proposal 2: Not discuss the number of PRS samples, if only accuracy requirements will be specified. Proposal 3: 60kHz and FR2 should be excluded for gNB Rx-Tx time difference accuracy requirements. |
| R4-2204472 | Qualcomm Incorporated | Observation 1: Simulation results for UE Rx-Tx time difference using TRS are shown in Table 1. The results do not include any margin to account for UE Rx/Tx group delay calibration error. Observation 2: The physical reference points for RTT measurements are at the antenna or antenna connector, while the reference point for ReferenceTimeInfo is an unspecified point at the network. ReferenceTimeInfo may not account for “RF propagation delay” at the gNB. If the two reference points are different and the delay between them is not compensated, PDC accuracy may be degraded. [Moderator: Simulation results will be collected separately] |
| R4-2204647 | vivo | Proposal 1: Accuracy requirements for RTT-based PDC with PRS are based on measurements with one sample. Proposal 2: Accuracy requirements for RTT-based PDC with TRS are based on measurements with one sample. Proposal 3: No measurement period requirements are specified for RTT-based PDC. Proposal 4: AWGN channel is assumed for defining accuracy requirements for PDC measurement. Additional LOS channel may be considered. Proposal 5: Test cases are discussed after completion of accuracy requirements. Observation 1: 60kHz and 120kHz SCS for FR2 are not excluded explicitly for PDC use cases. |
| R4-2204648 | vivo | Observation 1: Measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel. Observation 2: Measurement accuracy can be improved with 4 samples compared to 1 sample measurement under TDL-A fading channel. [Moderator: Simulation results will be collected separately] |
| R4-2205389 | Huawei, HiSilicon | Proposal 1: Measurement requirements for UE Rx-Tx are defined based on single-shot measurement. Proposal 2: Measurement requirements for UE Rx-Tx are defined based on AWGN. Proposal 3: Measurement requirements for TRS based UE Rx-Tx are defined based on 2-slot TRS configuration for FR1 at least. Proposal 4: Inform RAN2 about the RAN4 agreements on report mapping for UE and gNB Rx-Tx. |
| R4-2205390 | Huawei, HiSilicon | CR on requirements for UE Rx-Tx measurement for PDC |
| R4-2205418 | Ericsson | Observation 1: The 15 and 30 kHz SCS from RAN1 LS were for evaluation using control-to-control and smart grid set of requirements. The final LS states no SCS limitations. Proposal 1: Requirements for PDC shall be stated for all SCS. Observation 2: The control-to-control and smart grid set of evaluation requirements can be fulfilled with rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements. Proposal 2: Reuse rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements, for UE Rx-Tx time difference measurement accuracy based on PRS. Proposal 3: In order to facilitate reuse of rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements, for UE Rx-Tx time difference measurement accuracy based on PRS we can base PDC requirements on 4 samples. Observation 3: Observation 3: Our simulation results are reported in R4-2205419, Simulation results for Propagation Delay Compensation [5]. [Moderator: Simulation results will be collected separately] |
| R4-2205419 | Ericsson | [Moderator: Simulation results will be collected separately] |
| R4-2205815 | Nokia, Nokia Shanghai Bell | draftCR on requirements for UE Rx-Tx measurement for propagation delay compensation |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on the submitted papers from the companies and the WF agreed in last meeting following 4 sub-topics have been identified for propagation delay compensation enhancements for discussion:

- UE Rx-Tx time difference measurement requirement for PRS.

  - Issue 1-1: PRS measurement period requirement for PDC RTT

  - Issue 1-2: Define UE Rx-Tx time difference measurement requirement for PRS requirements for FR2

  - Issue 1-3: Define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz

  - Issue 1-4: Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS

  - Issue 1-5: Are simulations needed for defining accuracy requirements

  - Issue 1-6: Number of samples assumed for deriving the accuracy requirements

- UE Rx-Tx time difference measurement requirement for TRS.

  - Issue 1-7: TRS measurement period requirement for PDC RTT

  - Issue 1-8: Channel conditions for which RAN4 will develop requirements

  - Issue 1-9: Define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2

  - Issue 1-10: Define UE Rx-Tx time difference measurement requirement for TRS requirements for 60KHz

  - Issue 1-11: TRS resource number used for developing UE requirements

  - Issue 1-12: Number of samples assumed for deriving the accuracy requirements

- Inform RAN2 about the RAN4 agreements on report mapping for UE and gNB Rx-Tx.

  - Issue 1-13: LS to RAN2 group for Rel-17 enhanced IIOT/URLLC

- RAN4 to start test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS.

  - Issue 1-14: Test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS

Some issues identified for each sub-topic needs to be resolved in this meeting to progress the further work in RAN4 related to performance and simulations.

### Sub-topic 1-1

Sub-topic description: UE Rx-Tx time difference measurement requirement for PRS

Open issues and candidate options before e-meeting:

Issue 1-1: PRS measurement period requirement for PDC RTT

- Proposals

  - Option 1: (Nokia Proposal 3)

- TUERxTx, PRS=CSSF*NPRSslotN'Lavailable_PRSN*Nsample-1*Teffect+Tlast

- where

- Teffect= TTPRS*TPRS

  - Option 2: Other

- Recommended WF

  - More discussion needed

Issue 1-2: Define UE Rx-Tx time difference measurement requirement for PRS requirements for FR2

(decision needed because of possible simulations)

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed

Issue 1-3: Define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz

(decision needed because of possible simulations)

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed

Issue 1-4: Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS

(decision needed because of possible simulations)

- Proposals

  - Option 1: Rel-16 only

  - Option 2: Rel-17 only

  - Option 3: Rel-16 and Rel-17

- Recommended WF

  - More discussion needed

Issue 1-5: Are simulations needed for defining accuracy requirements

(decision needed because of possible simulations)

- Proposals

  - Option 1: No, RAN4 reuse PDC RTT accuracy from R16 defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method

  - Option 2: Yes

- Recommended WF

  - More discussion needed

Issue 1-6: Number of samples assumed for deriving the accuracy requirements

(open but will be decided based on simulation results)

- Proposals

  - Option 1: 1 sample

  - Option 2: 4 samples

  - Option 3: other

- Recommended WF

  - More discussion needed

### Sub-topic 1-2

Sub-topic description: UE Rx-Tx time difference measurement requirement for TRS

Open issues and candidate options before e-meeting:

Issue 1-7: TRS measurement period requirement for PDC RTT

- Proposals

  - Option 1: No measurement period requirements are specified for RTT-based PDC

  - Option 2: Nokia proposal 4

- The TRS measurement period requirement for PDC RTT is 
TUERxTx,TRS=Nsample *TTRS, period

- where

- Nsample is the number of TRS measurement samples for PDC RTT.

- TTRS, period is the periodicity of the TRS specific for PDC RTT UE Rx-Tx time difference measurement.

  - Option 3: Huawei proposal in CR

    - The UE shall be able to measure UE Rx-Tx time difference on PCell after receving [TBD, command from network that triggers the UE Rx-Tx measurement] within TUERx-Tx_PDC, where

    - TUERx-Tx_PDC = TBD

- Recommended WF

  - More discussion needed

Issue 1-8: Channel conditions for which RAN4 will develop requirements

- Proposals

  - Option 1: AWGN only

  - Option 2: AWGN and fading (TDL-A)

- Recommended WF

  - More discussion needed

Issue 1-9: Define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed

Issue 1-10: Define UE Rx-Tx time difference measurement requirement for TRS requirements for 60KHz

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed

Issue 1-11: TRS resource number used for developing UE requirements

- Proposals

  - Option 1: 4 (2-slot TRS configuration)

  - Option 2: 2 (2-slot TRS configuration)

- Recommended WF

  - More discussion needed

Issue 1-12: Number of samples assumed for deriving the accuracy requirements

(open but will be decided based on simulation results)

- Proposals

  - Option 1: 1 sample

  - Option 2: 4 samples

  - Option 3: other

- Recommended WF

  - More discussion needed

### Sub-topic 1-3

Sub-topic description: Inform RAN2 about the RAN4 agreements on report mapping for UE and gNB Rx-Tx

Open issues and candidate options before e-meeting:

Issue 1-13: LS to RAN2 group for Rel-17 enhanced IIOT/URLLC

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed

### Sub-topic 1-4

Sub-topic description: RAN4 to start test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS

Open issues and candidate options before e-meeting:

Issue 1-14: Test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - RAN4 should discuss an initial list of test cases.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 1-1: Sub-topic description: UE Rx-Tx time difference measurement requirement for PRS

Issue 1-1: PRS measurement period requirement for PDC RTT

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Fine to use option 1 as the starting point. - Some scaling factor is for UE processing capability, and RAN4 may need to revisit if there is conflict with RAN1 capability discussion. - CSSF should be FFS, as we agreed last meeting that PDC measurement does not require MG. |
| Ericsson | Option 1. |
| Nokia | Support option 1. |
| Qualcomm | The proposal in option 1 aims to leverage the measurement period requirement for NR positioning measurements. While it may be reasonable to use the existing requirements as a starting point, our view is that the PDC use case is quite different from positioning use cases and in the end we may not be able to leverage much of the requirements. The main differences are: - Only PRS transmitted by the serving cell is measured/processed - Small PRS timing uncertainty (since DL timing is known) - No MG is needed for PRS contained within active BWP and with the same SCS Given these differences, we understand that RAN1 may specify a different processing capability for PRS in the context of PDC. In addition, TRS can be used as the DL reference signal for the UE Rx-Tx measurement, instead of PRS. Ideally, there would be some commonality between the requirements for RTT-based PDC when TRS and PRS are used as the DL reference signal. |
| vivo | It depends on outcome of number of samples to be used. If measurements are based on 1 shot, then no measurement period requirements are needed. |

Issue 1-2: Define UE Rx-Tx time difference measurement requirement for PRS requirements for FR2

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Either option is fine for us. |
| Ericsson | Option 1. Define UE Rx-Tx time difference measurement requirement for PRS requirements for FR2. Industrial IIoT WID does not exclude any SCS. Existing positioning requirements exist for all SCS in FR1 and FR2. |
| Nokia | We can support option 1. |
| Qualcomm | We can support option 1. |
| vivo | Same comments as for Issue 1-1 |

Issue 1-3: Define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Either option is fine for us. |
| Ericsson | Option 1. Define UE Rx-Tx time difference measurement requirement for PRS requirements for SCS = 60 kHz. Industrial IIoT WID makes no exclusion. Existing positioning requirements exist for all SCS in FR1 and FR2. |
| Nokia | We can support option 1. |
| Qualcomm | OK with option 1. |
| vivo | Is the 60kHz SCS for FR1 and FR2? If for FR2, would 120kHz SCS be also considered? |

Issue 1-4: Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | The issue is not very clear to us. Does Rel-17 PRS refer to the PRS measurement requirements for reduced sample number that are defined in Rel-17? If so we support to consider Rel-17 PRS. We are open to consider REl-16 PRS if there is clear justification, but we see no point to consider both. |
| Ericsson | Our initial position into meeting is that rel-16 and rel-16 number of samples are fine as a baseline. Rel-17 will work as well. |
| Nokia | Option 1. In R4-2202776 WF on NR Positioning Enhancements (Part 1), it shows the agreed conditions for Rel-17 ePos 1 sample PRS measurement Parameters; Value. No of samples w/o AGC (M1); 1. PRS Ês/Iot (dB); ≥ -6. Propagation conditions; LOS. PRS BW (RBs); ≥ [48]. Accuracy; R16. Repetition; R16. We understand that if Rel-17 ePos PRS with 1 sample measurement is adopted, then the accuracy for BW 24 PRB will be missing for PDC RTT. If this is a correct understanding, we then think the simulation for 24 PRB PRS with 1 sample measurement will be needed. But at the end, the simulation accuracy result for 24 PRB 1 sample PRS may or may not satisfy the PDC error budget. For the purpose of reducing workload, and also because 4 sample PRS (Rel-16 PRS) has already been verified by RAN1, we still prefer to reuse the measurement accuracy with 4 sample PRS (Rel-16 PRS), and we don’t see any strong reason against it. Also, we would like to check whether it is a common understanding that reusing Rel-16 requirements means reusing PRS measurement accuracy with both AWGN and fading channels for PDC? |
| Qualcomm | Same question as Huawei. We assume Rel-16/17 refers to different number of PRS samples and side conditions. Note that for reduced number of samples, the performance requirements have not been finalized. PDC applications may not need low-latency measurements so Rel-16 assumptions would be sufficient. |
| vivo | The issue is not clear. The PRS would be the same for both Rel-16 and Rel-17. The difference is mainly on number of samples and side conditions. We think the requirements can be based on 1 shot measurement with Rel-17 side conditions (some of which can be revisited for PDC). |

Issue 1-5: Are simulations needed for defining accuracy requirements

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. |
| Ericsson | Option 1. |
| Nokia | Option 1. We support reusing the Rel-16 PRS requirements so no more stimulation is needed for PRS. |
| Qualcomm | We understand this question is for PRS. Option 1 is fine. However, let’s clarify that this means we also reuse the corresponding side conditions and assumptions (e.g. number of samples, etc.). |
| vivo | It depends on outcome of number fo samples, side conditions. We prefer to reuse Rel-17 accuracy requirements for positioning enhancement. |

Issue 1-6: Number of samples assumed for deriving the accuracy requirements

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. The benefits in assuming 1-sample is that gNB Rx-Tx requirements are defined based on 1-sample. It is well known that for RTT based PDC the proximity between UE Rx-Tx and gNB Rx-Tx measurements are important, and using 1-sample also for UE measurements can help to ensure UE and gNB measurements are taken close in time. |
| Ericsson | We propose to reuse PDC RTT accuracy from R16 defined in TR 38.133-10.1.25.2 for PRS (Issue 1-5), then we accept to inherit the number of samples form TR 38.133-10.1.25.2. For TRS we are open to discuss the number of samples needed 1 or 4. |
| Nokia | We prefer option 2. See our comments in 1-4. We understand that for 1 sample PRS measurement accuracy, the extra simulation will be needed since Rel-17 ePos did not define the measurement accuracy for 1 sample PRS with small BW (>=24PRB). |
| Qualcomm | See answer to issue 1-5. |
| vivo | Support option 1. We agree with Huawei’s comments. In addition, we would like to point out the accuracy would be almost the same for 1 sample and 4 samples in AWGN channel. |

Sub-topic 1-2: Sub-topic description: UE Rx-Tx time difference measurement requirement for TRS

Issue 1-7: TRS measurement period requirement for PDC RTT

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Fine to use option 2 as the starting point. |
| Ericsson | Option 2. |
| Nokia | Option 2 |
| Qualcomm | We understand measurement period requirements are within scope. FFS the detailed requirement. See answer to issue 1-1. |
| vivo | Option 1. |

Issue 1-8: Channel conditions for which RAN4 will develop requirements

| Company | Commens |
| --- | --- |
| XXX |  |
| Huawei | Option 1. We understand AWGN is more relevant for URLLC/IIOT scenarios. Accuracy is clearly better with AWGN than with TDL-A. It is noted that in RAN1 evaluation the group delay calibration margin was not considered, so we believe RAN4 should target better accuracy for PDC. gNB Rx-Tx measurement requirements are defined based on 1-sample and AWGN, and we prefer to align the assumption for UE and gNB as much as possible |
| Ericsson | Option 2, both. Existing SRS/PRS requirements exist for both AWGN and TDL-A. Even in IIoT there is a sufficiently rich set of scenarios to merit both channels. My reference: https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_112-e/Inbox/Chairmans_Notes/RAN2-112-e_NR-U_PowSav_2sRA%20-Rel-17%20Sdata_IioT%20Notes%20(Diana)_Nov9_17-00.docx • Scenario 1: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the CN. The 5GS introduced error is caused by the relative time-stamping inaccuracy at the NW-TT and the DS-TTs. • Scenario 2: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the UE. The 5GS introduced error is caused by the relative time-stamping inaccuracies at the involved DS-TTs. • Scenario 3: In the smart grid use case, where the TSC devices behind a target UE are synchronized to the 5G GM TD. The 5GS introduced error is caused by the synchronization of the 5G clock to the DS-TT. |
| Nokia | Option 2. We agree with Ericsson that both AWGN and TDL-A should be considered for IIOT, and we think the condition for TRS should be consistent with the conditions for PRS. |
| Qualcomm | Option 1. It makes sense to prioritize AWGN. |
| vivo | Option 1. Typical use cases would be AWGN and LoS channel. |

Issue 1-9: Define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Either option is fine for us. |
| Ericsson | Option 1. Define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2. Industrial IIoT WID makes no exclusion. Existing positioning requirements (PRS, SRS) exist for all SCS in FR1 and FR2. |
| Nokia | We can support option 1. |
| Qualcomm | Option 1 is fine. |
| vivo | Fine with option 1. |

Issue 1-10: Define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Either option is fine for us. |
| Ericsson | Option 1. Define UE Rx-Tx time difference measurement requirement for TRS requirements for SCS = 60 kHz. Industrial IIoT WID makes no exclusion. Existing positioning requirements (PRS,SRS) exist for all SCS in FR1 and FR2. |
| Nokia | We can support option 1. |
| Qualcomm | Option 1 is fine. |

Issue 1-11: TRS resource number used for developing UE requirements

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1 for FR1, and option 2 for FR2. |
| Ericsson | We agree with Nokia’s conclusion from R4-R4-2203655 that Observation he TRS resource number has minor effect on the TUE-RX error. |
| Nokia | In our simulation results, we observe that there is not too much variance between using 2 resources of TRS and 4 resources of TRS. Considering 4 TRS resources with 2 consecutive slots will be configured in FR1 and it is available in FR2 (38.214), we support option 1. |
| Qualcomm | Our preference is option 1. |

Issue 1-12: Number of samples

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1, and same comment as Issue 1-6 for PRS. |
| Ericsson | 1 or 4 samples impact depend on channel. One (1) sample works fine if we select AWGN as channel. Our results show no significant improvement for AWGN with 4 samples. For TDL-A we see improvement with 4 samples. |
| Qualcomm | For UE Rx-Tx accuracy requirements it would be good to be consistent in the assumptions for PRS and TRS. We prefer option 2. |
| vivo | Option 1. |

Sub-topic 1-3: Sub-topic description: Inform RAN2 about the RAN4 agreements on report mapping for UE and gNB Rx-Tx

Issue 1-13: LS to RAN2 group for Rel-17 enhanced IIOT/URLLC

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. The following is included in the incoming LS from RAN1 R1-2112834, so RAN4 should inform the related agreement to other WGs. Conclusion The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4. |
| Ericsson | We agree with Huawei. |
| Nokia | Same view with Huawei |
| Qualcomm | Option 1 |
| vivo | Agree to sending LS based on agreements on reporting. |

Sub-topic 1-4: Sub-topic description: RAN4 should define test cases for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS

Issue 1-14: Test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | This should be the work for the perf part? |
| Ericsson | We do this in performance part of WI. |
| Nokia | Option 1. |
| Qualcomm | Yes, during the performance part. |
| vivo | The work should be done in performance part. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2205390, Huawei, HiSilicon | Title: CR on requirements for UE Rx-Tx measurement for PDC |
|  | Qualcomm: Fix typo in section numbering: 9.12.2 instead of 9.11.2. Should there be separate sections for the measurement period using PRS and TRS? |
| R4-2205815, Nokia, Nokia Shanghai Bell | Title: draftCR on requirements for UE Rx-Tx measurement for propagation delay compensation |
|  | Qualcomm: 9.12.3 Measurement Capability We’re not sure the multi-RTT capability will be reused. Delete the text or put it in []. Numbering typo: 9.12.3 repeated Our understanding is that RAN2 is still discussing the procedures for RTT-based PDC. For PDC at the UE side, the UE will not report the UE Rx-Tx measurements. For now, it may be better to remove section 9.12.3 Measurement Reporting Requirements. Also in 9.12.3.1 Measurement Reporting Requirements for PRS: PDC procedure should not rely on LPP. |
|  | Ericsson: “If UE Rx-Tx time difference measurement is based on PRS, the capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].” RAN2 will follow the RAN1 capabilities below: RAN1 has agreed the following propagation delay compensation related capabilities in R1-2112902: - FG 25-19: Propagation delay compensation based on CSI-RS for tracking and SRS, per FS - FG 25-19a: Propagation delay compensation based on DL PRS and SRS, per FS - FG 25-20: Propagation delay compensation based on legacy TA procedure, per UE NR-Multi-RTT-ProvideCapabilities is provided to the LMF (i assume). But RAN2 agreed that LMF is not part of the PDC procedure. IIoT is not multi RTT. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: PRS measurement period requirement for PDC RTT | Tentative agreements:Yes There is no direct objection against using option 1 as starting point while all details are still to be discussed further. It is clear that the outcome of the further discussions will impact (e.g. number of samples, CSSF etc.) Candidate options: Moderator propose to use following as baseline with added [] around parameters: - TUERxTx, PRS=CSSF*NPRSslotN'LavailablePRSN*[Nsample-1]*Teffect+Tlast - where - Teffect= TTPRS*TPRS Recommendations for 2nd round: Companies to raise concern if moderator proposal is not agreeable. |

|  | Status summary |
| --- | --- |
| Issue 1-2: Define UE Rx-Tx time difference measurement requirement for PRS requirements for FR2 | Tentative agreements: Yes Most companies support deifning UE Rx-Tx time difference measurement requirement for PRS requirements for FR2. It is not clear what vivo’s comment is addressing as the question is about whether or not to define requirements for FR2 in addition to FR1. RAN4 to define UE Rx-Tx time difference measurement requirement for PRS requirements for FR2 Candidate options: Recommendations for 2nd round: Companies to raise concern if moderator proposal is not agreeable. |

|  | Status summary |
| --- | --- |
| Issue 1-3: Define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz | Tentative agreements: Yes No company objected defining UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz RAN4 will define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz Candidate options: Moderator also propose to confirm that RAN4 will define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz for both FR1 or FR2. Recommendations for 2nd round: No 2nd round |

|  | Status summary |
| --- | --- |
| Issue 1-4: Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS | Tentative agreements: No Question was: - shall RAN4 use Rel-16 PRS and Rel-16 assumed number of samples for defining the accuracy requirements - shall RAN4 use Rel-17 PRS and Rel-17 reduced assumed number of samples for defining the accuracy requirements 3 options listed: - Option 1: Rel-16 only: (QC, Ericsson, Nokia) - Option 2: Rel-17 only: (HW, [Ericsson], [vivo]) - Option 3: Rel-16 and Rel-17: none Moderator suggest to drop option 3 and continue discussion of options 1 and 2. Agreement: RAN4 will not define PRS-based PDC requirements based on both Rel-16 and Rel-17 PRS Candidate options: - shall RAN4 use Rel-16 PRS and Rel-16 assumed number of samples for defining the accuracy requirements - shall RAN4 use Rel-17 PRS and Rel-17 reduced assumed number of samples for defining the accuracy requirements Recommendations for 2nd round: Continue to discuss following two options: - shall RAN4 define accuracy requirements using Rel-16 PRS assumed number of samples - shall RAN4 define accuracy requirements using Rel-17 PRS reduced assumed number of samples Additionally, one company proposed to clarify: - Does reusing Rel-16 requirements means reusing PRS measurement accuracy with both AWGN and fading channels for PDC? |

|  | Status summary |
| --- | --- |
| Issue 1-5: Are simulations needed for defining accuracy requirements | Tentative agreements: Yes RAN4 see no need for simulations for defining accuracy requirements. RAN4 will reuse PDC RTT accuracy from R16 defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method Candidate options: Recommendations for 2nd round: No 2nd round |

|  | Status summary |
| --- | --- |
| Issue 1-6: Number of samples assumed for deriving the accuracy requirements | Tentative agreements: No Candidate options: Number of samples assumed for deriving the accuracy requirements - Option 1: 1 sample: HW (FR1), vivo - Option 2: 4 samples: Nokia, [Ericsson], [QC (re-use)] - Option 3: other Moderator suggest to not consider option 3. Recommendations for 2nd round: Continue the discussion and comment also whether Rel-16 or Rel-17 is assumed when replying: - Option 1: 1 sample (assuming Rel-16 or Rel-17) - Option 2: 4 samples (assuming Rel-16 or Rel-17) |

|  | Status summary |
| --- | --- |
| Issue 1-7: TRS measurement period requirement for PDC RTT | Tentative agreements: Yes Moderator propose to define measurement period requirements. It is moderator understanding that independently from the selected number of samples as assumption most companies support defining a measurement period. Tentative agreement: - The TRS measurement period requirement for PDC RTT is TUERxTx,TRS=Nsample *TTRS, period - where - Nsample is the number of TRS measurement samples for PDC RTT. - TTRS, period is the periodicity of the TRS specific for PDC RTT UE Rx-Tx time difference measurement. Candidate options: Recommendations for 2nd round: Companies are to raise concern if the tentative agreement is not agreeable. |

|  | Status summary |
| --- | --- |
| Issue 1-8: Channel conditions for which RAN4 will develop requirements | Tentative agreements: No Candidate options: Channel conditions for which RAN4 will develop requirements: - Option 1: AWGN only (HW, QC, vivo) - Option 2: AWGN and fading (TDL-A) (Ericsson, Nokia) Recommendations for 2nd round: More discussion needed related to channel conditions for which RAN4 will develop requirements: - Option 1: AWGN only - Option 2: AWGN and fading (TDL-A) |

|  | Status summary |
| --- | --- |
| Issue 1-9: Define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2 | Tentative agreements: Yes RAN4 will define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2 Candidate options: Recommendations for 2nd round: No 2nd round |

|  | Status summary |
| --- | --- |
| Issue 1-10: Define UE Rx-Tx time difference measurement requirement for TRS requirements for 60KHz | Tentative agreements: Yes RAN4 agree to define UE Rx-Tx time difference measurement requirement for TRS requirements for 60KHz Candidate options: Recommendations for 2nd round: No 2nd round |

|  | Status summary |
| --- | --- |
| Issue 1-11: TRS resource number used for developing UE requirements | Tentative agreements: No Candidate options: TRS resource number used for developing UE requirements - Proposals - Option 1: 4 (2-slot TRS configuration) (HW (FR1), Nokia, QC, [Ericsson]) - Option 2: 2 (2-slot TRS configuration) (HW (FR2)) Recommendations for 2nd round: Moderator would like to hear if companies can agre to use option 1 for both FR1 and FR2? |

|  | Status summary |
| --- | --- |
| Issue 1-12: Number of samples assumed for deriving the accuracy requirements | Tentative agreements: No Candidate options: Number of samples assumed for deriving the accuracy requirements (open but will be decided based on simulation results) - Proposals - Option 1: 1 sample (HW, Ericsson (AWGN), vivo) - Option 2: 4 samples (Ericsson (TDL-A), QC - Option 3: other Moderator suggest not to consider option 3 any further. Recommendations for 2nd round: Number of samples assumed for deriving the accuracy requirements - Proposals - Option 1: 1 sample (HW, Ericsson (AWGN), vivo) - Option 2: 4 samples (Ericsson (TDL-A), QC |

|  | Status summary |
| --- | --- |
| Issue 1-13: LS to RAN2 group for Rel-17 enhanced IIOT/URLLC | Tentative agreements: Yes RAN4 agree to send LS to RAN2 group for Rel-17 enhanced IIOT/URLLC on report mapping for UE and gNB Rx-Tx Candidate options: Recommendations for 2nd round: |

|  | Status summary |
| --- | --- |
| Issue 1-14: Test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS | Tentative agreements: No Candidate options: Recommendations for 2nd round: Postpone |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2205390, Huawei, HiSilicon | To be revised |
| R4-2205815, Nokia, Nokia Shanghai Bell | To be revised |

## Discussion on 2nd round (if applicable)

Based on the agreements from 1st round:

PRS based PDC:

- RAN4 will define UE Rx-Tx time difference measurement requirement for PRS requirements for 60KHz

- Agreement: RAN4 will not define PRS-based PDC requirements based on both Rel-16 and Rel-17 PRS

- RAN4 see no need for simulations for defining accuracy requirements. RAN4 will reuse PDC RTT accuracy from R16 defined in TR 38.133-10.1.25.2 for Rel-17 PDC RTT-based method

TRS based PDC:

- RAN4 will define UE Rx-Tx time difference measurement requirement for TRS requirements for FR2

- RAN4 agree to define UE Rx-Tx time difference measurement requirement for TRS requirements for 60KHz

There are still a number of open items for further discussion. Instead of discussing those one by one moderator has set up two sets of tables:

1) a table containing proposed simulation assumptions based on Rel-16 PRS simulations:

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 1 severing cell |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 10 MHz, 20 MHz, 50 MHz - 15 kHz - FDD, TDD - 4 GHz - 20 MHz, 50 MHz, 100 MHz - 30 kHz - FDD, TDD | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-A (30 ns delay spread, 5Hz), | AWGN, TDL-C (60 ns delay spread, 300 Hz) |
| Es/Iot for tw cells (cell 1, cell 2, cell 3), [dB] | -3 | -3 |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| NOTE 1: nr-DL-PRS-ReferenceInfo is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: - 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: - 48 (20MHz),132 (50MHz), 272 (100MHz) | - 120 kHz: - 32(50MHz),64(100MHz), 128 (200MHz) |
| Comb | Comb-4, comb-6, comb-2 (with muting), comb-12 |  |
| PRS periodicity | 40 ms |  |
| DL-PRS-ResourceRepetitionFactor | 1, 2, 4, 16 |  |
| PRS-ResourceTimeGap | 1, 2, 4 | 1, 8, 16 |
| PRS muting | No muting, muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 1 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| Evaluation period | 40 ms, 200 ms |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

2) a table containing proposed simulation assumptions based on agreed TRS simulations::

Table 1 General parameters (TRS)

| Parameter | Value |
| --- | --- |
| Cell layout | 1 serving cell |
| Duplex modes | FDD and TDD |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |
| Cyclic prefix | Normal |
| DRX | OFF |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD - [4] GHz - 20 MHz, 40 MHz, 100 MHz - 60 kHz - FDD, TDD - [30] GHz - 40 MHz, 80 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] |
| Es/Iot [dB] | -3 |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |
| UE measurement bandwidth | Full carrier bandwidth |

Table 2 CSI-RS for tracking transmission configuration parameters (TRS)

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz) - 60 kHz: 24(20MHz), 64 (50MHz),132 (100MHz) - 120 kHz: 32(50MHz), 64 (100MHz), 128 (200MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz) - 60 kHz: 128(20MHz), 64 (40MHz), 32 (100MHz) - 60 kHz: 128(40MHz), 64 (80MHz), 32 (200MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 2, 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

A number of aspects for each set of simulation assumptions are open and those have been listed in two separate tables:

1) open aspects regarding simulation assumption for PRS:

For PRS based PDC following three questions needs to be discussed and decided:

|  | Rel-16 PRS | Rel-17 PRS |
| --- | --- | --- |
| Ês/Iot | >=-3dB | >-=6dB |
| PRS sample number | 4 | 1 |
| Channel condition | AWGN, TDL-A | LOS |
| Carrier frequency range / SCS / BW | - FR1: - 15kHz: 5 MHz, 10 MHz, 20 MHz - 30kHz: 10 MHz, 20 MHz, 50 MHz - 60kHz: 20MHz, 50MHz, 100MHz - FR2: - 60kHz: 20MHz, 50MHz, 100MHz - 120kHz: 50MHz, 100MHz, 200MHz | - FR1: - 15kHz: 10 MHz, 20 MHz - 30kHz:, 20 MHz, 50 MHz - 60kHz: 50MHz, 100MHz - FR2: - 60kHz: 50MHz, 100MHz - 120kHz: 100MHz, 200MHz |
| Accuracy | 38.133 – 10.1.25.2 | 38.133 – 10.1.25.2 |

1) Shall RAN4 use Rel-16 or Rel-17 PRS assumptions as baseline (and hence the assumptions in the table above) (Issue 1-15)?

2) Once decided which Release to use as baseline: Shall RAN4 change any of the parameters (Issue 1-16)?

2) open aspects regarding simulation assumption for TRS (in []):

| Ês/Iot | >=-3dB |
| --- | --- |
| TRS sample number | [1, 4] |
| Channel condition | AWGN, [TDL-A] |
| TRS resources number | 4 TRS resources over 2 slots, [2 TRS resources over 2 slots] |
| Carrier frequency range / SCS / BW | - FR1: - 15kHz: 5 MHz, 10 MHz, 20 MHz - 30kHz: 10 MHz, 20 MHz, 50 MHz - 60kHz: 20MHz, 50MHz, 100MHz - FR2: - 60kHz: 20MHz, 50MHz, 100MHz - 120kHz: 50MHz, 100MHz, 200MHz |

For TRS based PDC following three questions needs to be discussed and decided:

1) TRS sample number (Issue 1-17)?

2) Channel conditions (Issue 1-18)?

3) TRS Resource number (Issue 1-19)?

And then RAN4 need to agree on measurement period for PRS based and TRS based PDC (Issue).

Following agreements have been agreed during GTW February 25th:

PRS based PDC:

Issue 1-6

- Agreements

  - Rel-17 PDC RTT-based method with PRS-based measurements

    - Reuse Rel-16 PRS accuracy requirements and number of samples (i.e., 4)

    - Define requirements for AWGN conditions. FFS for requirements for fading conditions.

    - Do not consider Rel-17 PRS accuracy requirements for Rel-17 PDC RTT-based method

Issue 1-18

- Agreement

  - Channel model simulation assumptions

    - AWGN

    - TDL-A

  - FFS for channel model for requirements definition and whether to include both models

Issue 1-19

- Agreement

  - 4 TRS resources over 2 slots

Based on these agreements a number of Issues below do not need further discussion. This is marked under the recommended WF.

### Sub-topic 1-5

Sub-topic description: simulation assumptions for Rel-16 PRS simulations

Open issues and candidate options before e-meeting:

Issue 1-15: Shall RAN4 use Rel-16 or Rel-17 PRS assumptions as baseline (and the assumptions in the table for PRS simulation assumptions)

- Proposals

  - Option 1: Use Rel-16 assumptions as baseline

  - Option 2: Use Rel-17 assumptions as baseline

- Recommended WF

  - Agreed during GTW. No further discussion needed

Issue 1-16: Shall RAN4 change any of the simulation assumption parameters in the table for PRS simulation assumptions (for the selection supported in Issue 1-15)

- Proposals

  - Option 1: No, there is no need to change any parameters

  - Option 2: Yes (please list exact parameter(s))

    - Channel condition shall be:

      - AWGN only

      - AWGN and fading channel

    - PRS sample number shall be: Not open for discussion. Closed in GTW.

- Recommended WF

  - More discussion needed

### Sub-topic 1-6

Sub-topic description: simulation assumptions for Rel-17 TRS simulations

Open issues and candidate options before e-meeting:

Issue 1-17: TRS sample number

- Proposals

  - Option 1: Use 1 sample

  - Option 2: Use 4 samples

- Recommended WF

  - More discussion needed

Issue 1-18: Channel conditions

- Proposals

  - Option 1: AWGN only

  - Option 2: AWGN and TDL-A

- Recommended WF

  - Agreed in GTW. No more discussion needed

- Agreement

  - Channel model simulation assumptions

    - AWGN

    - TDL-A

  - FFS for channel model for requirements definition and whether to include both models

Issue 1-19: TRS Resource number

- Proposals

  - Option 1: Only 4 TRS resources over 2 slots

  - Option 2: 4 TRS resources over 2 slots and 2 TRS resources over 2 slots

- Recommended WF

  - Agreed in GTW. No more discussion needed

- Agreement

  - 4 TRS resources over 2 slots

### Sub-topic 1-7

Sub-topic description: Measurement period for PRS and TRS based PDC

Open issues and candidate options before e-meeting:

Issue 1-20: PRS measurement period

use following as baseline with added [] around parameters CSSF and samples:

- TUERxTx, PRS=CSSF*NPRSslotN'LavailablePRSN*[Nsample-1]*Teffect+Tlast

- where

- Teffect= TTPRS*TPRS

- Proposals

  - Option 1: Yes, Proposed measurement period can be used as starting point

  - Option 2: No, Proposed measurement period cannot be used as starting point

- Recommended WF

  - More discussion needed

Issue 1-21: TRS measurement period

use following as baseline:

- The TRS measurement period requirement for PDC RTT is defined as 
TUERxTx,TRS=Nsample *TTRS, period

- where

  - Nsample is the number of TRS measurement samples for PDC RTT.

  - TTRS, period is the periodicity of the TRS specific for PDC RTT UE Rx-Tx time difference measurement.

- Proposals

  - Option 1: Yes, Proposed measurement period can be used as starting point

  - Option 2: No, Proposed measurement period cannot be used as starting point

- Recommended WF

  - More discussion needed

### Sub-topic 1-8

Sub-topic description: Define UE Rx-Tx time difference measurement requirement for PRS based PDC for FR2

Open issues and candidate options before e-meeting:

Issue 1-22: Confirm to define UE Rx-Tx time difference measurement requirement for PRS based PDC for FR2

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Confirm to define UE Rx-Tx time difference measurement requirement for PRS based PDC for FR2

## Companies views’ collection for 2nd round

### Open issues

Issue 1-15: Shall RAN4 use Rel-16 or Rel-17 PRS assumptions as baseline (and the assumptions in the table for PRS simulation assumptions)

| Company | Comments |
| --- | --- |
| XXX | Shall RAN4 use Rel-16 or Rel-17 PRS assumptions as baseline (and the assumptions in the table for PRS simulation assumptions) |
|  | Agreed during GTW. No further discussion needed |

Issue 1-16: Shall RAN4 change any of the simulation assumption parameters in the table for PRS simulation assumptions (for the selection supported in Issue 1-15)

| Company | Comments |
| --- | --- |
| XXX | Shall RAN4 change any of the simulation assumption parameters in the table for PRS simulation assumptions (for the selection supported in Issue 1-15) |
| Ericsson | Since it was agreed during GTW to: - Reuse Rel-16 PRS accuracy requirements and number of samples (i.e., 4) - Define requirements for AWGN conditions. FFS for requirements for fading conditions. - Do not consider Rel-17 PRS accuracy requirements for Rel-17 PDC RTT-based method We do not think we need to further simulate PRS. Maybe moderator can clarify? |
| Nokia | As it is agreed to reuse Rel-16 PRS accuracy requirements, no simulation will be needed for Rel-17 PDC RTT with PRS. |
| Qualcomm | RAN4 should leverage the Rel-16 simulation assumptions according to the GTW agreement. |
| Huawei | Agree that no simulation is needed for PRS. |
| vivo | There are many unclear issues regarding GTW agreements for Issue 1-15 as copied by Ericsson. - Does reusing R16 accuracy requirements mean everything is copied for PDC? At least, we don’t see there is any reason to specify accuracy requirements for Es/Iot =-13dB. PDC is only for serving cell measurement. - If requirements are specified for AWGN only, should we still use 4 samples? It would be common understanding that accraucy under AWGN is similar for 1 sample and 4 samples measurements. We should use 1 sample instead to avoid long measurement delay. However, we don’t think additional simulation would be needed. 4 samples accuracy requirements in Rel-16 would also be applicable to 1 sample measurement under AWGN. |
| Nokia | Response to vivo: For the first question, it has been agreed in RN4#101bis-e that Es/Iot = -3 dB is assumed for Rel-17 PDC RTT-based method using PRS as the DL reference signal. For the second question, it has been agreed in the GTW to use 4 samples of PRS measurement independent to channel condition. |

Issue 1-17: TRS sample number (TRS)

| Company | Comments |
| --- | --- |
| XXX | TRS sample number (TRS) |
| Erisson | For TRS with AWGN: 1 sample For TRS with TDL-A fading: 4 samples, if we agreed to develop fading requirements for TRS. |
| Nokia | We understand that from the GTW that we can wait for more simulation results then decide the TRS sample number. |
| Qualcomm | Prefer to align the number of samples with the value agreed for PRS. i.e. option 2. |
| Huawei | We support option 1, but can compromise to option 2 considering alignment with PRS based PDC. |
| vivo | Support option 1. We don’t think there is agreement to specify requirements for fading channel, even for PRS based measurement. It is noted that both gNB based PDC and UE based PDC are supported. Therefore, similar requriements should be specified for both gNB Rx-Tx measurement and UE Rx-Tx measurement. We do not see there is any proposal to specify gNB measurements under fading channel. Thus, no such requirements for UE measurements would be specified. |

Issue 1-18: Channel conditions (TRS)

| Company | Comments |
| --- | --- |
| XXX | Channel conditions (TRS) |
|  | Agreed in GTW. No more discussion needed |

Issue 1-19: TRS Resource number (TRS)

| Company | Comments |
| --- | --- |
| XXX | TRS Resource number (TRS) |
|  | Agreed in GTW. No more discussion needed |

Issue 1-20: PRS measurement period

| Company | Comments |
| --- | --- |
| XXX | PRS measurement period |
| Ericsson | Option 1. |
| Nokia | Option 1. |
| Qualcomm | We think this should be FFS. It’s not clear to us that the UE capabilities for NR positioning apply here. PDC will not rely on LPP and all those capabilities are exchanged via LPP. Also, the use case for PDC is quite different from the positioning use case. Maybe we need input from RAN1 and RAN2 here. |
| Huawei | Option 1. We agree with what QC mentioned and the applicable capabilities are defined in RAN1, so to address QC’s comments we suggest to add a bullet in WF that measurement period can be revisited based on further agreement from RAN1/2. |
| vivo | FFS. The measurements should be done outside of MG. most of the parameters needs further discussion. Tlast would not be needed etc,. Maybe simple requirements as for Issue 1-21 would be enough. |

Issue 1-21: TRS measurement period

| Company | Comments |
| --- | --- |
| XXX | TRS measurement period |
| Ericsson | Option 1. |
| Nokia | Option 1. |
| Qualcomm | We’re OK to use the proposal as a starting point. Realistically, this will have to be finalized in maintenance. |
| Huawei | Option 1. |
| vivo | Would measurement period requirements be needed if measurement is based on 1 sample? It is also noted here that both gNB based PDC and UE based PDC are supported. So it is preferable similar requirements are specified. |

Issue 1-22: Confirm to define UE Rx-Tx time difference measurement requirement for PRS based PDC for FR2

| Company | Comments |
| --- | --- |
| XXX | Confirm to define UE Rx-Tx time difference measurement requirement for PRS based PDC for FR2 |
| Ericsson | Option 1. |
| Nokia | Option 1. |
| Qualcomm | Option 1. |
| Huawei | Option 1. |
| vivo | Option 1. |

# Topic #2: Timing reference point for UE UL timing

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203656 | Nokia, Nokia Shanghai Bell | Proposal 1: TP for Rel-15/16/17 38.133 7.1.2 “The downlink timing is defined as the time when the first detected path in time of the corresponding downlink frame from the reference cell is received at the UE antenna” Proposal 2: If Option #1 is not agreeable, then Option # 3 can be compromised. |
| R4-2204423 | Intel | draftCR to clarify timing reference point for UE UL timing test cases |
| R4-2204649 | vivo | Proposal 1: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna. |
| R4-2205391 | Huawei, HiSilicon | Proposal 1a: Do not mention ‘detected’ nor ‘detectable’ in the definition of the “reference point” for Te requirements in clause 7.1.2 of 38.133. Proposal 1b: Add a note in the requirements that the requirements may not apply in all conditions but shall apply under conditions used in the test cases. Proposal 2: Update the definition of the “reference point” in clause 7.1.2 of 38.133 from Rel-15: “The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell arrives at the UE antenna.” Proposal 3: Send LS to inform RAN1 about the updated definition of the “reference point”. |
| R4-2206021 | Ericsson | • Observation 1: The term first “detected path” (in time) in the definition of the reference point for timing error control requirement in section 7.1.2 in TS 38.133 would mean that the reference point is inside the UE i.e. at the UE baseband. • Observation 2: The purpose of the reference point is for interpretation, derivation, verification or testing of the core requirements. But the reference point inside the UE (i.e. at baseband) means that the reference point cannot be determined/estimated by the test system • Observation 3: It is agreed to include “UE antenna” will in the reference point definition. But UE antenna does not ‘detect’ rather receive signal, • Observation 4: Testing of Te is done under AWGN which has one path. Therefore, the path arriving at the UE antenna and detectected by the UE is the same. That’s why the problem has not been observed or will not be observed in the test. • Observation 5: In principle use of first “detected” path in the reference point definition creates ambiguity and in principle such definition (with detected path) also leaves core requirements “untestable”. • Proposal #1: The term “detected” is not included in the reference point definition. • Proposal #2: Clarify reference point definition according to Option # 2 [2]: o The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna. • Proposal #3: If Option #2 is not agreeable then clarify reference point definition according to Option # 3 [2]: |
| R4-2206022 | Ericsson, Intel, Huawei, HiSilicon, Qualcomm | Correction to reference point defintion for UE timing in TS 38.133 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

TP Option 1:

The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna

TP Option 2:

The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna

TP Option 3:

The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna

WF:

Next meeting is last meeting to reach agreement. All 3 options are open for discussion.

### Sub-topic 2-1

Sub-topic description: TP for downlink timing definition

Open issues and candidate options before e-meeting:

Issue 2-1: Preference for TP for downlink timing definition

- Proposals

  - Option 1: Prefer TP option 1

  - Option 2: Prefer TP option 2

  - Option 3: Prefer TP option 3

  - Option 4: can compromise to TP option 3.

- Recommended WF

  - More discussion needed

## Companies views’ collection for 1st round

### Open issues

Issue 2-1: Preference for TP for downlink timing definition

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1: Preference for TP for downlink timing definition |
| Intel | We support option 2. We are also willing to look at the possibility to go with option 3. One note is that no impact on RAN5 conformance test cases is expected. |
| Huawei | Support option 2 but we can also compromise to option 3. |
| Ericsson | We support Option 2. But we can also compromise to Option 3 for the sake of progress. |
| Nokia | We prefer Option 1, but we can compromise to Option 4 to move forward. |
| Qualcomm | We support option 2. The wording in Option 3 sounds a bit circular and in our view it does not address the concern about testability. At the very least RAN4 should add “at the UE antenna” to the text in the specification. |
| Apple | Support option 1. Can compromise to option ¾. |
| vivo | We support TP option 1. It is also fine not to make any changes to existing requirements if there is no consensus to add “ at the UE antenna” as in TP option 1. We could consider to compromise to option 3 if this is only way to conclude the issue. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204423, Intel | Title: draftCR to clarify timing reference point for UE UL timing test cases |
|  | Company A |
| R4-2206022, Ericsson, Intel, Huawei, HiSilicon, Qualcomm | Title: Correction to reference point defintion for UE timing in TS 38.133 |
|  | Company A |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: Preference for TP for downlink timing definition | Tentative agreements: No Almost all companies can compromise to TP option 3: - TP Option 1 (Nokia, Apple,, vivo) - TP Option 2 (Intel, HW, Ericsson, QC) TP Option 3 (Intel, HW, Ericsson, Nokia, Apple, vivo)Candidate options: TP Option 1: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna TP Option 2: The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna TP Option 3: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna Recommendations for 2nd round: Moderator would like to hear if companies can compromise on TP option 3? |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2204423, Intel | To be revised |
| R4-2206022, Ericsson, Intel, Huawei, HiSilicon, Qualcomm | To be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 2-2

Sub-topic description: TP for downlink timing definition

Open issues and candidate options before e-meeting:

Issue 2-2: Is TP option 3 agreeable compromise?

TP Option 3:

The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - No discussion needed due to agreement in GTW.

Update after GTW session February 25th:

- Agreements

  - Update the downlink timing definition as follows

    - The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna

Session chair: Further discuss applicable release for the change (e.g., starting from Rel-15 or Rel-17)

Issue 2-3: Is TP option 3 is applicable from which release?

- Proposals

  - Option 1: Rel-15

  - Option 2: Rel-17

- Recommended WF

## Companies views’ collection for 2nd round

### Open issues

Issue 2-3: Is TP option 3 is applicable from which release?

| Company | Comments |
| --- | --- |
| XXX | TP option 3 is applicable from which release? |
| Huawei | We suggest Rel-15. As commented during GTW, starting from Rel-17 could imply there is some technical change in Rel-17 compared to Rel-15, but this is not the case. |
| vivo | Though from our understanding, option 3 would be similar to option 1 at least from UE perspective. However, during GTW there was comment that option 3 is similar to option 2. Obviously, companies may have different interpretation of the agreed TP option 3. Such ambiguity by introducing new TP is really undesirable. To make sure there is no potential impact to Rel-15/16 UEs/gNBs on the market, we do believe the new TP option 3 should be changed from Rel-17 as a final package of compromise. If companies want to clarify Rel-15/16 specs, TP option 1 should be used as baseline. Or using following revised wording would also be another compromise. The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used detected by the UE to determine downlink timing is received from the reference cell at the UE antenna. |
| Ericsson | Option 1 (from Rel-15). Reference point definition has led to misinterpretation of requirements. Therefore it is important to have consistent definition of the reference point in all releases. We uphold GTW agreemets on TP Option 3. We already made compromise on the wording. We do not agree to modify the GTW agreement, which was reached after very long debate. |

# Topic #3: UE features for enhanced IIoT and URLLC

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2206015 | Nokia, Nokia Shanghai Bell | Proposal: RAN4 does not introduce additional UE feature group for Rel-17 enhanced IIOT/URLLC. |
| R4-2204472 | Qualcomm Incorporated | Proposal 1: The UE capability for RTT-based PDC should be defined per feature set. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1: Is there a need for RAN4 input to the feature group for Rel-17 enhanced IIOT/URLLC

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2. Technically, we support Proposal 1 from R4-2204472, but we assume it is more proper to be discussed in RAN1. |
| Ericsson | Option 2. There is no need for any RAN4 feature. |
| Nokia | Option 2. |
| Qualcomm | The UE capability should be defined per feature set. We need to check the status in RAN1. |
| vivo | Option 2. |

|  | Status summary |
| --- | --- |
| Issue 3-1: Is there a need for RAN4 input to the feature group for Rel-17 enhanced IIOT/URLLC | Tentative agreements: No Candidate options: Is there a need for RAN4 input to the feature group for Rel-17 enhanced IIOT/URLLC - Proposals - Option 1: Yes - Option 2: No (HW, Ericsson, Nokia, vivo) One company needed more time to check. Recommendations for 2nd round: Is there a need for RAN4 input to the feature group for Rel-17 enhanced IIOT/URLLC - Proposals - Option 1: Yes - Option 2: No |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 3-2

Sub-topic description: TP for downlink timing definition

Open issues and candidate options before e-meeting:

Issue 3-2: Is there a need for RAN4 input to the feature group for Rel-17 enhanced IIOT/URLLC

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - There is no need for RAN4 input to the feature group for Rel-17 enhanced IIOT/URLLC

| Company | Comments |
| --- | --- |
| Nokia | Support the WF. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR_IIOT_URLLC_enh_RRM | Nokia, Nokia Shanghai Bell |  |
| reply LS on propagation delay compensation | Huawei | To: RAN1, RAN2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-22xxxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2204423 | draftCR to clarify timing reference point for UE UL timing test cases R15 | Intel | Revised |  |
| R4-2206022 | Correction to reference point defintion for UE timing in TS 38.133 | Ericsson, Intel, Huawei, HiSilicon, Qualcomm | Revised |  |
| R4-2205390 | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, HiSilicon | Revised |  |
| R4-2205815 | draftCR on requirements for UE Rx-Tx measurement for propagation delay compensation | Nokia, Nokia Shanghai Bell | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-22xxxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2205390 | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, HiSilicon | Merged | merged to R4-2205815 |
| R4-2205815 | draftCR on requirements for UE Rx-Tx measurement for propagation delay compensation | Nokia, Nokia Shanghai Bell | Agreeable | Based on GTW agreement Thursday |
| R4-2204423 | draftCR to clarify timing reference point for UE UL timing test cases R15 | Intel | Agreeable | Based on GTW agreement |
| R4-2206022 | Correction to reference point defintion for UE timing in TS 38.133 | Ericsson, Intel, Huawei, HiSilicon, Qualcomm, Nokia, Nokia Shanghai Bell | Agreeable | Based on GTW agreement Thursday |
| WF on NR_IIOT_URLLC_enh_RRM | Nokia, Nokia Shanghai Bell | WF on NR_IIOT_URLLC_enh_RRM | Agreeable |  |
| reply LS on propagation delay compensation | Huawei | To: RAN1, RAN2 | Agreeable | Agreeable based on updated version after GTW |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Intel | Meng | Meng.zhang@intel.com |
| Huawei | Li Zhang | zhangli164@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)