3GPP TSG-RAN WG4 Meeting # 102-e 												R4-2207145

Electronic Meeting, Feb. 21- Mar. 03, 2022

Agenda item:			4.1.4

Source:	Moderator (Huawei)

Title:	Email discussion summary for [102-e][302] NR_Conformance_Maintenance

Document for:	Information

0 Introduction

The scope of this email discussion is to discuss the contributions submitted at agenda 4.1.4 on NR BS conformance maintenance.

- Topic #1: Sweep time setting for unwanted emission testing

- Topic #2: Test configuration for NC operation

- Topic #1: Sweep time setting for unwanted emission testing

  - Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2203562 | Anritsu Corporation | Sweep time setting of spectrum analyzer for BS unwanted emission TCs Observation 1: There is a concern that sweep time may need to be increased with an order of 10 times depending on the target measurement uncertainty to fulfil. Observation 2: A huge amount of discussion in the group is expected to derive appropriate sweep time / average setting per test case. Observation 3: It is not practical to apply the fixed sweep time setting to all DUTs based on the worst case assumption. Observation 4: Since the test tolerance is defined as zero with the unwanted emission tests, the DUT has already been designed to have some margin against the minimum requirements, taking into account of the measurement uncertainty. Observation 5: It is possible to carry out the unwanted emission tests with a shorter sweep time setting as far as the tightened test requirement is applied. Proposal 1: The fixed sweep time setting is not defined in the conformance specification for the sake of avoiding unnecessary longer test time, and the actual test procedures are left to the test case implementation. Proposal 2: Add a statement in the spec to judge each test result with a tightened test requirement which corresponds to the sweep time and its expected variance. The test shall be carried out again with the longer sweep time setting or longer average setting in case the obtained measurement results have exceeded the tightened test requirement. |
| R4-2203977 | CATT | Discussion on sweep time for unwanted emission testing Proposal 1: To adopt Option 3, the sweep time for a sample is a fix value of [40] us irrespective of the OFDM length. |
| R4-2203978 | CATT | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15) |
| R4-2203981 | CATT | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15) |
| R4-2204435 | NEC | Discussion on the sweep time for unwanted emission testing Proposal: Keep current text for unwanted emission testing in BS conformance test specification, unless otherwise reasonable justifications are provided by the test equipment vendors to modify it. |
| R4-2204711 | Keysight Technologies UK Ltd | Setting Sweep Time Requirement on Measuring BS Conformance Unwanted Emission testing. Observation-1, There seems some misunderstanding in proposal about measured result of faster sweep time. Observation-2, Total measurement time of unwanted emission measurement takes longer and always the concern of user of test equipment/system Observation-3, Most suitable measurement setting other than detection mode should be chosen based on device characteristic and test system characteristic. This is to maintain good balance between necessary SNR and measurement speed. Such adjustment should be done as preparation and calibration steps of test system. Observation-4, There are multiple method to mitigate larger variation in measured result which is from use of faster sweep on random signal power measurement. Observation-5, Defining minimum sweep time as proposal is too much of time penalty for everyone. Right adjustment could be done on case-by-case basis. Proposal, choose either one of following approach, - Leaving current text as is for allowing most appropriate setting on measurement parameters. - Add some calibration preparation text in procedure of TS38.141 for leading towards correctly adjust measurement parameters. (Detection mode should be true RMS as already defined) This is for appropriate testing and test time optimization, and which is depending on characteristic of test system and characteristic of device under test. Also, to add note for when averaging method is used (calculate average of multiple measured results). For this case, TP is proposed as below. |
| R4-2205149 | Huawei, HiSilicon | Clarification for unwanted emission testing Observation: Considering 1 dB deviation for the power detection, 100 us average duration is a good trade-off and at least a symbol length is required. Proposal 1: it is proposed to include the clarification on average time duration for RMS detection in the conformance test procedure, as below. ===for example for Transmitter spurious emissions in 38.141-1=== 2) Measurements shall use a measurement bandwidth in accordance to the conditions in clause 6.6.5.5. The measurement device characteristics shall be: - Detection mode: True RMS. The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1. The time duration includes time duration spent on each sweep point and the averaging from multiple measured results for the sweep point if average trace is used. NOTE: The setting of the average time also depends on the input signal characteristics, e.g. at least a symbol length for each sweep point should be adopted to measure the mean power. |
| R4-2205150 | Huawei, HiSilicon | Draft CR to 38.141-1: Clarification for unwanted emission testing |
| R4-2205153 | Huawei, HiSilicon | Draft CR to 38.141-2: Clarification for unwanted emission testing |

1.2 Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

1.2.1 Sub-topic 1-1 –Sweep time setting for unwanted emission testing

Issue 1-1-1: Sweep time setting for unwanted emission testing

In RAN4#101-e meeting, RAN4 agreed to further analyse the appropriate time to get a stable test results for unwanted emissions in WF R4-2120636. From the contributions submitted for this meeting, it is observed that longer sweep time or longer average setting can reduce the variation of measurement results. Some contributions propose to set a minimum value to get a stable results, while some contributions raise the concern on a fixed value which may need huge amount of work and will take longer test time, and hence propose the fixed sweep time setting is not defined and propose to add some clarification statements to address the issue.

- Proposals:

  - Proposal 1: The fixed sweep time setting is not defined in the conformance specification.

  - Proposal 2: Add a statement in the spec to judge each test result with a tightened test requirement which corresponds to the sweep time and its expected variance. The test shall be carried out again with the longer sweep time setting or longer average setting in case the obtained measurement results have exceeded the tightened test requirement.

  - Proposal 3: the sweep time for a sample is a fix value of [40] us irrespective of the OFDM length.

  - Proposal 4: Keep current text for unwanted emission testing in BS conformance test specification.

  - Proposal 5: Add some calibration preparation text in procedure of TS38.141 for leading towards correctly adjust measurement parameters and add note for when averaging method is used (calculate average of multiple measured results).

  - Proposal 6: To include clarification that an appropriate time duration should be set to ensure the measurement is within the MU.

- Recommended WF

  - Discuss on the proposals to address the issue.

Issue 1-1-2: Comments collection for the draft CRs

- Recommended WF

  - Comments collection on 1st round discussion

1.3 Companies views’ collection for 1st round

1.3.1 Open issues

Collection of comments:

Issue 1-1-1: Sweep time setting for unwanted emission testing

| Company | Comments |
| --- | --- |
| Anritsu | Issue 1-1-1: As already summarized by the moderator in Issue 1-1-1 and also explained in our discussion paper (R4-2203562), it is expected that activities to derive the fixed mean time per test case may require a huge amount of work since the variance of the obtained signal power may change depending on the setting of the test equipment, such as RBW whose selection may vary per test equipment. Hence, we support Proposal 1, 2, 5 and 6. |
| Keysight Keysight2 | 1-1-1; We are OK with Proposal 1,2,4,5,6, we are OK to add (even not to add) some clarification text, but not supporting Proposal 3 which is to set fixed time length value for sweep time per a sample (and even total sweep time to set), even with minimum value. (2nd time comment, There is typo in 1st time comment, and now it’s corrected. The one we don’t support is proposal 3 not 4) While measurement should be done with “mean power” to measure (this is already stated in existing spec text) and needs to be followed, at the same, as Anritsu commented, there are vary many parameters including total test system performance as well as device performance, setting appropriate value per individual case is possible but not possible to choose one fixed sweep time value to define in standard. Other aspect of this issue, this NR spec change proposal, change request is from R15 spec which has been used for actual device already and some kind of test setting already exist for each induvial case, which could be affected with this proposed change and not good to see if some test needed to be revised or forced to be re-done. I worry such could happen when fixed value is set. |
| Nokia | Support proposals 1 and 4, we do not see it necessary to mandate the sweep time in the standards. Do not support proposals 2 and 3, proposal 2 will increase test complexity and duration, proposal 3 will reduce flexibility of test setting to suit the signal under test. For proposals 5 and 6, we are open to discuss further if some clarification text is beneficial. |
| Huawei | Issue 1-1-1: we agree most of observations from the contributions submitted in this meeting. It is observed that longer sweep time or longer average setting will help to get a stable test results, on the other hand, the variation will be larger for a shorter sweep time or average setting. Hence we think at least some clarification on the appropriate time setting will be needed. We are ok with proposal 2, 5, 6. On proposal 1 and 3, we can further discuss whether an example value for a specific case would be helpful. |
| NEC | Issue 1-1-1: we support proposals 1 and 4. For proposal 2, we may be ok as an alternate test procedure and test requirements if we can agree the sweep time and the test requirements to be stated in the TS. However, we do not think RAN4 can agree them. For proposal 3, we need further study to agree the value. We can not accept it for the time being. For proposal 5 and 6, we are open to discuss. |
| Samsung | Issue 1-1-1: we tend to agree that the clarification to configuration of TE would be beneficial to facilitate the readability of spec especially for engineer outside 3GPP to avoid potential misunderstanding on test parameters. Hence we are open to discuss the detail specification impact based on description on rationale to select the sweep time properly in generic way as proposal 1. 5 and 6. But it's not favour to specify the specific value as mandatory one as pointed by other vendors. Hence there is concern on proposal 2 and 3. |
| CATT | Proposal 3 is based on the simulation outcome of signal level deviation v.s. average time. It might not be the final solution for this issue. However it shows the problem we have with current specification text. We can understand that TE vendors want to keep some implementation flexibility and do not want to have fixed value. However as indicated in R4-2117395, there is already some requirement or clarifications in ECC and FCC. I am wondering why adding similar text or clarification in 3GPP will be a problem. We are open to discuss feasible solutions. |

Issue 1-1-2: Comments collection for the draft CRs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2203978 | Anritsu: Thank you for the proposal. However as commented in issue 1-1-1 above, we prefer not defining the fixed sweeping time for the sake of a flexibility of test case implementation and test time optimization. |
|  | Keysight: Thank you for proposal. However, we don’t support setting fixed sweep time. same reason as we stated in contribution and above, also echo what Anritsu wrote. |
|  | Nokia: This will reduce flexibility of test setting to suit the signal under test. |
|  | NEC: Need further study if we adopt the fixed sweep time. |
| R4-2203981 | Anritsu: Same comment with R4-2203978. |
|  | Keysight: same comment as on R4-2203978 |
|  | Nokia: Same comment with R4-2203978. |
|  | NEC: Same comment with R4-2203978. |
| R4-2205150 | Anritsu: Thanks for proposing the compromised solution in the CR. We are fine with the proposed text. We are also open to discuss for adding some descriptions proposed by P5 in issue 1-1-1 above. |
|  | Keysight: Thank you for proposal. If I see key point correctly, which is to state “appropriate time duration” rather fixed number, we are OK with this approach, however, text under “NOTE” especially “e.g. at least a symbol length for each sweep point…” needs to be removed because same reason. |
|  | Nokia: Same comment with R4-2203978. |
|  | Huawei: thanks for the comments, we are ok to add some descriptions proposed by P5. We think the approach can address the identified issue and meanwhile can leave the full flexibility to the test implementation. On the text under “NOTE”, our intention is to give an example which is for information. We are open to further discussion on this aspect. |
|  | NEC: We are not fine with the NOTE. Text including “at least a symbol length for each sweep point” can not be accepted because it may be interrupted wrongly. Looking at this text, some may think “a symbol length for each sweep point” would be enough and meet the spec. However, for example, for 960 kHz SCS, a symbol length equals to 1.156 us. It would be too short. |
| R4-2205153 | Anritsu: Same comments with R4-2205150. |
|  | Keysight: same comment with R4-2205150 |
|  | Nokia: Same comment with R4-2203978. |
|  | Huawei: see the reply with R4-2205150. |
|  | NEC: Same comment with R4-2205150. |

1.4 Summary for 1st round

1.4.1 Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: Sweep time setting for unwanted emission testing | Based on 1st round discussion, most companies can support or be open for proposal 1, 5 and 6. For 2nd round discussion, moderator suggests to modify the draft CRs using proposal 1, 5 and 6 as starting point. - Proposal 1: The fixed sweep time setting is not defined in the conformance specification. (Anritsu, Keysight, Nokia, NEC, Samsung) - Proposal 2: Add a statement in the spec to judge each test result with a tightened test requirement which corresponds to the sweep time and its expected variance. The test shall be carried out again with the longer sweep time setting or longer average setting in case the obtained measurement results have exceeded the tightened test requirement.( Anritsu, Keysight, Huawei) - Proposal 3: the sweep time for a sample is a fix value of [40] us irrespective of the OFDM length. (CATT) - Proposal 4: Keep current text for unwanted emission testing in BS conformance test specification.(Keysight, Nokia, NEC) - Proposal 5: Add some calibration preparation text in procedure of TS38.141 for leading towards correctly adjust measurement parameters and add note for when averaging method is used (calculate average of multiple measured results). (Anritsu, Keysight, Huawei, Samsung) - Proposal 6: To include clarification that an appropriate time duration should be set to ensure the measurement is within the MU. (Anritsu, Keysight, Huawei, Samsung) |

1.4.2CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2203978 | Merged |
| R4-2203981 | Revised, using proposal 1, 5 and 6 as starting point |
| R4-2205150 | Revised, using proposal 1, 5 and 6 as starting point |
| R4-2205153 | Merged |

1.5 Discussion on 2nd round (if applicable)

2 Topic #2: Test configuration for NC operation

The CR provides corrections to the test conguration for NC operation which are constructed with fixed number of carriers, which was discussed in RAN4#101-e and one company require more time to check.

2.1 Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposal summary |
| --- | --- | --- |
| R4-2205159 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.141 R16 Reason/Summary of change: Existing NTC3 is constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. NTC21 has the similar issue |
| R4-2205161 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.145-1 R16 Reason/Summary of change: Existing ANTC3 and ANTC6 are constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers may not be required, but wider CBW and/or more carrier may be placed to reach the rated total output power. ANTC8 has similar issue |
| R4-2205163 | Huawei, HiSilicon | Correction on the test configuration for NC operation 37.145-2 R16 Reason/Summary of change: ANTC3, ANTC7 and ANTC9 are constructed with fixed two carriers to reflect high PSD scenarios. The test with total number of supported carriers is not required which need to corrected. |

2.2 Companies views’ collection for 1st round

2.2.1 CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2205159 | Nokia: - for ‘Consequences if not approved’, propose to use ‘NTC3 and NTC21 cannot be constructed as defined in some cases.’ - for additional statement in 4.8.3a.1, propose to use: In case rated total output power is not reached, the narrowest E-UTRA channel BW which supports rated carrier output power shall be selected. If still there is some output power room, alternately place an E-UTRA carrier of this BW adjacent to the carrier at the lower Base Station RF Bandwidth edge and UTRA carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge until the rated total output power or the total number of supported carriers is reached. |
|  | Huawei: we are ok to the proposed update from Nokia |
|  | Samsung: one clarification question is that whether update is needed for power allocation for NTC21a/b similar as NTC3 as to “If still there are some output power room, alternately place more carrier to reach rated total output power”? |
| R4-2205161 | Nokia: - for ‘Consequences if not approved’, propose to use ‘ANTC3, ANTC6 and ANTC8 cannot be constructed as defined in some cases.’ - for additional statement in 4.11.2.6.2, propose to use: In case rated total output power is not reached, the narrowest E-UTRA channel BW which supports rated carrier output power shall be selected. If still there is some output power room, alternately place an E-UTRA carrier of this BW adjacent to the carrier at the lower Base Station RF Bandwidth edge and UTRA carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge until the rated total output power or the total number of supported carriers is reached. - for additional statement in 4.11.2.10.2, propose to use: In case rated total output power is not reached, the narrowest E-UTRA and NR channel BW which supports rated carrier output power shall be selected. If still there is some output power room, alternately place an E-UTRA carrier of this BW adjacent to the carrier at the lower Base Station RF Bandwidth edge and NR carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge until the rated total output power or the total number of supported carriers is reached. - for additional statement in 4.11.2.14.2, ‘the narrowest channel BW’ is ambiguous as it is UTRA carrier. |
|  | Huawei: we are ok to the proposed update from Nokia. On the change in 4.11.2.14.2, it can be updated to “for the NR carrier adjacent to the lower Base Station RF Bandwidth edge, the narrowest NR channel BW which supports rated carrier output power shall be selected” |
| R4-2205163 | Nokia: - for ‘Consequences if not approved’, propose to use ‘ANTC3, ANTC7 and ANTC9 cannot be constructed as defined in some cases.’ - for additional statement in 4.11.2.6.2, propose to use: In case rated transmitter TRP per RIB is not reached, the narrowest E-UTRA channel BW which supports the rated carrier OTA BS power shall be selected. If still there is some output power room, alternately place an E-UTRA carrier of this BW adjacent to the carrier at the lower Base Station RF Bandwidth edge and UTRA carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge until the rated transmitter TRP per RIB or the total number of supported carriers is reached. - last paragraph in 4.11.2.6.3 should be deleted as there is no case (2) anymore. - for additional statement in 4.11.2.11.2, propose to use: In case rated transmitter TRP per RIB is not reached, the narrowest E-UTRA and NR channel BW which supports the rated carrier OTA BS power shall be selected. If still there is some output power room, alternately place an E-UTRA carrier of this BW adjacent to the carrier at the lower Base Station RF Bandwidth edge and NR carrier adjacent to the carrier at the upper Base Station RF Bandwidth edge until the rated transmitter TRP per RIB or the total number of supported carriers is reached. |
|  | Huawei: we are ok to the proposed update from Nokia. |
|  | Samsung: similar clarification question on power allocation to ANTC9. |
| Ericsson | With the new proposal from Nokia it is still not clear what happens, for example, if we have very little power left until total rated total output power and there is also one more WCDMA carrier possible to add. Will we add a carrier with only 1W, for example, just to fill in the power until total rated output power is reached? Or we place only carriers that can be allocated the whole rated output power per carrier? |

2.3 Summary for 1st round

2.3.1CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2205159 | To be revised, modification is needed to capture companies’ comments. |
| R4-2205161 | To be revised, modification is needed to capture companies’ comments. |
| R4-2205163 | To be revised, modification is needed to capture companies’ comments. |

2.4 Discussion on 2nd round (if applicable)

3 Recommendations for Tdocs

3.1 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2203562 | Sweep time setting of spectrum analyzer for BS unwanted emission TCs | Anritsu Corporation | Noted |  |
| R4-2203977 | Discussion on sweep time for unwanted emission testing | CATT | Noted |  |
| R4-2203978 | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-15) | CATT | Merged |  |
| R4-2203979 | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-16) | CATT | Not treated |  |
| R4-2203980 | draft CR for TS 38.141-1 On sweep time for unwanted emission testing (Rel-17) | CATT | Not treated |  |
| R4-2203981 | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-15) | CATT | Revised |  |
| R4-2203982 | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-16) | CATT | Return to |  |
| R4-2203983 | draft CR for TS 38.141-2 On sweep time for unwanted emission testing (Rel-17) | CATT | Return to |  |
| R4-2204435 | Discussion on the sweep time for unwanted emission testing | NEC | Noted |  |
| R4-2204711 | setting Sweep Time Requirement on Measuring BS Conformance Unwanted Emission testing | Keysight Technologies UK Ltd | Noted |  |
| R4-2205149 | Clarification for unwanted emission testing | Huawei, HiSilicon | Noted |  |
| R4-2205150 | Draft CR to 38.141-1: Clarification for unwanted emission testing | Huawei, HiSilicon | Revised |  |
| R4-2205151 | Draft CR to 38.141-1: Clarification for unwanted emission testing | Huawei, HiSilicon | Return to |  |
| R4-2205152 | Draft CR to 38.141-1: Clarification for unwanted emission testing | Huawei, HiSilicon | Return to |  |
| R4-2205153 | Draft CR to 38.141-2: Clarification for unwanted emission testing | Huawei, HiSilicon | Merged |  |
| R4-2205154 | Draft CR to 38.141-2: Clarification for unwanted emission testing | Huawei, HiSilicon | Not treated |  |
| R4-2205155 | Draft CR to 38.141-2: Clarification for unwanted emission testing | Huawei, HiSilicon | Not treated |  |
| R4-2205159 | Correction on the test configuration for NC operation 37.141 R16 | Huawei, HiSilicon | Revised |  |
| R4-2205160 | Correction on the test configuration for NC operation 37.141 R17 | Huawei, HiSilicon | Return to |  |
| R4-2205161 | Correction on the test configuration for NC operation 37.145-1 R16 | Huawei, HiSilicon | Revised |  |
| R4-2205162 | Correction on the test configuration for NC operation 37.145-1 R17 | Huawei, HiSilicon | Return to |  |
| R4-2205163 | Correction on the test configuration for NC operation 37.145-2 R16 | Huawei, HiSilicon | Revised |  |
| R4-2205164 | Correction on the test configuration for NC operation 37.145-2 R17 | Huawei, HiSilicon | Return to |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

3.2 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |
| Keysight | Takao Miyake | takao_miyake@keysight.com |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |
| Ericsson | Aurelian Bria | aurelian.bria@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)