3GPP TSG-RAN WG4 Meeting # 102-e	R4-2207223

# Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.16.10

Source:	Intel Corporation

Title:	WF on demodulation performance requirements definition for 52.6 - 71 GHz

Document for:	Approval

# Background

- Corresponding Email summary in RAN4#102-e

  - R4-2207446 Email discussion summary for [102-e][327] NR_exto71GHz_Demod_NWM.

# WF on topic #1: General

## General aspects of demodulation requirement definition

Issue 1-2-1: General scope of BS demodulation performance requirements

Define PUSCH, PUCCH, and PRACH performance requirements.

Issue 1-2-2: General scope of UE demodulation and CSI reporting performance requirements

Define PDSCH, PDCCH, PBCH, SDR performance requirements.

Issue 1-2-3: Scenarios to be considered for requirements definition

RAN4 defines demodulation requirements for the following scenarios:

- SA FR2-2

- NR DC or CA with FR1 anchor and FR2-2

RAN4 does not consider FR2-2 CA scenario in Rel-17

Issue 1-2-4: Shared spectrum access requirements

Define BS requirements without LBT

Define UE requirements without LBT

- FFS UE requirements with LBT

Issue 1-2-5: TDD pattern

Consider the following TDD pattern as a baseline for simulation results alignment:

- 120 kHz: 3D1S1U, S=10D:2G:2U

- 480 kHz: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U

- 960 kHz: 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U

Other options are not precluded

## Channel model for requirements definition

Issue 1-3-1: Propagation conditions

Both static propagation condition (for PRACH, CQI definition test) and Multi-path fading propagation conditions are considered.

Issue 1-3-2: RMS delay spread

TDL channel model:

- Option 1: TDL-A

- Option 2: TDL-D

RMS Delay spread:

- Option 1: 5ns

- Option 2: 10ns

- Option 3: 20ns

Note: Definition of requirements with different channel models is not precluded

Issue 1-3-3: Max Doppler frequency

Consider 3 km/h UE speed ( 200 Hz ).

FFS on higher UE speed.

- Option 1: 10 km/h (650 Hz)

- Option 2: 30 km/h (2000 Hz)

## Phase noise

Issue 1-4-1: Study on phase noise impact for requirements derivation

Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise.

- No Tx phase noise is modelled

- Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB)

- 70 GHz carrier frequency is assumed

Issue 1-4-2: Phase noise model

Consider PN model set 1 and 2 in TR 38.808 for initial simulation results alignment.

## Implementation of FR2-2 requirements into specification

Issue 1-5-1: Implementation of FR2-2 requirements into specification

Capture FR2-2 demodulation requirement into same section as FR2-1 but with different tables if possible.

# WF on topic #2: BS performance requirements

## SCS/CBW combinations

Issue 2-1-1: SCS for UL requirements definition

Consider the following SCS for UL requirements definition:

- 120, 480 kHz

- FFS 960 kHz

Issue 2-1-2: CBW for UL requirements definition

Consider minimum CBW for each SCS for UL requirements definition.

Consider maximum CBW for each SCS for UL requirements definition if no testability aspects are identified

- Perform link budget analysis to study max achievable SNR during the test

## General issues

Issue 2-2-1: FR2-1 requirements reuse

Do not apply FR2-1 performance requirements for FR2-2.

Issue 2-2-2: General simulation assumptions

Define UL performance requirements with:

- Normal CP

- With 1 and 2 Tx antennas

- With 2 demodulation branches

Issue 2-2-3: Test SNR limit

Take [20] dB SNR limit FR2-2 at starting point. New test cases and method should be defined if it is finally approved that FR2-2 SNR limit is much lower than [20] dB.

## PUSCH performance requirements

Issue 2-3-1: Scope of PUSCH performance requirements

Define the following PUSCH performance requirements:

- Performance requirements with transform precoding disabled with 70% throughput

- FFS performance requirements with transform precoding enabled

- FFS performance requirements for PUSCH repetition type A

Do not define the following PUSCH performance requirements:

- Performance requirements for 2-step RA type

- Performance requirements with mapping type B with non-slot transmission

- Performance requirements with UCI multiplexed on PUSCH

- Performance requirements with transform precoding disabled with 30% throughput

Issue 2-3-2: PUSCH performance requirements for multi-PUSCH scheduling

Do not define BS demodulation requirements for multi-PUSCH scheduling

Issue 2-3-3: PUSCH performance requirements with 32 UL HARQ processes

Companies converged to not define requirements with 32 UL HARQ processes

Issue 2-3-4: General PUSCH test setup

Consider the following simulation assumptions at starting point for PUSCH performance requirements:

- MCS 4, 16, 20

  - FFS MCS 4, 16, 20 for 1 Tx and MCS 4, 16 for 2 Tx

- DM-RS/PT-RS configuration Rel-15 assumptions

- 1+1 DMRS configuration

- PTRS Tx on

- Temporary PRB number (for simulations next meeting):

- (66)(264)(66)(132)(264)(32)(66)(132)(165) for SCS (kHz CWB (MHz)) = (120 100)(120 400)(480 400) (480 800)(480 1600)(960 400)(960 800)(960 1600)(960 2000)

Issue 2-3-5: Detailed PUSCH test setup

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric | Normalized throughput | 70% |

Issue 2-3-6: Rx processing assumptions

Define PUSCH performance requirements by using phase noise compensation

- Companies are encouraged to bring results for CPE and CPE+ICI compensation methods.

Issue 2-3-7a: Other

CBW for performance requirements with transform precoding enabled:

- Only the smallest CBWs for each SCS.

Issue 2-3-7b: Other

Applicability rule:

- FFS consider application of FR2-1 applicability rules for FR2-2 as well.

## PUCCH performance requirements

Issue 2-4-1: Scope of PUCCH performance requirements

Define performance requirements for the enhanced PUCCH formats 0, 1, and 4.

Define new requirements for PUCCH formats 2 and 3.

Issue 2-4-2a: Detailed PUCCH test setup for PUCCH format 0

Number of PRBs:

- Option 1: 16 (enhanced format)

- Option 2: 10 (enhanced format)

Number of OFDM symbols:

- Option 1: 1

- Option 2: 1 and 2

Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1

Test metric: ACK missed detection rate < 1%

Issue 2-4-2b: Detailed PUCCH test setup for PUCCH format 1

Number of PRBs:

- Option 1: 1

- Option 2: 16 (enhanced format)

- Option 3: 10 (enhanced format)

Number of OFDM symbols:

- Option 1: 14

Intra-slot frequency hopping: enabled

Test metric: ACK missed detection rate < 1% and NACK to ACK probability < 0.1%

Issue 2-4-2c: Detailed PUCCH test setup for PUCCH format 2

Number of PRBs:

- Option 1: 9

Number of OFDM symbols:

- Option 1: 2

- Option 2: 1 and 2

Intra-slot frequency hopping: enabled

Test metric: ACK missed detection rate < 1%, UCI BLER < 1%

Issue 2-4-2d: Detailed PUCCH test setup for PUCCH format 3

Number of PRBs:

- Option 1: 1 and 3

Number of OFDM symbols:

- Option 1: 14 and 4

- Option 2: 4

Additional DMRS

- Option 1: With and without

- Option 2: Without

Intra-slot frequency hopping: enabled

Test metric: UCI BLER < 1%

Issue 2-4-2e: Detailed PUCCH test setup for PUCCH format 4

Number of PRBs:

- Option 1: 16 (Enhanced)

- Option 2: 10 (Enhanced)

Number of OFDM symbols:

- Option 1: 14

Additional DMRS

- Option 1: With and without

Intra-slot frequency hopping: enabled

Test metric: UCI BLER < 1%

## PRACH performance requirements

Issue 2-5-1: Scope of PRACH requirements

Define the following PRACH performance requirements:

- False alarm probability requirements

- PRACH miss detection requirements

Issue 2-5-2: SCS

PRACH SCS:

- 120, 480 kHz

- FFS 960 kHz

Issue 2-5-3: Sequence length

PRACH sequence length:

- 120 kHz:

  - 1151, 571, 139

- 480 kHz:

  - 571, 139

- 960 kHz:

  - 139

Issue 2-5-4: PRACH formats

PRACH formats: A2, B4, C2

Issue 2-5-6: Frequency offset

- 7100 Hz

Issue 2-5-7: Time error tolerance

Time error tolerance:

- 0.07us for AWGN

- FFS for multi-path fading

Issue 2-5-8: (Ncs, logical sequence index, v)

- Consider the following PRACH parameters as starting point: Ncs = 69, logical sequence index = 0, v =0 as starting point

- Other options are not precluded

# WF on topic #3: UE performance requirements

## SCS/CBW combinations

Issue 3-1-1: SCS for DL requirements definition

Consider the following SCS for DL requirements definition:

- 120, 480 kHz

- FFS 960 kHz

Issue 3-1-2: CBW for DL requirements definition

120 kHz:

- 100, 400 MHz

480 MHz:

- 400 MHz

- FFS on 1600 MHz

960 kHz:

- 400 MHz

- FFS on 2000 MHz

## General issues

Issue 3-2-1: FR2-1 requirements reuse for 120 kHz SCS

Do not apply FR2-1 performance requirements for FR2-2.

Issue 3-2-2: General simulation assumptions

Consider the following simulation assumptions at starting point for PDSCH performance requirements:

- Normal CP

- 1 and 2 Tx antennas and 2 Rx antennas

- Up to 64QAM modulation order

- Temporary PRB number:

  - (66)(264)(66)(132)(264)(32)(66)(132)(165) for SCS (kHz CWB (MHz)) = (120 100)(120 400)(480 400) (480 800)(480 1600)(960 400)(960 800)(960 1600)(960 2000)

Issue 3-2-3: Other

Increase the FLD_high max frequency to cover FR2-2 frequency bands, i.e. “FDL_high may not exceed 71000 MHz” and reconsider existing.

## PDSCH performance requirements

Issue 3-3-2: PDSCH performance requirements for multi-PDSCH scheduling

Option 1: Define PDSCH performance requirements with the following assumptions:

- 120 kHz SCS: Single TB scheduling

- 480 kHz SCS:  4-TB scheduling

- 960 kHz SCS: 8-TB scheduling

Option 2: Do not define PDSCH performance requirements with multi-TB scheduling

Option 3: Define multi-slot scheduling PDSCH performance requirements with the following assumptions:

- 480 kHz SCS:  4-TB scheduling

- 960 kHz SCS: 8-TB scheduling

Issue 3-3-3: PDSCH performance requirements with 32 DL HARQ processes

Option 1: Define one test case to verify PDSCH performance with 32 DL HARQ processes with the test metric 30% of maximum throughput.

Option 2: Do not define requirements for PDSCH with 32 DL HARQ processes

Issue 3-3-4: Mapping type

Define PDSCH requirements with mapping type A

FFS define PDSCH test case to verify mapping type B processing.

Issue 3-3-5: Requirements with 30% throughput

Option 1: Define PDSCH requirement with 30% throughput.

Option 2: Do not define PDSCH requirement with 30% throughput.

Issue 3-3-6: Requirements for enhanced receiver type 1

Do not define PDSCH requirements with enhanced receiver type 1.

Issue 3-3-7: MCS, modulation order for PDSCH requirements

Define PDSCH requirements with MCS 4, MCS 13 and MCS [22]

- Other MCS values are not precluded

Issue 3-3-8: Rank

Define PDSCH requirements with:

- Rank 1

  - FFS FD-OCC is not applied to all the antenna ports for DMRS

- FFS Rank 2

Issue 3-3-9: PTRS configuration

Consider PTRS configuration as K=2, L=1

Issue 3-3-11: Transmission burst model (if requirements with LBT will be introduced)

Option 1: Use transmission burst model defined in LAA as start point to be discussed and set the gap between two transmission bursts at least to 4/8/16 OFDM symbols for 120/480/960 kHz SCS.

Option 2: Use Rel-16 NR-U Transmission burst model

Issue 3-3-12: Rx processing assumptions

Define PDSCH performance requirements by using phase noise compensation

- Companies are encouraged to bring results for CPE and CPE+ICI compensation methods

## PDCCH performance requirements

Issue 3-4-1: Performance requirements for multi-slot PDCCH monitoring

Option 1: Define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS

Option 2: Do not define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS

Issue 3-4-2: PDCCH simulation assumptions

Use following assumptions for PDCCH performance test.

- Antenna configuration: 1T2R and 1T4R

- Aggregation level: 2 and 4 for 1T2R; 8 and 16 for 2T2R

## PBCH performance requirements

PBCH simulation assumptions (if introduced pending on outcome of issues Issue 1-2-2)

SSB Index:

- Option 1: Only with not known SSB index

- Option 2: Both known and not known SSB index

- Option 3: Only with known SSB index

- Option 4: Not known SSB index for 120 kHz and known SSB index for 480/960 kHz.

## SDR performance requirements

Issue 3-6-1: SDR performance requirements (if introduced pending on outcome of issues Issue 1-2-2

If requirements will be introduced, study MCS configuration applicability for FR2-2 from FR2-1 and SNR values to achieve 85% throughput.

## CSI reporting requirements

Issue 3-7-1: Scope of CSI reporting requirements

- Define CQI reporting requirements only under static propagation conditions.

- Do not define PMI and RI reporting requirements in Rel-17

Issue 3-7-2: CQI reporting requirements under static propagation conditions

Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

# Work split (for information)

Table 1. Demod CR work split for TS 38.104

| draftCR and bigCR work-split - TS 38.104 |  |  | Company |
| --- | --- | --- | --- |
| TS 38.104 bigCR |  |  | Intel |
| TS 38.104 BS Performance requirements (draftCRs) |  |  |  |
|  | 11 Radiated performance requirements |  |  |
|  |  | PUSCH requirements | Intel |
|  |  | PUCCH requirements | Huawei |
|  |  | PRACH requirements | Nokia |
|  | Appendix |  |  |
|  |  | PUSCH FRCs | Ericsson |
|  |  | , PRACH Test preambles, propagation conditions | Samsung |

Table 2. Demod CR work split for TS 38.141-2

| draftCR and bigCR work-split - TS 38.141-2 |  |  | Company |
| --- | --- | --- | --- |
| TS 38.141-2 bigCR |  |  | Nokia |
| TS 38.141-2 BS Radiated conformance testing (draftCRs) |  |  |  |
|  | 8 Radiated performance requirements |  |  |
|  |  | PUSCH requirements | Huawei |
|  |  | PUCCH requirements | Intel |
|  |  | PRACH requirements | Nokia |
|  | Appendix |  |  |
|  |  | PUSCH FRCs | Ericsson |
|  |  | PRACH Test preambles, propagation conditions, test tolerance | Samsung |

Table 3. Demod CR work split for TS 38.101-4

| draftCR and bigCR work split - TS 38.101-4 |  |  |  | Company |
| --- | --- | --- | --- | --- |
| TS 38.101-4 bigCR |  |  |  | Huawei |
| TS 38.101-4 UE performance requirements (draftCRs) |  |  |  |  |
|  | 7 Demodulation performance requirements (Radiated requirements) |  |  |  |
|  |  | General (incl. applicability rule) |  | Qualcomm |
|  |  | PDSCH requirements |  | Huawei, Ericsson |
|  |  | PDCCH requirements |  | Nokia |
|  |  | (PBCH requirements) |  | Qualcomm |
|  |  | SDR requirements |  | Ericsson |
|  | 8 CSI reporting requirements (Radiated requirements) |  |  |  |
|  |  |  | General (incl. applicability rule), CQI reporting requirements | Apple |
|  | Appendix |  |  |  |
|  |  | PDSCH, PDCCH, (PBCH) RMCs |  | Apple |
|  |  | CSI RMCs and propagation conditions |  | Apple |