3GPP TSG-RAN WG4 Meeting # 102-e							R4-2207284

Electronic Meeting, February 21 – March 3, 2022

Source: 	CMCC

Title:  	WF on FR1 OOB gain and ACRR

Agenda Item:	10.5.2

Document for:	Approval

# Introduction

This WF capture the agreements on FR1 OOB gain and ACRR requirements.

# OOB gain

## WA / MR repeater class for both DL and UL

    - For below 2496MHz frequencies

Follow same OOB gain as E-UTRA:

Table 1: out of band gain limits 1

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0,2 f_offset_CW < 1,0 MHz | 60 dB |
| 1,0 f_offset_CW < 5,0 MHz | 45 dB |
| 5,0 f_offset_CW < 10,0 MHz | 45 dB |
| 10,0 MHz f_offset_CW | 35 dB |

For 10,0 MHz  f_offset_CW the out of band gain shall not exceed the maximum gain of table 1 or the maximum gain stated in table 2 whichever is lower.

Table 2: Out of band gain limits 2

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 10 MHz f_offset_CW | Out of band gain minimum donor coupling loss |

    - For above 2496MHz frequencies

Follow following limits:

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0,2< f_offset_CW < 4,0 MHz | 60 dB |
| 4,0< f_offset_CW < 15,0 MHz | 45 dB |
| 15,0 MHz <f_offset_CW | 35 dB |

## LA repeater class for both DL and UL

Two sets of requirements. Repeater declare which limit is supported based on the real deployment case.

- The first one: in which the operator owns the whole band or collaborates with operators in the whole band and so the repeater passband covers the whole 3GPP band

  - No OOB gain requirement



- The second one: in which there are other un-coordinated operators in the band and so the repeater passband covers a portion of the 3GPP band.

  - Same OOB gain requirement as WA in this case

# ACRR

ACRR is differentiated between UL and DL. For ACRR, consider the co-existence scenario with NR, LTE and UTRAN system. Adjacent channel frequency offset for ACRR requirements: minimum {100MHz, passband BW}.

## UL

- For WA, 33dBc ACRR.

- For LA,

  - Whole passband: no ACRR

  - Part of band:

    - <2496MHz: 33dBc

    - >2496MHz: 20dBc in 10MHz BW, 33dBc with minimum {100 MHz, passband BW} (2 tests)

## DL

- For below 2496MHz:

  - WA/MR class: 45dBc

  - LA: NA for whole band case and 33dBc for part of band case

- For above 2496MHz:

  - WA/MR class: 33dBc

  - LA: NA for whole band case and 33dBc for part of band case