3GPP TSG-RAN4 Meeting #103-e	R4-2208200

# Electronic meeting, May 9 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | Draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TS 38.171 requirements for support of A-GNSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CAICT, CENC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI |  |  |  |  |  | Date: |  |  | 2022-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The BDS ICD reference for B1I signal in TS 38.171 is incorrect. - The applicable clause of EN-DC combination in TS 38.171 section B.1.13 is incorrect (the band combination is defined for several GNSSs but referred to clause 5.1 which is for GPS only). - Table C.2: Ref. BDS ICD, the value X for B1C signal is incorrect. - Table C.3: Ref. BDS ICD, the ratio for B1C signal is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the BDS ICD reference in TS 38.171. - Correct the applicable clause in TS 38.171 section B.1.13. - Table C.2: Ref. BDS ICD, update the value in the BDS entry as: Signal = B1C, X [m] = 0.5 * c / 1.023 Mcps = 150 m. - Table C.3: Ref. BDS ICD, update the value in BDS entry in this table as: Signal = B1C, Ratio N = 1575.420 MHz (Carrier Freq) / 1.023 Mcps (Chipping rate) = 1540. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The BDS reference for B1I in TS 38.171 is incorrect. - The applicable clause of EN-DC combination in TS 38.171 section B.1.13 is incorrect. - The values in Table C.2 and Table C.3 for B1C signal are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, B.1.13, C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[2]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[3]	3GPP TS 37.571-1: " User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance".

[4]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[5]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[6]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements".

[7]	ETSI TR 102 273-1-2: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[8]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[9]	P. Axelrad, R.G. Brown, "GPS Navigation Algorithms", in Chapter 9 of "Global Positioning System: Theory and Applications", Volume 1, B.W. Parkinson, J.J. Spilker (Ed.), Am. Inst. of Aeronautics and Astronautics Inc., 1996.

[10]	S.K. Gupta, "Test and Evaluation Procedures for the GPS User Equipment", ION-GPS Red Book, Volume 1, p. 119.

[11]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Special conformance testing functions for User Equipment (UE)".

[12]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[13]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[14]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[15]	Galileo OS Signal in Space ICD (OS SIS ICD), Draft 0, Galileo Joint Undertaking, May 23rd, 2006.

[16]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[17]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[18]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", China Satellite Navigation Office, February 2019.

[19]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[20]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[21]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[22]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

<End of Change 1>

<Start of Change 2>

B.1.13	EN-DC band combinations for testing A-GNSS sensitivity

B.1.13.1	EN-DC band combination groups

For the A-GNSS sensitivity requirements in EN-DC operation mode with uplink assigned to E-UTRA and NR frequency bands, the requirements in clause 6.1 can be verified by one EN-DC band combination in each of the applicable Frequency Group Combination specified in Table B.1.13.1-1. The A-GNSS sensitivity requirements for the remaining applicable EN-DC band combinations in each Frequency Group Combination are considered to have been verified by using the one EN-DC band combination in each Frequency Group Combination. The Frequency Groups are defined in Table B.1.13.1-2.

The applicable EN-DC band combinations for verifying A-GNSS sensitivity requirements in EN-DC operation mode are specified in clause B.1.13.2.

Table B.1.13.1-1: EN-DC band combination groups for verifying A-GNSS sensitivity requirements in EN-DC operation mode

| Frequency Group Combination | EN-DC Band Combinations |
| --- | --- |
| Group VHF-VHF | NA |
| Group VHF-LB | NA |
| Group VHF-MLB | NA |
| Group VHF-MB | NA |
| Group VHF-HB | NA |
| Group VHF-UHB1 | NA |
| Group VHF-UHB2 | NA |
| Group LB-VHF | NA |
| Group LB-LB | DC_5A_n12A DC_5A_n71A DC_8A_n20A DC_8A_n28A DC_12A_n5A DC_20A_n8A DC_20A_n28A DC_28A_n5A DC_28A_n8A DC_71A_n5A DC_20A_n83A |
| Group LB-MLB | DC_20A_n50A DC_20A_n51A DC_28A_n51A DC_28A_n50A |
| Group LB-MB | DC_5A_n2A DC_5A_n66A DC_8A_n1A DC_8A_n3A DC_8A_n34A DC_8A_n39A DC_12A_n2A DC_12A_n25A DC_12A_n66A DC_18A_n3A DC_20A_n1A DC_20A_n3A DC_26A_n25A DC_28A_n3A DC_71A_n66A DC_8A_n80A DC_20A_n80A |
| Group LB-HB | DC_5A_n7A DC_5A_n38A DC_5A_n40A DC_8A_n40A DC_8A_n41A DC_12A_n7A DC_12A_n38A DC_12A_n41A DC_20A_n7A DC_20A_n38A DC_20A_n41A DC_26A_n41A DC_28A_n7A DC_28A_n40A DC_28A_n41A DC_71A_n38A |
| Group LB-UHB1 | DC_5A_n48A DC_5A_n78A DC_8A_n77A DC_8A_n78A DC_12A_n78A DC_18A_n77A DC_18A_n78A DC_19A_n77A DC_19A_n78A DC_20A_n77A DC_20A_n78A DC_26A_n77A DC_26A_n78A DC_28A_n77A DC_28A_n78A DC_71A_n48A DC_71A_n78A |
| Group LB-UHB2 | DC_5A_n79A DC_8A_n79A DC_18A_n79A DC_19A_n79A DC_26A_n79A DC_28A_n79A |
| Group MLB-VHF | NA |
| Group MLB-LB | DC_11A_n28A |
| Group MLB-MLB | NA |
| Group MLB-MB | DC_11A_n3A |
| Group MLB-HB | NA |
| Group MLB-UHB1 | DC_11A_n77A DC_11A_n78A DC_21A_n77A DC_21A_n78A |
| Group MLB-UHB2 | DC_11A_n79A DC_21A_n79A |
| Group MB-VHF | NA |
| Group MB-LB | DC_1A_n5A DC_1A_n8A DC_1A_n20A DC_1A_n28A DC_1A_n71A DC_2A_n5A DC_2A_n12A DC_2A_n71A DC_3A_n5A DC_3A_n8A DC_3A_n20A DC_3A_n28A DC_3A_n71A DC_66A_n5A DC_66A_n12A DC_66A_n71A DC_3A_n82A |
| Group MB-MLB | DC_1A_n50A DC_1A_n51A DC_3A_n50A DC_3A_n51A |
| Group MB-MB | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_3A_n34A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_2A_n2A2 DC_66A_n66A2 DC_3A_n3A2 DC_3A_n84A |
| Group MB-HB | DC_1A_n7A DC_1A_n38A DC_1A_n40A DC_1A_n41A DC_2A_n7A DC_2A_n38A DC_2A_n41A DC_3A_n7A DC_3A_n38A DC_3A_n40A DC_3A_n41A DC_4A_n38A DC_4A_n41A DC_25A_n41A DC_39A_n40A DC_39A_n41A DC_66A_n7A DC_66A_n38A DC_66A_n41A |
| Group MB-UHB1 | DC_1A_n77A DC_1A_n78A DC_2A_n48A DC_2A_n78A DC_3A_n77A DC_3A_n78A DC_4A_n78A DC_39A_n78A DC_66A_n48A DC_66A_n78A |
| Group MB-UHB2 | DC_1A_n79A DC_3A_n79A DC_39A_n79A |
| Group HB-VHF | NA |
| Group HB-LB | DC_7A_n5A DC_7A_n8A DC_7A_n20A DC_7A_n28A DC_7A_n71A DC_30A_n5A DC_41A_n28A |
| Group HB-MLB | DC_7A_n51A |
| Group HB-MB | DC_7A_n1A DC_7A_n3A DC_7A_n66A DC_30A_n2A DC_30A_n66A DC_40A_n1A DC_41A_n3A DC_7A_n80A DC_38A_n3A |
| Group HB-HB | DC_7A_n40A DC_40A_n41A DC_7A_n7A2 DC_41A_n41A |
| Group HB-UHB1 A(NOTE 1): | DC_40A_n77A DC_40A_n78A |
| B(NOTE 2): | DC_7A_n78A DC_38A_n78A DC_41A_n78A |
| C(NOTE 3): | DC_7A_n77A DC_41A_n77A |
| Group HB-UHB2 | DC_40A_n79A DC_41A_n79A |
| Group UHB1-VHF | NA |
| Group UHB1-LB | NA |
| Group UHB1-MLB | NA |
| Group UHB1-MB | NA |
| Group UHB1-HB | NA |
| Group UHB1-UHB1 | NA |
| Group UHB1-UHB2 | NA |
| Group UHB2-VHF | NA |
| Group UHB2-LB | NA |
| Group UHB2-MLB | NA |
| Group UHB2-MB | NA |
| Group UHB2-HB | NA |
| Group UHB2-UHB1 | NA |
| Group UHB2-UHB2 | NA |
| NOTE 1: This sub-group generates second order intermodulation products. NOTE 2: This sub-group generates third order intermodulation products. NOTE 3: This sub-group generates second and third order intermodulation products. |  |

Table B.1.13.1-2: Definition of Frequency Groups

| Frequency Group | Frequency Range (MHz) |
| --- | --- |
| VHF | 400.0 – 458.0 |
| LB | 662.0 – 916.0 |
| MLB | 1426.0 – 1518.0 |
| MB | 1626.0 – 2025.0 |
| HB | 2300.0 – 2690.0 |
| UHB1 | 3300.0 – 4201.0 |
| UHB2 | 4400.0 – 5000.0 |

B.1.13.2	Applicable EN-DC band combinations for verifying A-GNSS sensitivity requirements

The A-GNSS sensitivity requirements in clause 6.1 when in EN-DC operation mode shall be verified for EN-DC band combinations that can generate second or third order intermodulation products falling into the following GNSS receiver bands for the particular GNSS (where supported by the UE):

-	GPS L1 C/A:				1574.3970 – 1576.4430 MHz

-	Galileo E1 / GPS L1C:	1573.3740 – 1577.4660 MHz

-	GLONASS G1:			1597.5515 – 1605.8860 MHz

-	BDS B1I:					1559.0520 – 1563.1440 MHz

For each frequency group combination in Table B.1.13.2-1 only one EN-DC band combination needs to be tested for the supported GNSS.

Table B.1.13.2-1: EN-DC band combinations for verifying A-GNSS sensitivity requirements

| Frequency Group Combination | EN-DC band combinations |  |  |
| --- | --- | --- | --- |
|  | GPS L1 / Galileo E1 | GLONASS G1 | BDS B1 |
| Group LB-LB | DC_20A_n28A DC_28A_n5A DC_20A_n83A | DC_8A_n28A DC_20A_n28A DC_28A_n8A DC_20A_n83A | DC_5A_n12A DC_12A_n5A DC_20A_n28A DC_28A_n5A DC_20A_n83A |
| Group LB-MLB | NA | NA | NA |
| Group LB-MB | NA | NA | NA |
| Group LB-HB | DC_5A_n40A DC_28A_n40A | DC_8A_n41A DC_28A_n40A | DC_5A_n40A DC_28A_n40A |
| Group LB-UHB1 | DC_8A_n77A DC_8A_n78A DC_20A_n77A DC_20A_n78A | DC_5A_n78A DC_8A_n77A DC_8A_n78A DC_20A_n77A DC_20A_n78A DC_26A_n77A DC_26A_n78A | DC_8A_n77A DC_8A_n78A |
| Group LB-UHB2 | NA | NA | NA |
| Group MLB-LB | NA | NA | NA |
| Group MLB-MLB | NA | NA | NA |
| Group MLB-MB | NA | NA | NA |
| Group MLB-HB | NA | NA | NA |
| Group MLB-UHB1 | NA | NA | NA |
| Group MLB-UHB2 | DC_11A_n79A DC_21A_n79A | DC_11A_n79A DC_21A_n79A | DC_11A_n79A DC_21A_n79A |
| Group MB-LB | NA | NA | NA |
| Group MB-MLB | NA | NA | NA |
| Group MB-MB | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A |
| Group MB-HB | DC_1A_n40A | DC_1A_n7A | DC_1A_n7A |
| Group MB-UHB1 | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n78A DC_4A_n78A DC_39A_n78A DC_66A_n78A | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n79A DC_4A_n78A DC_39A_n78A DC_66A_n78A | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n79A DC_4A_n78A DC_39A_n78A DC_66A_n78A |
| Group MB-UHB2 | DC_3A_n79A | NA | DC_3A_n79A |
| Group HB-LB | NA | DC_7A_n8A | NA |
| Group HB-MLB | NA | NA | NA |
| Group HB-MB | DC_40A_n1A | DC_40A_n1A | DC_40A_n1A |
| Group HB-HB | NA | NA | NA |
| Group HB-UHB1 A: | DC_40A_n77A | DC_40A_n77A | DC_40A_n77A |
| B: | DC_7A_n78A DC_38A_n78A DC_41A_n78A | DC_7A_n78A DC_38A_n78A DC_41A_n78A | DC_7A_n78A DC_38A_n78A DC_41A_n78A |
| C: | DC_7A_n77A DC_41A_n77A | DC_7A_n77A DC_41A_n77A | DC_7A_n77A DC_41A_n77A |
| Group HB-UHB2 | NA | NA | NA |

B.1.13.3	Test frequencies for EN-DC band combinations

For verifying the sensitivity requirements in clause 6.1 in EN-DC operation mode, the E-UTRA and NR frequency and channel configuration shall be selected to ensure the intermodulation products fall into the GNSS receiver bands as defined in clause B.1.13.2 for the particular GNSS.

<End of Change 2>

<Start of Change 3>

C.1	Static propagation conditions

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading and multi-paths exist for this propagation model.

C.2	Multi-path case

Doppler frequency difference between direct and reflected signal paths is applied to the carrier and code frequencies. The Carrier and Code Doppler frequencies of LOS and multi-path for GNSS signal are defined in table C.1.

Table C.1: Multipath case

| Initial relative delay [m] | Carrier Doppler frequency of tap [Hz] | Code Doppler frequency of tap [Hz] | Relative mean power [dB] |
| --- | --- | --- | --- |
| 0 | Fd | Fd / N | 0 |
| X | Fd - 0.1 | (Fd-0.1) /N | Y |
| NOTE: Discrete Doppler frequency is used for each tap. |  |  |  |

Where the X and Y depends on the GNSS signal type and is shown in Table C.2, and N is the ratio between the transmitted carrier frequency of the signals and the transmitted chip rate as shown in Table C.3 (where k in Table C.3 is the GLONASS frequency channel number).

Table C.2: Parameter values

| System | Signals | X [m] | Y [dB] |
| --- | --- | --- | --- |
| BDS | B1I | 75 | -4.5 |
|  | B1C | 150 | -4.5 |
| Galileo | E1 | 125 | -4.5 |
|  | E5a | 15 | -6 |
|  | E5b | 15 | -6 |
| GLONASS | G1 | 275 | -12.5 |
|  | G2 | 275 | -12.5 |
| GPS/Modernized GPS | L1 C/A | 150 | -6 |
|  | L1C | 125 | -4.5 |
|  | L2C | 150 | -6 |
|  | L5 | 15 | -6 |

Table C.3: Ratio between carrier frequency and chip rate

| System | Signals | N |
| --- | --- | --- |
| BDS | B1I | 763 |
|  | B1C | 1540 |
| Galileo | E1 | 1540 |
|  | E5a | 115 |
|  | E5b | 118 |
| GLONASS | G1 | 3135.03 + k 1.10 |
|  | G2 | 2438.36 + k 0.86 |
| GPS/Modernized GPS | L1 C/A | 1540 |
|  | L1C | 1540 |
|  | L2C | 1200 |
|  | L5 | 115 |

The initial carrier phase difference between taps shall be randomly selected between 0 and 2. The initial value shall have uniform random distribution.

<End of Change 3>