3GPP TSG-RAN WG4 Meeting #103-e                                                                                      R4-2208253

Electronic Meeting, May 09 – May 20, 2022

Source: 	CATT

Title: 	General consideration on radiated conformance testing for SAN

Agenda item:	9.12.4.3

Document for:	Approval

- Introduction

In this contribution, we provide our general consideration on radiated conformance testing for SAN.

- Discussion

According to the agreed TP[1,2], the frequency range, operating bands in FR1 and Maximum transmission bandwidth configuration for NTN SAN are described in Table 2-1, Table 2-2, and Table 2-3, respectively.

Table 2-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |

Table 2-2: NTN satellite operating bands in FR1

| NTN satelliteoperating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE 1: NTN satellite bands are numbered in descending order from n256.. |  |  |  |

Table 2-3: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 |
| 30 | 11 | 24 | 38 | 51 |
| 60 | N/A | 11 | 18 | 24 |

Acceptable uncertainty of Test System

The measurement uncertainty is dependent on operating frequency and carrier numerology parameters (such as carrier channel bandwidth and sub-carrier spacing). The measurement uncertainty for f ≤ 3 GHz for transmitter tests and receiver tests defined in 38.141-2 are copied in the following Table 4.1.2.2-1 and Table 4.1.2.3-1, respectively.

The carrier channel bandwidths (5, 10, 15 and 20MHz) in above Table 2-3 are same with carrier channel bandwidths defined in TS 38.104.  So measurement uncertainty for occupied bandwidth defined in 38.141-2 can be reused for NTN SAN (n255, and n256).

The maximum frequency of n256 and n255 defined in above Table 2-2 is 2.2 GHz, which is less than 3GHz. So, the measurement uncertainty for f ≤ 3 GHz for transmitter tests and receiver tests defined in 38.141-2 can be reused for NTN SAN (n255, and n256).

Proposal 1: The measurement uncertainty for f ≤ 3 GHz for transmitter tests and receiver tests defined in 38.141-2 can be reused for SAN type 1-H and SAN type 1-O (n255, and n256).

Table 4.1.2.2-1: Maximum OTA Test System uncertainty for FR1 OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.1 dB, f ≤ 3 GHz |
|  | Extreme condition: ±2.5 dB, f ≤ 3 GHz |
| 6.3 OTA base station output power | ±1.4 dB, f ≤ 3.0 GHz |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±3.4 dB, f ≤ 3.0 GHz |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | ±1 % |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | ±100 kHz, BWChannel 5 MHz, 10 MHz ±300 kHz, BWChannel 15 MHz, 20 MHz |
| 6.7.3 OTA ACLR/CACLR | f ≤ 3.0 GHz ±1 dB, BW ≤ 20MHz ±1 dB, BW > 20MHz Absolute power ±2.2 dB, f ≤ 3.0 GHz |
| 6.7.4 OTA operating band unwanted emissions | Absolute power ±1.8 dB, f ≤ 3.0 GHz |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz < f ≤ 6 GHz ±4.2 dB, 6 GHz < f ≤ 26 GHz |
| 6.7.5.3 OTA transmitter spurious emissions, protection of BS receiver | ±3.1 dB, f ≤ 3 GHz |
| 6.7.5.4 OTA transmitter spurious emissions, additional spurious emissions requirements | ±2.6 dB, f ≤ 3 GHz |
| 6.7.5.5 OTA transmitter spurious emissions, co-location | ±3.1 dB, f ≤ 3 GHz |
| 6.8 OTA transmitter intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in6.7.3 (ACLR), 6.7.4 (OBUE) and 6.7.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±3.2 dB, f ≤ 3.0 GHz |

Table 4.1.2.3-1: Maximum OTA Test System uncertainty for FR1 OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 7.2 OTA sensitivity | ±1.3 dB, f ≤ 3.0 GHz |
| 7.3 OTA reference sensitivity level | ±1.3 dB, f ≤ 3.0 GHz |
| 7.4 OTA dynamic range | ±0.3 dB |
| 7.5.1 OTA adjacent channel selectivity | ±1.7 dB, f ≤ 3.0 GHz |
| 7.5.2 In-band blocking (General) | ±1.9 dB, f ≤ 3.0 GHz |
| 7.5.2 In-band blocking (Narrowband) | ±1.7 dB, f ≤ 3.0 GHz |
| 7.6 OTA out-of-band blocking (General) | fwanted ≤ 3.0 GHz: ±2.0 dB, finterferer ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < finterferer ≤ 6.0 GHz ±3.5 dB, 6.0 GHz < finterferer ≤ 12.75 GHz |
| 7.6 OTA out-of-band blocking (Co-location) (NOTE 1) | fwanted ≤ 3.0 GHz: ±3.4 dB, finterferer ≤ 3.0 GHz ±3.5 dB, 3.0 GHz < finterferer ≤ 4.2 GHz ±3.7 dB, 4.2 GHz < finterferer ≤ 6.0 GHz |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 30 MHz ≤ f ≤ 6.0 GHz ±4.2 dB, 6.0 GHz < f ≤ 26 GHz |
| 7.8 OTA receiver intermodulation | ±2.0 dB, f ≤ 3.0 GHz |
| 7.9 OTA in-channel selectivity | ±1.7 dB, f ≤ 3.0 GHz |

Manufacturer declarations

For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H For SAN type 1-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Figure 4.3.2-1: Radiated reference points for SAN type 1-O

According to agreed TP[3], the SAN type 1-H  and SAN type 1-O was defined for NTN. So for radiated conformance testing, the BS declarations for the radiated requirements testing of the SAN type 1-O are required. And for SAN type 1-H declarations for the radiated requirements testing are also required. The method of manufacturer declarations for BS type 1-H, BS type 1-O radiated test requirements in TS 38.141-2 can be reused for manufacturer declarations for SAN type 1-H, SAN type 1-O radiated test requirements.

Proposal 2: The method of manufacturer declarations for BS type 1-H, BS type 1-O radiated test requirements in TS 38.141-2 can be reused for manufacturer declarations for SAN type 1-H, SAN type 1-O radiated test requirements.

SAN configurations

The radiated transmitter and receiver characteristics are specified at RIB. So figures for the RIB interfaces for SAN transmitter and receiver are needed.

Proposal 3:  To define figures for the RIB interfaces for SAN transmitter and receiver.

Test configurations

The current test configurations in TS 38.141-2 are defined for multi-carrier or multi-band conformance testing.  Since it was agreed to not consider multi-band operation in Rel-17 for NTN, only test configurations for multi-carrier operation can be reused from TN.

Proposal 4: Test configurations for multi-carrier operation can be reused for SAN type 1-H and SAN type 1-O.

Applicability of requirements

The requirement set applicability for SAN type 1-H and SAN type 1-O is need to be defined. And applicability of test configurations for multi-carrier operation is needed.

Proposal 5: To define requirement set applicability for SAN type 1-H and SAN type 1-O and applicability of test configurations for multi-carrier operation.

RF channels

Same with discussion on RF channels for conductive conformance testing[4], for the single carrier testing, , many tests for SAN type 1-H and SAN type 1-O can be performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the SAN.  Definition of RF channels B (bottom), M (middle) and T (top) in TS 38.141-2 can be reused for NTN.

Proposal 6: For the single carrier testing, many tests for SAN type 1-H and SAN type 1-O can be performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the SAN.  Definition of RF channels B (bottom), M (middle) and T (top) in TS 38.141-2 can be reused for NTN.

Test models

Test models defined for SAN type 1-H are also applicable for SAN type 1-O conformance testing.

Proposal 7: Test models defined for SAN type 1-H are also applicable for SAN type 1-O conformance testing.

- Conclusion

This contribution provides our general consideration on radiated conformance testing for SAN. The following proposals are concluded as follows:

Proposal 1: The measurement uncertainty for f ≤ 3 GHz for transmitter tests and receiver tests defined in 38.141-2 can be reused for SAN type 1-H and SAN type 1-O (n255, and n256).

Proposal 2: The method of manufacturer declarations for BS type 1-H, BS type 1-O radiated test requirements in TS 38.141-2 can be reused for manufacturer declarations for SAN type 1-H, SAN type 1-O radiated test requirements.

Proposal 3:  To define figures for the RIB interfaces for SAN transmitter and receiver.

Proposal 4: Test configurations for multi-carrier operation can be reused for SAN type 1-H and SAN type 1-O.

Proposal 5: To define requirement set applicability for SAN type 1-H and SAN type 1-O and applicability of test configurations for multi-carrier operation.

Proposal 6: For the single carrier testing, many tests for SAN type 1-H and SAN type 1-O can be performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the SAN.  Definition of RF channels B (bottom), M (middle) and T (top) in TS 38.141-2 can be reused for NTN.

Proposal 7: Test models defined for SAN type 1-H are also applicable for SAN type 1-O conformance testing.

- References

- R4-2207335, TP for TS 38.108: Genera(5.1) and Operating Band(5.2), ZTE Corporation, RAN4#102-e

- R4-2207332, TP on TS 38.101-5 for UE channel bandwidth and channel arrangement, Qualcomm Incorporated, RAN4#102-e

- R4-2207337, TP to TS 38.108: clause 4.3 requirement reference point, CATT, RAN4#102-e

- R4-2208252, General consideration on conductive conformance testing for SAN, CATT, RAN4#103-e