# 3GPP TSG-RAN WG4 Meeting # 103-e                                                        R4-2208692

# Electronic Meeting, May. 9-May. 20, 2022

# Source:	ZTE Corporation

# Title:	TP for TR 38.717-02-01: CA_n34A-n41A/C

# Agenda Item:	8.5.2

# Document for:	Approval

- Introduction

This contribution provides a text proposal on 2DL CA band combination of NR band n34+ NR Band n41 for TR38.717-02-01[1], in which CA_n34A-n41A and CA_n34A-n41C are included.

Since the frequency range of band n34 is larger than band 39 but smaller than band n40, and for  CA_n39A-n41A and  CA_n40A-n41A, where without simultaneous Rx/Tx is applied, which is also aligned with DC_n41A_34A.

- Reference

- TR38.717-02-01, Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) v0.8.0

- Text Proposal

----- Start of TP -----

# 6.x			CA_n34-n41

- 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n8+n34

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n34-n41 | n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx |  |  |  |  |  |  |  |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination of band n8+n34

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n34A-n41A | CA_n34A-n41A | n34 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30 ,40 ,50, 60, 70, 80, 90, 100 |  |
| CA_n34A-n41C | CA_n34A-n41A | n34 | 5, 10, 15 | 0 |
|  |  | n41 | CA_n41_BCS1 |  |

#### 6.x.1.3		Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n34-n41.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |

Based on above table, there is no harmonic issue for the band combination of CA_n34-n41.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n34 and n41.

# 6.x.1.4		∆TIB and ∆RIB values

For CA_n34-n41 it is proposed to use the same TIB,c and RIB values of DC_n41-34, which are given in the tables below.

# Table 6.x.1.4-1: ΔTIB,c

| NR CA Combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n34-n41 | n34 | 0.3 |
|  | n41 | 0.3 |

# Table 6.x.1.4-2: ΔRIB,c

| NR CA Combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n34-n41 | n34 | 0 |
|  | n41 | 0 |

- 6.x.1.5		REFSEN requirements

Since both NR band n41 and band n34 are TDD band, also as mentioned above, non-simultaneous Tx/Rx operation between NR band n41 and band n34 is supported, therefore there are no harmonic and intermodulation for this NR configuration.

- 6.x.2		Specific for 2 bands UL CA

      - 6.x.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n34A-n41A | 23 | +2/-3 |

- 6.x.2.2		UE co-existence studies

Due to band n34 and band n41 are all TDD bands, so there are no IMD issues for CA_n34-n41.

Table 6.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

- Table 6.x.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA Combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n34-n41 | - UTRA Band 1, 3, 8, 26, 28, 39, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

- 6.x.2.3		REFSENS requirements

No MSD issues for NR CA n34+n41 band combination.

----- End of TP -----