3GPP TSG-RAN WG4 Meeting #103-e	R4-2210226

Online Meeting, May 9 – 20, 2022

Agenda item:	9.22.2

Source: 	Qualcomm Incorporated

Title: 	Open performance requirements for RTT-based propagation delay compensation

Document for:	Discussion

# Introduction

In this paper we discuss the following topics

- Simulation results for RTT-based PDC using TRS

# Simulation results for RTT-based PDC using TRS

In RAN4#101-bis-e and RAN4#102-e, RAN4 agreed on simulation assumptions to evaluate UE Rx-Tx time difference performance in FR1 and FR2 using TRS as the DL reference signal [1].

Table 1 and Table 2 show UE Rx-Tx time difference accuracy in AWGN derived from link-level simulations, using the agreed simulation assumptions. For FR1, the results are consistent with previous results obtained with equivalent PRS configurations, with a single PRS comb per slot and no inter-slot repetitions. For FR2 the simulation results are still pending. Note that these results do not include any margin to account for UE Rx/Tx group delay calibration error.

RAN4 reached agreements on most of the group delay calibration margins to be added to measurement accuracy requirements for UE Rx-Tx measurements for Rel-16 NR positioning [2]. Note that the margins will be the dominant factor of the measurement accuracy in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Accuracy [90%] (Tc) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SINR = -3 dB |
| FR1 | 24 | 15 | 256 | 4 | 120.5 |
|  | 52 |  | 128 | 4 | 56.8 |
|  | 104 |  | 64 | 4 | 29.9 |
|  | 24 | 30 | 128 | 4 | 56.4 |
|  | 48 |  | 64 | 4 | 28.3 |
|  | 132 |  | 32 | 4 | 15.1 |

Table 1: Simulation results for FR1 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Accuracy [90%] (Tc) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SINR = -3 dB |
| FR2 | 32 | 120 | 32 | 4 |  |
|  | 64 |  | 16 | 4 |  |
|  | 128 |  | 8 | 4 |  |

Table 2: Simulation results for FR2 in AWGN.

Observation 1: Simulation results for UE Rx-Tx time difference using TRS are shown in Table 1 and Table 2. The results do not include any margin to account for UE Rx/Tx group delay calibration error.

# Conclusions

Observation 1: Simulation results for UE Rx-Tx time difference using TRS are shown in Table 1 and Table 2. The results do not include any margin to account for UE Rx/Tx group delay calibration error.

# References

[1] R4-2207020, WF on NR_IIOT_URLLC_enh_RRM requirements, RAN4#102-e.

[2] R4-2206821, WF on maintenance to R16 POS requirements, RAN4#102-e.