3GPP TSG-RAN WG4 Meeting # 104-e											R4-2214457

Electronic Meeting, Aug. 2022

Title: 	WF on FR2 multi-Rx chain DL reception

Agenda Item:	11.8.1, 11.8.2

Source: 	Sony

Document for:	Approval

# Work Plan

## Proposal: Revised work plan for UE RF:

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Discussion on system assumptions, UE assumptions | Agree on system assumptions |
| RAN4#104-Bis | Oct. 2022 | UE implementation assumptions and Requirement concept |  |
| RAN4#105 | Nov. 2022 | UE implementation assumptions and Requirement concept | Agree on UE assumptions |
| RAN4#106 | Feb. 2023 | Requirement concept and Simulation discussion | Agree on details pertaining to simulation effort |
| RAN4#106-Bis | Apr. 2023 | Simulation results discussion | Draft CR? |
| RAN4#107 | May 2023 | Requirement Parameter details | Draft CR |
| RAN4#108 | Aug. 2023 | Requirement Parameter details | Feature CR |

Agreement: the work plan above is agreed.

# UE RF

## System assumptions

### General deployment assumption for DL split between TRPs to enable 4L reception

Agreement:

- Proposal: To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

  - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

### Scope definition for UE RF requirements

Agreement:

- Proposal: UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

### DCI scheme when UE is configured for 2 active TCI states for UE RF requirements

Agreement:

- Proposal: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme isadopted as a baseline.

## UE assumptions

### UE architecture assumption

Agreement:

- FFS whether the concept of panel should not be explicitly used in core requirements and test configurations.

- FFS whether the single panel should be excluded.

## Test setups

### How to determine candidate AoA pairs for setting the UE RF requirement

Agreement:

- Further discuss on the on the candidate AoA pairs for setting the UE RF requirement

  - One Fixed AoA1 (e.g. Peak) + Full set AoA2.

  - Multiple AoA1 + Full set AoA2.

  - Fixed offset between the two AoAs, both probes swept simultaneously.

  - Full set AoA1 + Full set AoA2

Other solutions are not precluded. Companies are also encouraged to bring the analysis on how to quantify the Refsens value when receiving multiple signals.