3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211682

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.12.1

Source: 	CATT

Title: 	Discussion on RRM core requirements maintenance for UE Power Saving Enhancements

Document for:	Discussion

Introduction

In last meeting, there are still some open issues for RRM core requirements for UE power saving enhancements.

In this contribution, we provide our consideration of these open issues and give our proposals.

Discussion

In RAN4#103-e meeting, low mobility criteria evaluation for NR-DC is still open as listed in [1]:

- Issue 1-8: Clarifications for Low mobility criteria evaluation For NR-DC, the following proposals can be discussed in R4#104-e: - Proposal 1-1: For NR-DC, UE may also evaluate low mobility criteria in PSCell if network configures, and apply the evaluated low mobility state in the corresponding cell group. - Proposal 1-2: - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell. - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell - Proposal 1-3: keep the previous agreement that evaluate low mobility criteria on PCell.

In RAN2#118 meeting, the conclusion is low mobility criterion has been configured in NR PCell and MN will inform SN when low mobility criterion. It is also included in the LS [2] as below:

- MN informs SN when low mobility criterion has been configured in NR PCell. How to capture it could be further discussed in CR (assume impact to TS38.331 and TS37.340)

Therefore, we think it should keep the previous agreement that evaluate low mobility criteria on PCell.

Proposal 1: For NR-DC, to align with RAN2, low mobility criterion is configured in NR PCell.

In last meeting, the minimum requirement at transitions hasn’t been concluded.

- Issue 1-9: Introduce minimum requirement at transitions FFS: Define the minimum requirement at transitions between relaxed and non-relaxed RLM/BFD measurements for OOS evaluation.

For the RLM/BFD measurement, when UE is changed from non-relaxed mode to relaxed mode, UE shall use the evaluation period of non-relaxed mode at transitions period then enter relaxed mode. When UE is changed from relaxed mode to non-relaxed mode, UE is no longer fulfilled the relaxation condition and sent OOS. UE shall send OOS and use the evaluation period of non-relaxed mode at transitions period and go into normal evaluation.

Proposal 2: For the RLM/BFD measurement, when UE is changed from non-relaxed mode to relaxed mode or from relaxed mode to non-relaxed mode, UE shall use the evaluation period of non-relaxed mode at transitions period.

Conclusion

In this contribution, we provide our consideration of open issues for RRM core requirements for UE power saving enhancements and give our proposals.

Proposal 1: For NR-DC, to align with RAN2, low mobility criterion is configured in NR PCell.

Proposal 2: For the RLM/BFD measurement, when UE is changed from non-relaxed mode to relaxed mode or from relaxed mode to non-relaxed mode, UE shall use the evaluation period of non-relaxed mode at transitions period.

Reference

[1] R4-2210613, WF on RLM/BFD relaxation for UE Power Saving enhancements, MediaTek

[2] R2-2206675, Reply LS to RAN4 on RLM/BFD relaxation, RAN2