3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211813

Electronic Meeting, 15 – 26 August 2022

Agenda Item:	11.7.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on simulation assumptions for FR2-1 UL 256QAM MPR simulations

Document for:	Approval

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is the MRP simulations to study the required MPR for the target signal quality and emission requirements. This contribution provides proposals on simulation assumptions for FR2 UL 256QAM MPR simulations based on the assumptions used for FR1 UL 256QAM MPR simulations [2] and the existing FR2 requirements in TS 38.101-2 [3].

# 2.	Discussion

## 2.1	Simulator set-up

For the uplink 256QAM MPR study a simulator with the following impairments should be used [2]:

- Transceiver noise -38.5 dBc

- Modulator I/Q imbalance -33.7 dBc

- Modulator CIM3 -60 dBc

- Carrier suppression 25 dB

- Phase noise -35 dBc

The following signal quality and emission requirements should be used in the simulation [3]:

- Occupied bandwidth for FR2 (Table 6.5.1-1 in [3])

- General NR spectrum emission mask for FR2 (Table 6.5.2.1-1 in [3])

- NRACLR1 for FR2-1 (Table 6.5.2.3-1 in [3])

- General NR spurious emission limits for FR2 (Table 6.5.3-2 in [3])

- Maximum error vector magnitude (EVM) = 3.5% (same as UL 256QAM for FR1)

- General in-band emissions limit, assuming the EVM limit specified for each modulation (Tables 6.4.2.3.2-1, 6.4.2.3.3-1, 6.4.2.3.4-1, 6.4.2.3.6-1 in [3])

- PA operating point was set so that the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR (clause 6.2.2.0 in [3])

## 2.2	EVM partitioning

The EVM contributions of error sources should be according to Table 1 [2].

Table 1. EVM partitioning

| TX EVM source | EVM |  |
| --- | --- | --- |
|  | % | C/N [dBc] |
| PA | 1.85 | 34.7 |
| Transmitter | 1.19 | 38.5 |
| Phase noise | 1.78 | 35.0 |
| I/Q image | 2.06 | 33.7 |
| Total | 3.5 | 29.1 |

# 3.	Initial simulation results

In LTE and NR FR1, the MPR of 256QAM is entirely determined by the EVM. This can be expected also for FR2-1, as the EVM limit remains the same and the other requirements are either similar or less strict than in LTE and FR1. The worst case for EVM is when the I/Q image maximally overlaps the RB allocation. This occurs when the allocation is located at the channel center. Thus, we run a small simulation with maximum RB allocations placed at channel center. The results are presented in Table 2.

Table 2. Initial MPR simulation results for 256QAM

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| OFDM |  | 60 | 8.5 | 8.5 | 8.6 | N/A | 8.6 |
|  |  | 120 | 8.5 | 8.6 | 8.5 | 8.5 |  |
| DFT-S-OFDM |  | 60 | 6.4 | 6.4 | 6.4 | N/A | 6.5 |
|  |  | 120 | 6.5 | 6.4 | 6.4 | 6.5 |  |

# 4.	Conclusion

This contribution has provided proposals on simulation assumptions for FR2 UL 256QAM MPR simulations based on the assumptions used for FR1 UL 256QAM MPR simulations [2] and the existing requirements in TS 38.101-2 [3]. They are provided above in section 2.

Proposal 1: Approve the simulation assumptions provided above in section 2 for FR2 UL 256QAM MPR simulations.

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-166954, “WF on MPR\AMPR simulation assumptions for UL 256QAM”, Huawei, Hisilicon, Qualcomm, CMCC, Nokia, Skyworks, Ericsson.

[3]	3GPP TS 38.101-2 v17.6.0: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.