3GPP TSG-RAN WG4 Meeting # 104-e                                  R4-2211915

Electronic Meeting, August 15 – August 26, 2022

Source: 	Apple

Title: 	Beam correspondence requirement for RRC_INACTIVE and initial access

Agenda Item:	11.7.3

Document for:	Discussion

# Introduction

An Rel-18 WI on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved in [1]. The following scopes are included for UE beam correspondence requirement.

Beam correspondence requirements for RRC_INACTIVE and initial access

- Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

- For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

  - Requirements for other transmission within RRC_INACTIVE state are not precluded.

- For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least)

- Study the potential impact on testability aspects (i.e., test time).

This contribution will present our consideration for beam correspondence for initial access and RRC_INACTIVE state.

# Discussion

## Beam correspondence requirement

Beam correspondence is the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping. In different RRC state, the UE may utilize different reference signals to determine the suitable beam.

In Rel-15, RAN4 introduced beam corresponding requirement assuming both SSB and CSI-RS are configured. How to determine the suitable beam is UE implementation [2]. In Rel-16, RAN4 further defined SSB-based beam corresponding requirement and CSI-RS based beam correspondence requirement as well as the related side conditions [2].

2.1.1	Initial access

A UE needs to do DL synchronization and SSB-based SS-RSRP measurement before accessing the network. QCL configuration for PRACH cannot be configured in this stage and SSB is the only available reference signal for UE to determine a suitable Tx beam for the follow-up PRACH transmission. The UE will determine the suitable uplink beam corresponding to the direction of the selected SSB according to SS-RSRP measurement. Further in [3], it summarized the situation for beam correspondence work in Rel-16. And it indicates that the only SSB based beam correspondence requirement is applicable for initial access.

Requirements for SSB-based and CSI-RS based BC were developed. This allows network to efficiently deploy different beam widths for SSB and CSI-RS. This enables e.g. to deploy wider SSB based beams for idle mode UEs and narrower CSI-RS based beams for RRC connected mode UEs.

Observation 1: The only SSB-based beam correspondence requirement is applicable for initial access.

2.1.2	SDT over RACH or Type 1 CG resources

In Rel-17 Small Data Transmission (SDT) feature is introduced, which allows UE to conduct small amount of data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT procedure is initiated with either a transmission over RACH (configured via system information) or over Type 1 CG resources (configured via dedicated signalling in RRCRelease).

- According to SDT specification design, RA SDT can be supported in both 2-step RACH or 4-step RACH. The uplink small data can be sent in MSGA of 2-step RACH or msg3 of 4-step RACH. As such, the essence of RA SDT is a RACH procedure with different data transmitted in MSG 1 or MSGA. In the case of multiple beam operation, the existing procedure of determining UL TX beam for RACH based on SSB measurements can be reused for Random Access SDT.

- SDT procedure over CG resources can only be initiated with valid UL timing alignment. The UL timing alignment is maintained by the UE based on a SDT-specific timing alignment timer configured by the network and, for initial CG-SDT transmission, also by DL RSRP of configured number of highest ranked SSBs which are above a configured RSRP threshold. It is seen that a UE will determine the suitable Tx beam for CG-SDT over PUSCH also depends on SSB measurement.

Observation 2: For Random Access SDT and Configured Grant SDT in RRC_INACTIVE, UEs need to measure SSBs to determine its suitable TX beam for transmitting data over RACH or PUSCH, both of which have a resource mapping to SSB beam index and SS-RSRP measurement.

Observation 3: The current only SSB-based requirement is also applicable for RA-SDT and CG-SDT.

Based on the discussion and observations above, we have the following proposal 1.

Proposal 1: It is proposed that the current SSB based beam correspondence requirement are reused for Initial access, Random Access SDT and Configured Grant SDT.

## Testability aspects

2.2.1	How many tests for SSB based BC

Among all the scenarios in the WID scope (initial access, RA-SDT and CG-SDT), our understanding is that it is enough to only test the beam correspondence requirement in initial access. The reason is that,

- As discussed in section 2.1, from a procedure point of view, the selection of suitable Tx beam in the UL is quite similar. E.g. only based on SSB measurement.

- Initial access will be most fundamental one where UE don’t have any history information for the cell to be connected.

- It is already agreed in RRM session that Random Access SDT and Configured Grant SDT assume UE have beam correspondence capability in initial access.

Proposal 2: To save test effort, beam correspondence requirement is only tested for initial access.

2.2.2	Testability aspects

According to the current specification, beam correspondence requirement for a UE is fulfilled if the UE meets the minimum peak EIRP requirement and the spherical coverage requirement for the corresponding power class. All these requirement tests should be done at UE maximum output power.

Currently the beam correspondence test is designed for connected state with a closed loop power control. The test results will be recorded after the UE reaches its maximum output power during the test. While for initial access, there is not a condition to form the closed loop power control. New test method needs to be developed. The analysis in [4] gives some good thinking on using open loop power control test, which can be a starting point for further investigation.

Proposal 3: It is proposed to further discuss the following points in Oct. meeting

- How to achieve the maximum output power condition in initial access.

- How to balance testing time and test performance, e.g. whether it is feasible to use sparse grid.

- New test procedures and test settings

# Summary

This contribution presented our consideration on beam correspondence requirement for initial access, Random Access SDT and Configure Grant SDT. The testability aspects are also briefly touched. The following observations and proposals are concluded.

Observation 1: The only SSB-based beam correspondence requirement is applicable for initial access.

Observation 2: For Random Access SDT and Configured Grant SDT in RRC_INACTIVE, UEs need to measure SSBs to determine its suitable TX beam for transmitting data over RACH or PUSCH, both of which have a resource mapping to SSB beam index and SS-RSRP measurement.

Observation 3: The current only SSB-based requirement is also applicable for RA-SDT and CG-SDT.

Proposal 1: It is proposed that the current SSB based beam correspondence requirement are reused for Initial access, Random Access SDT and Configured Grant SDT.

Proposal 2: To save test effort, beam correspondence requirement is only tested for initial access.

Proposal 3: It is proposed to further discuss the following points in Oct. meeting

- How to achieve the maximum output power condition in initial access.

- How to balance testing time and test performance, e.g. whether it is feasible to use sparse grid.

- New test procedures and test settings

# References

[1] RP-220967, NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia

[2] RP-201984, Summary for WI NR RF requirement enhancements for frequency range 2, Nokia, Nokia Shanghai Bell

[3] TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone.

[4]	R4-1914121, Test configuration for beam correspondence during initial access, Ericsson