3GPP TSG-WG RAN4 Meeting #104-e	R4-2212066

# Online, 15th – 26th Auguest, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 				30 MHz UE in 40MHz network

Agenda Item: 	4.1.1

Document for:	Approval

# Introduction

The issue to deploy 40 MHz system bandwidth with 30 MHz UE was reported in RAN4#103e [1]. Five potential solutions were summarized in WF [2] and the agreements below were made.

- The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used.

- Tentative agreement for further check: UE dedicated carrier edge is allowed to extend over the duplexer edge, e.g. 788MHz for n28 30MHz CBW. It’s noted that maximum transmission bandwidth configuration doesn’t extend over the duplexer edge in such case.

The following open issues are left for further discussion.

- issue 3-2-1: whether to allow unalignment between channel raster and the maximum transmission bandwidth configuration in SIB1, i.e. resource grid in SIB1

  - Option 1: yes, some exception is allowed

  - Option 2: no

- issue 3-2-2: whether to allow unalignment between channel raster and UE dedicated maximum transmission bandwidth configuration i.e. UE dedicated resource grid

  - Option 1: yes, some exception is allowed

  - Option 2: no

  - Option 3: yes, UE dedicated channel does not need to be at 100 kHz raster for any band. It should not be an exception for n28.

# Discussion

According to the first WF agreement above, Solution 1 in [2] is not a viable option anymore as the guard band is outside of the band edge.

We understand Solution 2 is NBC, as the legacy requirement is changed to guarantee unwanted emissions with smaller guard band than the is already defined.

The intention of the WF agreement and tentative agreement is that the guard band can be outside of a configured channel edge as far as it is within the system bandwidth. This will not cause any coexistence issue with adjacent service regarding unwanted emissions and receiver blocking. Therefore, Solution 3 is not aligned with this intention, even it may work for 30 MHz UE that only uses the PRBs on the lower frequency side. The last PRB on the higher frequency side violates the minimum guard band requirement.

Solution 4 needs to be further checked if this is backward compatible and is supported by legacy UEs regarding the alignment of 100 kHz channel raster and UE specific CBW configured via dedicated signalling.

Solution 1a can avoid the issues described for Solution 3, as the 30 MHz channels are configured within 40 MHz system bandwidth. The alignment of the 100 kHz channel raster needs to be further checked if this is backward compatible or not.

The issues discussed regarding channel raster and CBW configuration are the same as the ones discussed in the study item on “Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths.” A common consensus shall be made to agree a solution for the issue to deploy 40 MHz system with 30 MHz UE.

Observation 1: Open issues regarding the channel raster alignment and CBW configuration (SIB1 and UE specific) are the same as discussed for SI on irregular channel bandwidth.

Once these open issues are clarified and RAN4 reaches at the common understanding of legacy UE behaviours, the agreement shall be clarified in the Rel-15 specifications to avoid possible future confusions and misunderstanding.

Proposal 1: The legacy UE behaviour regarding the channel raster alignment and valid CBW configuration shall be clarified in Rel-15 UE specifications.

Furthermore, the solution to be adopted shall not be specific to a deployment of 40 MHz channel in band n28. It shall be universal to other bands and bandwidths. The channel bandwidth nesting may be used also in other bands and bandwidths, where similar issues may arise later on unless the solution is universal.

Proposal 2: A solution regarding 30 MHz UE in 40 MHz system for n28 shall be universal to other bands and channel bandwidths.

# Conclusion

Observation 1: Open issues regarding the channel raster alignment and CBW configuration (SIB1 and UE specific) are the same as discussed for SI on irregular channel bandwidth.

Proposal 1: The legacy UE behaviour regarding the channel raster alignment and valid CBW configuration shall be clarified in Rel-15 UE specifications.

Proposal 2: A solution regarding 30 MHz UE in 40 MHz system for n28 shall be universal to other bands and channel bandwidths.

# Reference

[1] R4-2208401 Discussion on 30MHz reconfiguration failure when accessing 40MHz network of n28, CMCC

[2] R4-2210540 WF on 30MHz reconfiguration failure when accessing 40MHz network of n28, RAN4#103e