3GPP TSG-RAN WG4 Meeting #104-e	R4-2212091

Electronic meeting, 15th – 26th August 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 					PC1 A-MPR for n71 and n85

Agenda Item: 			10.15.2

Document for:	Approval

# 1	Introduction

This contribution presents PC1 A-MPR simulation results for bands n71 and n85 based on the way forward from RAN4#103-e [1].

# 2	Discussion

## 2.1	Simulation assumptions

The simulation assumptions are based on the WF [1]:

o	PC1 PA linearity is assumed (37dB ACLR for DFT-s-OFDM QPSK 20MHZ 100RB0 waveform with 1dB MPR)

o	Normal TRX impairments are assumed: 28dB carrier and image leakage, 60dB CIM3 and 70dB CIM5

o	BB WOLA windowing aspects are covered

o	At least QPSK DFT-s-OFDM full and edge allocations are evaluated for 5 and 20MHz CBW

o	Other CBW and allocations are not precluded

Other assumptions:

- Channel bandwidths and subcarrier spacings according to TS 38.101-1, Table 5.3.5-1:

| NR band | SCS [kHz] | UE channel bandwidth [MHz] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
| n71 | 15 | yes | yes | yes | yes |
|  | 30 | - | yes | yes | yes |
| n85 | 15 | yes | yes | yes | - |
|  | 30 | - | yes | yes | - |

- Both OFDM and DFT-S-OFDM waveforms

- All modulations: pi/2-BPSK, QPSK, 16-QAM, 64-QAM, 256-QAM

- WOLA processing (window slope is 33 % of the short CP duration)

- SEM for n71 (NS_35):

Table 6.5.2.3.1-1: Additional requirements for "NS_35"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |  |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

- SEM for n85 (NS_06):

Table 6.5.2.3.4-1: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

- General requirements as specified in TS 38.101-1

# 3	Simulation results

We simulated all valid channel bandwidth / SCS combinations for n71 and 85, both waveforms, all modulations, all contiguous RB allocations. Results of the two bands were quite similar due to the similar SEMs. Examples of the results are shown in Figures 1-5.

Figure 1. n71 back-off for 5 MHz, 15 kHz

Figure 2. n71 back-off for 10 MHz, 15 kHz

Figure 3. n71 back-off for 20 MHz, 15 kHz

Figure 4. n85 back-off for 5 MHz, 15 kHz

Figure 5. n85 back-off for 15 MHz, 15 kHz

In both n71 and n85, the allocations needing A-MPR are confined to narrow allocations close to either channel edge. Figures 6-7 depict two different cases. By coincident, both cases require a similar back-off.

Figure 6. Example spectrum: 10 MHz, OFDM, QPSK, 15 kHz, 1RB@0,
gated by IMD5 hitting the spurious emission limit

Figure 7. Example spectrum: 10 MHz, DFT-S-OFDM, QPSK, 15 kHz, 1RB@0,
gated by regrowth hitting the innermost SEM segment with 30 kHz MBW

# 4	Proposed A-MPR

Proposal 1: Specify the A-MPR for both n71 and n85 as follows:

If RB allocation is within Region A, apply the A-MPR given in Table 1. Otherwise, apply the MPR.

Table 1. PC1 A-MPR for n71 and n85

| Waveform | Modulation | Region A |
| --- | --- | --- |
| DFT-S-OFDM | pi/2-BPSK | 7.5 |
|  | QPSK | 7.5 |
|  | 16-QAM | 7.5 |
|  | 64-QAM | 7.5 |
|  | 256-QAM | 7.5 |
| OFDM | QPSK | 7.5 |
|  | 16-QAM | 7.5 |
|  | 64-QAM | 7.5 |
|  | 256-QAM | 7.5 |

An RB allocation is within Region A if
( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )
or
( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

Above, RBstart and LCRB are the lowest RB index and size of the RB allocation, NRB is the total number of RBs in the channel, and CBW is the channel bandwidth.  The colors in the formula are used below when referring to subexpressions. Region A is illustrated in Figure 8.

The first condition covers narrow allocations at channel edge with spectral regrowth hitting the SEM (the vertical / slanted subregions in Figure 8). The second condition covers cases with IMD5 (of the allocation and its image, superpositioned with CIM5) hitting the spurious limit (the horizontal subregions in Figure 8). The expression
| RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS gives the approximate distance of allocation center from channel center (given that LCRB=1). When multiplied with 5, we obtain the distance of the IMD5 component center from the channel center. The spurious region starts at the distance 1.5 ∙ CBW + 5 MHz from the channel center.

The allocation ratio limit of 0.20 is required, e.g., in Figure 1 (DFT-S-OFDM) where the total RB count is  NRB = 25 and the greatest allocation size requiring A-MPR is LCRB = 5.

Figure 8. The proposed Region A

# 4	Conclusions

We presented PC1 A-MPR simulation results for n71 and n85, and proposed an A-MPR definition for both:

Proposal 1: If RB allocation is within Region A, apply the A-MPR given in Table 1. Otherwise, apply the MPR.

Table 1. PC1 A-MPR for n71 and n85

| Waveform | Modulation | Region A |
| --- | --- | --- |
| DFT-S-OFDM | pi/2-BPSK | 7.5 |
|  | QPSK | 7.5 |
|  | 16-QAM | 7.5 |
|  | 64-QAM | 7.5 |
|  | 256-QAM | 7.5 |
| OFDM | QPSK | 7.5 |
|  | 16-QAM | 7.5 |
|  | 64-QAM | 7.5 |
|  | 256-QAM | 7.5 |

An RB allocation is within Region A if
( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )
or
( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

# 5	References

[1] R4-2210569 WF on A-MPR for bands n71 and n85, RAN4#103-e

# 6	Appendix: A-MPR triangle plots