3GPP TSG-RAN4 Meeting #104-e	R4-2212092

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	11.6.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Essential considerations of lower MSD signaling

Document for:	Approval

# Introduction

Further RF requirements enhancement for NR frequency range 1 (FR1) Work Item description [1] defines the following objective for lower MSD investigation:

Investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations [RAN4] - Select a limited set of band combinations (2-4 combinations) to cover all types of MSD (harmonic, harmonic mixing, IMD and cross band isolation) - Study how the MSD performance can be improved for the example band combinations - Study of MSD improvement with different MSD sources (harmonics, IMD2/3/4/5, cross band isolation and harmonic mixing) - Study the feasibility of and options for allowing a UE to signal improved lower MSD performance capability for combinations where MSD is allowed - Aim to conclude the study phase by RAN#99, and further discuss in RAN#99 how to handle the objective based on the study progress.

In this contribution we will present Nokia’s point of view concerning lower MSD performance capability for band combinations (BCs) where MSD is allowed.

# Discussion

## Signaling and spec structure of MSD

### Per MSD type and order per BC vs per BC

There were discussions on whether lower MSD capability is reported per BC or per MSD type and order per BC (i.e., H2, IM2, IMD3, etc) in [2, 3].

Table 1: MSD allowed for DL_CA_n2-n5-n77 with CA_n2-n77configuration

Per BC

If lower MSD capability, e.g., MSD = 0 dB, is reported per BC (or per pair of UL bands per BC), it means a UE reporting it shall perform MSD = 0 dB for all the MSD in Table 1.

Per MSD type and order per BC

This approach provides UE vendors/chipset vendors with more flexibility in terms of UE design. As observed in our companion paper of [4], there are several ways to improve MSD due to the same MSD type (and order). There is a RF component that improves a specific MSD type and/or order while there is a RF component that improves several MSD types and/or order simultaneously. For example, in order to improve MSD due to 2nd harmonic, a UE vendor may use a PA with better H2 suppression. In this case, it may improve MSD due to only the 2nd harmonic, but it may not improve MSD due to other MSD types and/or orders. And if lower MSD capability per MSD type and order per BC is selected, UE can report lower MSD capability specific to a certain MSD type and/or order while lower MSD capability per MSD type and/or order per BC would require more signaling overhead than per BC.

Observation 1: Per BC makes signaling simpler than per MSD type and/or order per BC while the former may not allow UE to report lower MSD capability at all if the UE’s MSD performance for one or some of MSD type and order is better than the others.

Table 2 shows all the allowed exceptions when a UE is configured with DL_CA_n2-n5-n77 with UL_CA_n5-n77.

Table 2: MSD allowed for DL_CA_n2-n5-n77 with CA_n5-n77configuration

Comparing Table 1 to Table 2, colored rows are included in both tables. In case lower MSD capability is per MSD type and/or order per BC, the information is quite redundant if a UE supports both (DL, UL) = (n2-n5-n77, n2-n77) and (n2-n5-n77, n5-n77). The redundancy comes from the fact that both BCs include (DL, UL) = (n2-n77, n77) and (n5-n77, n77) as fallback. In our understanding, a minimum BC unit to cause harmonic mixing and cross band isolation is 1UL/2DL and all the higher order BCs including fallbacks in a minimum BC unit level inherit the same MSD. For example, (DL, UL) = (n2-n5-n77, n2-n77) includes (n2-n5-n77, n77), (n2-n77, n2-n77) and (n2-n77, n77) as fallback including n77 UL. All the three BCs in blue including (n2-n77, n77) will face MSD that (n2-n77, n77) has. Hence, (n2-n77, n77) is the fallback in a minimum BC unit. It means that if the UE reports following lower MSD capability per MSD type per BC, the higher order BCs do not need to report them.

- Harmonic mixing: (DL, UL) = (n2-n77, n77)

- Harmonic mixing: (DL, UL) = (n5-n77, n77)

- Cross band isolation: (DL, UL) = (n2-n77, n77)

In summary, if lower MSD capability is reported per minimum BC unit defined in Table 3, signaling overhead specifically, for lower MSD capability per MSD type and/or order would be mitigated. Note that if RAN2 reflects this aspect or not depends on RAN2 decision if lower MSD capability is specified.

Observation 2: Lower MSD capability for all the BCs supported by a UE may not be necessary. It would be sufficient to report the capabilities for only fallback BCs categorized as minimum BC unit that is the smallest component of each MSD type. NW can assume that all the supported higher order BCs inherit the reported MSD performance per fallback BCs in minimum BC unit levels.

Observation 3: Minimum BC unit per MSD type is summarized in Table 3.

Table 3: Minimum BC unit to report MSD

| MSD Type | Minimum BC unit |  |  |
| --- | --- | --- | --- |
|  | 1UL/2DL | 2UL/2DL | 2UL/3DL |
| UL Harmonic | X |  |  |
| Harmonic mixing | X |  |  |
| Cross band | X |  |  |
| IMD |  | X | X1 |
| NOTE 1: Only MSD impacting on the DL whose UL is not configured with is reported. |  |  |  |

### Other aspects to be considered

Victim bands or frequency component of the order:

In case lower MSD is reported per MSD type and order, signaling may need clarification on affected victim DL bands or frequency component of the order. As can be seen in Table 4 where MSD due to IMD for CA_n28-n74 is captured, there are two IMD4 per pair of UL bands per BC. Hence, it is not clear which MSD due to IMD4 is improved even if lower MSD capability is reported per MSD type and order per BC.

Table 4: MSD due to IMD for dual UL CA_n28-n74

Observation 4: There is a case that the same BC has two different MSD due to the same MSD type and order, e.g., dual UL CA_n28-n74 has two MSD due to IMD4 where each MSD has a different victim band, i.e., n28 DL or n74 DL. In this case, per MSD type and order per BC cannot differentiate them without additional identification, e.g., victim DL band or frequency component of the order.

Power Class (PC):

If a single low MSD capability is applicable to both PC2 and PC3 or not was discussed in [2]. MSD minimum requirements for different power classes can be significantly different depending on MSD type and the order. For instance, MSD due to IMD4 for PC3 is 8.3 dB while MSD for PC2 is 18.6 dB for dual UL CA_n5-n77 and the difference more than 10 dB.

Observation 5: MSD minimum requirements for the same MSD type and/or order can be very different from PC to PC, e.g., in some cases, the difference is more than 10 dB, e.g., IMD4 for dual UL CA_n5-n77. The impact of different PCs on MSD and signaling need to be studied further.

Multiple MSD for the same MSD type and order:

As shown in Table 1, there are two MSD values due to 2nd harmonic of n2 UL on n77 DL according to two different UL/DL CBW conditions for direct-hit, where an MSD value of 23.9 dB due to small aggressor UL CBW and victim CBW is larger than that of 13.8 dB due to larger aggressor UL CBW and victim CBW. In addition, there is an MSD for near-miss on top of two MSD for direct-hit. Thus far, the maximum MSD value for near-miss across all the BC is 1.9 dB.

Observation 6: There are two different MSD values for the same MSD type and order with different aggressor CBW and victim CBW pairs. The MSD difference is more than 10 dB in some cases.

Observation 7: MSD for near-miss is defined for UL harmonic MSD type on top of MSD for direct-hit. The maximum MSD for near-miss is at most 1.9 dB across all the BC in 38.101-1/-3.

## Lower MSD capability and granularity

Lower MSD capability

There was a proposal to define so-called delta between a minimum requirement and a requirement that lower MSD capable UE can meet. It means that if a minimum requirement is 30 dB and reported delta is 10 dB, then, UE’s MSD is lower than 20 dB (30 - 10). Another way would be UE directly indicates an improved MSD value (it’s called “direct approach” in this contribution for convenience).  It means if a UE reports 10 dB, the UE’s MSD is at least lower than 10 dB. It would worth discussing which way is better.

Observation 8: At least there could two approaches to report a lower MSD capability. One is report “delta” from minimum requirement (delta approach). The other is directly report improved MSD value (direct approach).

Provided that the same MSD type and order may have different minimum requirements depending on PC and/or UL/DL pair conditions etc., delta approach may have an advantage over direct approach. As can be seen in Observation 5, there may be a case that minimum requirement for PC2 is more stringent than that for PC3 by 10 dB. If a minimum requirement for PC2 is 20 dB and a UE reports a lower MSD capability of 10 dB for PC2 with direct approach, it’s not clear MSD performance for PC3. On the other hand, if delta approach is used and if the spec has a discipline that delta equally applies to both MSD for all the power classes for at least the same MSD type and order per BC, network can consider that MSD for PC2 is lower than that for PC3 by 10 dB which means MSD for PC3 is 0 dB for this example.

Observation 9: If RAN4 has a discipline that the same amount of delta per MSD type and order per BC is applicable regardless of power classes, delta approach would have advantage over direct approach.

Granularity of a lower MSD capability

For the sake of convenience, we call the proposed delta approach in [2] “fixed” delta approach in this paper. There was, however, a raised concern on the fixed delta approach in [3] that “Furthermore, if the “low” MSD requirement is defined as 5 dB, wouldn’t it unfair for UEs with 5.5dB MSD to be excluded from using the combination?”. To make the discussion more concrete, we’ll take MSD for PC2 CA_n66-n77 as an example.

Table 5: MSD test points due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| PC2 CA_n66-n77 | n66 | 1730 | 5 | 25 | 1730 | 34.33 | FDD | IMD2 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 11.27 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

MSD due to IMD2, i.e., 34.33 dB

Assume that a UE is allowed to report n * delta, where n is non-negative integer and n = 3 here. MSD range could be defined as shown in Table 6 and Figure 1.

Table 6: MSD range with delta approach, where n=3 (2 bits)

| n | MSD range |  |
| --- | --- | --- |
|  | Lower limit | Higher limit |
|  | Minimum requirement – (n+1) * delta | Minimum requirement – n * delta |
| 1 | 14.33 dB | 24.33 dB |
| 2 | 4.33 dB | 14.33 dB |
| 3 | 0 dB | 4.33 dB |
| 0 | No MSD |  |

Figure 1: Fixed delta with 2 bits and uneven granularity problem

One of the issues with this fixed delta approach would create a situation that even if a UE can meet MSD of 5 dB, the UE can only report 2*delta of 20 dB in an example shown in Figure 1. Thus, from a network perspective, the performance looks MSD of 14.33 dB and it is far beyond from the real performance of 5 dB since the difference is 9.33 dB. The fixed delta approach brings uneven granularity and the impact on unfairness depends on the relation between fixed delta value, a minimum requirement and the number of bits allowed for lower MSD capability.

MSD due to IMD5, i.e., 11.27 dB

The other issue would be that if a UE’s MSD due to IMD5 for e.g., PC2 CA_n66-n77 is 2 dB, the UE cannot indicate the delta at all if the granularity of the delta is 10 dB since the MSD is larger than 1.27(11.27 - 10) dB as shown in Figure 2. NW may consider the MSD is 11.27 dB and it’s worse than actual MSD by 9.27 (11.27 - 2) dB.

Figure 2: An issue that granularity of delta becomes more rough

The issue can be easily mitigated by increasing the granularity, for example, by decreasing delta from 10 dB to 5 dB or even smaller values. However, considering that one UE supports multiple BCs and each of them may have multiple MSD values due to different MSD types and orders such as different IMD orders like IMD2, 3 or 5 etc., finer resolution increases signalling overhead as the price for paying for it,

Observation 10: Reporting a fixed amount of improved MSD value(s) as reference to the minimum requirement value per BC (fixed delta approach) may generate uneven granularity within an MSD as well as MSD across different MSD type and order for the same BC.

In addition, our MSD analysis in [4] observed that larger MSD improvement becomes, i.e., as MSD approaches to MSD = 0 dB, even higher RF components performance improvement it requires as shown in Figure 3 which is an extract of Figure 7 from [4]. It’s noted that Figure 3 is an example of harmonic mixing.

Figure 3: Figure 5 [4]: MSD relation b/w PCB isolation and Mixer spur rejection for 3rd harmonic mixing

Hence, if a fixed delta approach is adopted, efforts to improve MSD close to zero may be underappreciated.

Observation 11: Larger MSD improvement becomes, even higher RF components performance improvement it requires [4] and if a fixed delta approach is adopted, efforts to improve MSD close to zero may be underappreciated.

Given that NW can know minimum requirement values and the number of available bits is limited, e.g., two bits (00, 01, 10, 11), perhaps, it may be a way to divide the minimum requirement values into four MSD regions in an equidistant manner. More specifically, 34.33 for IMD2 is divided into four regions and it generates 0-8.6, 8.6-17.2, 17.2-25.7 and 25.7-34.33 dB as shown in Figure 4. Note that if MSD = 0 dB, 11 is reported.

Figure 4: comparison b/w fixed delta and delta in an equidistant way for IMD2

In Figure 4, a benefit of the delta in an equidistant way is that NW can consider UE’s MSD as 8.6 dB (actual value is 5 dB) while with the fixed delta approach, NW considers that the UE’s MSD is 14.33 dB. Figure 5 shows a comparison in terms of IMD5. Since the minimum requirement for IMD5 is smaller than that for IMD2, delta in an equidistant way can use even finer granularity than fixed delta so that the difference between actual MSD and reported MSD can become smaller.

Figure 5: comparison b/w fixed delta and delta in an equidistant way for IMD5

Observation 12: If delta is defined as the minimum requirement/2n (equidistant delta within each of the minimum requirements) where n is the number of bits, the maximum difference between actual MSD and reported MSD can become smaller than that by fixed delta.

Or perhaps, considering that more efforts, i.e., cost for RF components performance improvement, in order to achieve MSD = 0 dB or at around it, are needed as shown in Figure 3, it may be more suitable to place finer granularity into a region that MSD improvement is larger (e.g., 2 x delta, 3 x delta) and coarser granularity into a region that MSD improvement is smaller (e.g., 1 x delta). More specifically, 34.33 dB for IMD2 is divided into four regions by two bits in a following way.

1st delta: 34.33 dB / 21 = 17.2 dB  MSD range is 8.6-17.2 dB (01)

2nd delta: 1st delta + 34.33 dB / 22 = 17.2 + 8.6 = 25.8 dB   MSD range is 4.3-8.6 dB (10)

3rd delta: 2nd delta + 34.33 dB / 23 = 25.8 + 4.3 = 30.1 dB  MSD range is 0-4.3 dB (11)

MSD=0 dB (00) are as shown in Figure 6.

Figure 6: comparison b/w fixed delta and finer delta as MSD approaches to 0 dB for IMD2

Figure 7: comparison b/w fixed delta and finer delta as MSD approaches to 0 dB for IMD5

As can be seen in Figure 6 and Figure 7, “finer delta as MSD approaches to 0 dB” can provide more finer resolutions as MSD approaches to 0 dB. The drawback is the resolutions become coarser if MSD is large.

Observation 13: If delta is defined as delta = minimum requirement/M where 1 < M <= M=2n, n is the number of bits, the maximum difference between actual MSD and reported MSD can become even smaller than that by equidistant delta when MSD is small while the difference can become even larger than that by fixed delta when MSD is large.

Table 7: Comparison table for the three approaches to indicate MSD ranges.

|  | Approaches |  |  |
| --- | --- | --- | --- |
|  | Approach 1 (Fixed delta) | Approach 2 (Equidistant delta according to minimum requirement) | Approach 3 (Finer delta as MSD approaches to 0 dB) |
| Granularity | Bad and Worst Suitability changes according to minimum requirement. May not be able to indicate MSD improvement at all when MSD is close to 0 dB. If minimum requirement is similar or smaller than the delta, the granularity can be the worst. | Good The difference b/w actual MSD and reported MSD can be minimized in a more fair fashion than the other two approaches. | Best and Worst Granularity close to MSD=0 dB is even finer than approach 2 and Granularity close to minimum requirement can be even coarse than approach 1 when minimum requirement is large. |
| NW impact | NW needs to store minimum requirements and calculate MSD range from the reported bits. |  |  |
| Others | Simpler than the others | Well balanced | Can provide finer resolution when actual MSD is close to 0 dB where more efforts per MSD improvement are needed |

Observation 14: Provided that whichever approach is selected, NW needs to store MSD information per BC, MSD per MSD source per BC or MSD per order per MSD source per BC, fixed delta approach cannot have advantage compared to the other approaches.

# Conclusion

We obtained following 13 observations and propose to consider them in the future meetings.

Proposal: Further consider following observations in the future meetings.

Observation 1: Per BC makes signaling simpler than per MSD type and/or order per BC while the former may not allow UE to report lower MSD capability at all if the UE’s MSD performance for one or some of MSD type and order is better than the others.

Observation 2: Lower MSD capability for all the BCs supported by a UE may not be necessary. It would be sufficient to report the capabilities for only fallback BCs categorized as minimum BC unit that is the smallest component of each MSD type. NW can assume that all the supported higher order BCs inherit the reported MSD performance per fallback BCs in minimum BC unit levels.

Observation 3: Minimum BC unit per MSD type is summarized in Table 3.

Table 3: Minimum BC unit to report MSD

| MSD Type | Minimum BC unit |  |  |
| --- | --- | --- | --- |
|  | 1UL/2DL | 2UL/2DL | 2UL/3DL |
| UL Harmonic | X |  |  |
| Harmonic mixing | X |  |  |
| Cross band | X |  |  |
| IMD |  | X | X1 |
| NOTE 1: Only MSD impacting on the DL whose UL is not configured with is reported. |  |  |  |

Observation 4: There is a case that the same BC has two different MSD due to the same MSD type and order, e.g., dual UL CA_n28-n74 has two MSD due to IMD4 where each MSD has a different victim band, i.e., n28 DL or n74 DL. In this case, per MSD type and order per BC cannot differentiate them without additional identification, e.g., victim DL band or frequency component of the order.

Observation 5: MSD minimum requirements for the same MSD type and/or order can be very different from PC to PC, e.g., in some cases, the difference is more than 10 dB, e.g., IMD4 for dual UL CA_n5-n77. The impact of different PCs on MSD and signaling need to be studied further.

Observation 6: There are two different MSD values for the same MSD type and order with different aggressor CBW and victim CBW pairs. The MSD difference is more than 10 dB in some cases.

Observation 7: MSD for near-miss is defined for UL harmonic MSD type on top of MSD for direct-hit. The maximum MSD for near-miss is at most 1.9 dB across all the BC in 38.101-1/-3.

Observation 8: At least there could two approaches to report a lower MSD capability. One is report “delta” from minimum requirement (delta approach). The other is directly report improved MSD value (direct approach).

Observation 9: If RAN4 has a discipline that the same amount of delta per MSD type and order per BC is applicable regardless of power classes, delta approach would have advantage over direct approach.

Observation 10: Reporting a fixed amount of improved MSD value(s) as reference to the minimum requirement value per BC (fixed delta approach) may generate uneven granularity within an MSD as well as MSD across different MSD type and order for the same BC.

Observation 11: Larger MSD improvement becomes, even higher RF components performance improvement it requires [R4-2212093] and if a fixed delta approach is adopted, efforts to improve MSD close to zero may be underappreciated.

Observation 12: If delta is defined as the minimum requirement/2n (equidistant delta within each of the minimum requirements) where n is the number of bits, the maximum difference between actual MSD and reported MSD can become smaller than that by fixed delta.

Observation 13: If delta is defined as delta = minimum requirement/M where 1 < M <= M=2n, n is the number of bits, the maximum difference between actual MSD and reported MSD can become even smaller than that by equidistant delta when MSD is small while the difference can become even larger than that by fixed delta when MSD is large.

Observation 14: Provided that whichever approach is selected, NW needs to store MSD information per BC, MSD per MSD source per BC or MSD per order per MSD source per BC, fixed delta approach cannot have advantage compared to the other approaches.

# Reference

- RP-220987, New WID on Further RF requirements enhancement for NR frequency range 1 (FR1), TSG RAN Meeting #95e

- R4-2119438, Proposals on capability-based MSD for CA and DC combinations, Qualcomm Incorporated, TSG-RAN WG4 #101-e

- R4-2114700, “Views on defining “low MSD” for CA and DC”, Apple, TSG-RAN WG4 #100-e

- R4-2212093, “Possible approaches to improve MSD”, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #104-e