3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212108

Electronic Meeting, 15-26 Aug., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On PDCCH and PBCH Requirements for ext71GHz

Agenda item:	9.14.8.2.2

Document for:	Discussion

Introduction

During the last RAN4#103-e meeting, good progress was made on the topic of PDCCH and PBCH requirements for FR2-2.

In this contribution we will express our views on the open issues and open new discussions, if necessary.

Discussion

PDCCH Performance Requirements

Whether to define requirements for multi-slot PDCCH monitoring

In RAN4#103e it was discussed if requirements shall be defined for multi-slot PDCCH monitoring [1]:

Issue 3-1-1: Whether to define requirements for multi-slot PDCCH monitoring - Option 1: Yes - Option 1a: Define one requirement for both UE supporting monitor one slot and two slots every four slots. - Option 2: No - Option 2a: Implicitly test this in PDSCH requirements.

This is a required feature for UEs supporting 480kHz and/or 960kHz, RAN4 shall define requirements for this (i.e. define requirements with 1 slot monitoring within a slot-group of 4 slots which is mandatory for 480KHz SCS). In case 960kHz is agreed, the mandatory requirement of 8 slots should be used for 960kHz.

Multi-slot PDCCH monitoring is a mandatory feature for 480kHz/960kHz SCS, hence requirements should be defined for the feature

We see implicitly testing multi-PDCCH monitoring in the PDSCH requirements as a viable way, however directly defining requirements would be preferred.

Define direct requirements for multi-slot PDCCH monitoring.

Test Cases

In RAN4#103e it was discussed which test cases select for PDCCH performance requirements. In the GTW meetings an initial list of test cases was agreed [1]:

Issue 3-1-2: Test Cases - Option 1: CORESET Duration: SCS (kHz) / CBW(MHz); CORESET Duration. 120/100; 1 for AL 2,4,8; 2 for AL16 (60 RBs). 120/400; 1 and 2 for each AL (240 RBs). 480/400; 1 and 2 for each AL (60 RBs). 480/1600; 1 (240 RBs). Cover following cases for each CBW/SCS and Propagation Condition: Test case; Interleaver size; REG bundle size; Aggrega-tion level; Antenna config. Test 1-1; 3; 2; 2; 1x2 ULA Low. Test 1-2; 2; 6; 4; 1x2 ULA Low. Test 1-3; 3; 2; 8; 2x2 ULA Low. Test 1-4; 3; 2; 16; 2x2 ULA Low. - Option 2: Cover following for 120kHz/100MHz and 480kHz/400MHz. Align propagation condition with PDSCH. Test number; CORESET duration; CORESET RB; Interleaver size; REG bundle size; Aggregation level; Propagation Condition; DCI format; Information Bit; Antenna configuration and correlation Matrix. 1; 1; 60; 3; 2; 2; TBD; 1_0; 40; 1x2 Low. 2; 1; 60; 2; 6; 4; TBD; 1_1; 56; 1x2 Low. 3; 1; 60; 3; 2; 8; TBD; 1_1; 56; 2x2 Low. 4; 2; 60; 3; 2; 16; TBD; 1_0; 40; 2x2 Low. - Option 3: Cover Test1,3 in Option 2 for 120kHz/100MHz, Test 2,4 in Option 2 for 480kHz/400MHz. Align propagation condition with PDSCH. - Option 4: Cover all tests in option 2 for 120kHz/100MHz , and additionally test 2,4 in option 2 for 480kHz/400MHz. Initial list of test cases for simulation results in the next meeting is given below as a starting point. Agree on final list of test cases in the next meeting based on simulation results, considering testability and minimize the number of test cases. Below list was used for initial simulation purpose to align the simulation assumption. RAN4 not deicide the final test case list which subject to further discussion and it’s not precluded to specify requirements for the configurations which not captured in current list. Test number; CBW/SCS; CORESET duration; CORESET RB; Interleaver size; REG bundle size; Aggregation level; Propagation Condition; DCI format; Information Bit; Antenna configuration and correlation Matrix. 1; 100/120; 1; 60; 3; 2; 2; TDLA10-200; 1_0; 40; 1x2 Low. 2; 100/120; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. 3; 100/120; 1; 60; 3; 2; 8; TDLA10-200; 1_1; 56; 2x2 Low. 4; 100/120; 2; 60; 3; 2; 16; TDLA10-200; 1_0; 40; 2x2 Low. 5; 400/480; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. 6; 400/480; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. Note: Interested companies can provide simulation results for other cases not included in above table, as needed..

Option 4 provides a good coverage for both 120 kHz SCS and 480 kHz SCS and is also used to define the agreed initial list of test cases which we see as a good starting point.

It should however in our view still be considered if option 4 should be extended with additional tests for 480kHz SCS and 960kHz SCS in case 960kHz SCS is agreed.

Cover following for 120kHz/100MHz. Align propagation condition with PDSCH.

| Test number | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 60 | 3 | 2 | 2 | TBD | 1_0 | 40 | 1x2 Low |
| 2 | 1 | 60 | 2 | 6 | 4 | TBD | 1_1 | 56 | 1x2 Low |
| 3 | 1 | 60 | 3 | 2 | 8 | TBD | 1_1 | 56 | 2x2 Low |
| 4 | 2 | 60 | 3 | 2 | 16 | TBD | 1_0 | 40 | 2x2 Low |

NOTE: Other test cases not precluded.

Cover following for 480kHz/400MHz and 960kHz/400MHz. Align propagation condition with PDSCH.

| Test number | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 60 | 2 | 6 | 4 | TBD | 1_1 | 56 | 1x2 Low |
| 2 | 2 | 60 | 3 | 2 | 16 | TBD | 1_0 | 40 | 2x2 Low |

NOTE: Other test cases not precluded.

PBCH Performance Requirements

SSB index assumption

In RAN4#103e it was discussed if SBB index knowledge can be assumed [1]:

Issue 3-2-2: SSB index assumption - Option 1: Only with not known SSB index - Option 2: Both known and not known SSB index - Option 3: Only known SSB index for 120kHz and 480kHz SCS

In our view SSB index detection is also important to be tested. The index detection is not used only in initial access, but for RRM measurements, like inter-frequency measurements. Based on this, we think that defining requirements “with not known SSB index” is needed as minimum. It would however be good to also define requirements “with known SSB index” to cover both cases.

SSB index detection is used for RRM measurements, so in our understanding it is important to have requirements with “not known SSB index”.

Defining requirements with both cases would be preferred.

Define requirement for both known and not known SSB index.

Conclusion

In this contribution we have provided our views on various open issues with relation to PDCCH and PBCH requirements for the extension to 71GHz.

We have made the following observations and proposals:

PDCCH Performance Requirements

Whether to define requirements for multi-slot PDCCH monitoring

- Multi-slot PDCCH monitoring is a mandatory feature for 480kHz/960kHz SCS, hence requirements should be defined for the feature

We see implicitly testing multi-PDCCH monitoring in the PDSCH requirements as a viable way, however directly defining requirements would be preferred.

- Define direct requirements for multi-slot PDCCH monitoring.

Test Cases

Cover following for 120kHz/100MHz. Align propagation condition with PDSCH.

| Test number | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 60 | 3 | 2 | 2 | TBD | 1_0 | 40 | 1x2 Low |
| 2 | 1 | 60 | 2 | 6 | 4 | TBD | 1_1 | 56 | 1x2 Low |
| 3 | 1 | 60 | 3 | 2 | 8 | TBD | 1_1 | 56 | 2x2 Low |
| 4 | 2 | 60 | 3 | 2 | 16 | TBD | 1_0 | 40 | 2x2 Low |

NOTE: Other test cases not precluded.

Cover following for 480kHz/400MHz and 960kHz/400MHz. Align propagation condition with PDSCH.

| Test number | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 60 | 2 | 6 | 4 | TBD | 1_1 | 56 | 1x2 Low |
| 2 | 2 | 60 | 3 | 2 | 16 | TBD | 1_0 | 40 | 2x2 Low |

NOTE: Other test cases not precluded.

PBCH Performance Requirements

SSB index assumption

SSB index detection is used for RRM measurements, so in our understanding it is important to have requirements with “not known SSB index”.

Defining requirements with both cases would be preferred.

Define requirement for both known and not known SSB index.

- References

- R4-2210665 - WF on UE demodulation performance requirements definition for 52.6 - 71 GHz

- R4-2210332 - Email discussion summary for [103-e][326] NR_exto71GHz_Demod_Part2

- draft_R4-2210332_Summary_326_2ndRound.docx

- TS 38.101-4, NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements