3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212185

# Electronic Meeting, Aug. 15 – 26, 2022

Source:	LG Electronics

Title:	Discussion on UE RF requirements for simultaneous DL reception with up to 4 Layer MIMO

Agenda item:	11.8.2

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirements for simultaneous DL reception with up to 3 Layer MIMO in FR2-1.

Discussion

Based on the objective of the latest WID [1] which is captured below, at first, RAN4 should introduce enhance RF requirements for PC3 with simultaneous reception from different directions with different QCL TypeD RSs.

[Objective]

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTE: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

In Rel-17, FR2-1 UE can support up to 2 layers with 2 RX ports in a single carrier. For the UE, RF requirements such as REFSENS (EIS) and EIS spherical coverage were specified with 1 layer and demodulation performance requirements were specified with up to 2 layers.

The following is the formula for REFSENS. Herein, a diversity gain of 0dB was assumed.

-    REFSENS = -174dBm(kT) + 10*log10(Max. RX BW) + NF – Total Ant. gain - diversity gain + SNR + ILs

For PC3, EIS spherical coverage requirement was specified with EIS at CCDF of 50%-tile considering the activated one panel at a time.

In Rel-18, compared to Rel-17, the main difference is that UE should support simultaneous reception from different directions with different QCL TypeD RSs up to 4 layers.

To support it, at least 2 panels need to be activated simultaneously. Figure 2-1 shows the possible example of 2-panel placements, such as Back to Back(a), Orthogonal(b), and In line(c) in Figure 2-1. Herein, we assume the opposite direction.

Figure 2.1: Example for 2-panel placements

From Figure2-1, Back to Back placement is more applicable for the opposite direction. For RF requirements, we need to consider which panel placements.

Proposal 1: Consider at least the activated Back to Back 2 panels to define RF requirements for enhanced FR2-1 UEs.

With 2 TRPs and 2 panels, up to 4 layers can be configured. However, 1 layer needs to be considered for UE RF Rx requirements as baseline same as the existing RF requirements.

Proposal 2: Consider 1 layer as a baseline for enhanced FR2-1 UE RF Rx requirements.

The existing RF test procedure was made based on assuming one TRP and one activated panel as follows(one panel is assumed with consisting of θ-pole and ϕ-pole).

-   Position the UE so that the beam is formed towards the measurement antenna in the RX beam peak direction.

-   Switch the downlink to the PolLink= θ-polarization of the measurement antenna.

-   Determine EIS(PolMeas= PolLink= for θ-polarization

-    Switch the downlink to the PolLink=ϕ-polarization of the measurement antenna.

-    Determine EIS(PolMeas= ϕ, PolLink= ϕ) for ϕ-polarization

-    Calculate the resulting averaged EIS as:

EIS = 2*[1/EIS(PolMeas= PolLink= +1/EIS(PolMeas= ϕ, PolLink= ϕ)]-1

In Rel-18, compared to the existing RF test procedure, 2 links and 2 measurement antennas are required for 2 TRPs and 2 panels. Here, Link_A and Link_B for 2 links, and Meas_A and Meas_B for 2 measurement antennas can be assumed. The expected RF test procedure can be as follows.

-   Position the UE so that the beam is formed towards the measurement antennas in the RX beam peak directions.

-   Switch the downlink Link_A and Link_B to

PolLink_A= θ-polarization of the measurement antenna, Meas_A and

PolLink_B= θ-polarization of the measurement antenna, Meas_B.

-   Determine EIS for θ-polarization (EIS_

-   Switch the downlink Link_A and Link_B to

PolLink_A= ϕ-polarization of the measurement antenna, Meas_A and

PolLink_B= ϕ-polarization of the measurement antenna, Meas_B.

-   Determine EIS for ϕ-polarization (EIS_ϕ

-   Calculate the resulting averaged EIS as:

EIS = 2*[1/EIS_ +1/EIS_ϕ]-1

EIS_and EIS_ϕ can be determined with the received signal, y_ and y_ϕ respectively as follow.

-   y_ = y(PolMeas_A= PolLink_A=y(PolMeas_B= PolLink_A=

y(PolMeas_B= PolLink_B=y(PolMeas_A= PolLink_B=

-   y_ϕ = y(PolMeas_A=ϕ PolLink_A=ϕy(PolMeas_B=ϕ PolLink_A=ϕ

y(PolMeas_B=ϕ PolLink_B=ϕy(PolMeas_A=ϕ PolLink_B=ϕ

Assuming that the received signal from cross-link is ‘0’, y_and y_ϕ are simplified as,

-   y_ = y(PolMeas_A= PolLink_A=y(PolMeas_B= PolLink_B=

-   y_ϕ = y(PolMeas_A=ϕ PolLink_A=ϕy(PolMeas_B=ϕ PolLink_B=ϕ

Assuming 1 layer, y(PolMeas_A= PolLink_A=can be equal to y(PolMeas_B= PolLink_B= y_ϕ is the same story.

-   y_ = 2*y(PolMeas_A= PolLink_A=

-   y_ϕ = 2*y(PolMeas_A=ϕ PolLink_A=ϕ

It means that diversity gain needs to be considered for both y_and y_ϕ with 1 layer transmitting the same signal from Link_A and Link_B.

Proposal 3: Consider diversity gain of ‘non-zero’ for REFSENS of enhanced FR2-1 UEs.

For EIS spherical coverage of enhanced FR2-1 UE, the CDF of EIS is different from the legacy FR2-1 UE. Because CDF needs to be made based on the collected data from all combinations of Link_A and Link_B.  It can require huge test time compared to legacy FR2-1. It is better how to solve it to be discussed in OTA testability.

Proposal 4: For EIS spherical coverage, consider CDF based on all combinations of two different Rx directions.

Conclusion

In this contribution, we provided our views on the enhanced FR2-1 UE RF Rx requirements. Based on the views, we proposed as follows.

Proposal 1: Consider at least the activated Back to Back 2 panels to define RF requirements for enhanced FR2-1 UEs.

Proposal 2: Consider 1 layer as a baseline for enhanced FR2-1 UE RF Rx requirements.

Proposal 3: Consider diversity gain of ‘non-zero’ for REFSENS of enhanced FR2-1 UEs.

Proposal 4: For EIS spherical coverage, consider CDF based on all combinations of two different Rx directions.

- References

[1] RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm