3GPP TSG-RAN WG4 Meeting # 104-e										R4-2212321

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	14.1

Source: 	CMCC

Title: 	Discussion on R18 high power UE basket work items

Document for:	Discussion

# 1. Introduction

During RAN#96 e-meeting, HPUE basket work items were discussed in [1] and some related work items are postponed to next RAN #97 meeting. Some remaining issues are listed as below that needs further discussion in this RAN4 meeting.

- Issue #2-1: how to handle RP-221312 (New WID on high power UE for NR inter-band CA with 26dBm on FDD band). Companies commented that the general requirements need be finalized first.

- Issue #2-2: how to handle RP- RP-221329? The general requirements for PC1.5 should be finalized first.

- Issue #2-5: Add the objective “Investigate solution which targets at reducing sensitivity degradation" into 1311 WID.

In this contribution, we focus on the discussion of HPUE basket work items proposed in RAN plenary meeting and propose a new basket WID on High power UE (power class 1.5) operation on single TDD UL carrier for NR single band and band combinations.

# 2. Discussion

For the new WID on HPUE for NR inter-band CA with 26dBm on FDD band, the main concern is that general requirements need to be specified at first before band combination specific requirements. The common issue is about SAR. If we utilize current mechanism to resolve SAR issues, FDD band could transmit max 26dBm power but with limited UL duty cycle. If so, left power for TDD band is less for 26dBm total output power case and FDD could only work TDM. In conclusion, UE FDD band works like TDMed. SAR solutions for PC2 inter-band CA with PC2 TDD band maybe also applicable for PC2 inter-band CA with PC2 FDD band. But without any detailed analysis, we just can’t directly utilize legacy solutions.

Proposal 1: it’s suggested to study SAR issues and solutions in FR1 enhancement WID for NR inter-band CA with 26dBm on FDD band.

For the new WID on high power UE (power class 2) for NR FDD bands, it’s suggested to add the objective “Investigate solution which targets at reducing sensitivity degradation”. For FDD bands especially less than 1GHz spectrum, the TX/RX gap is much smaller and sensitivity desensitization would be even severe. We need to find out solutions to help reduce sensitivity desensitization rather than directly defining requirement using legacy method. Band n28 and n8 are suggested as example band for related discussion.

During actual deployment, UE vendors will optimize Rx performance with less MSD to fully use spectrum especially the low band golden spectrum. It is suggested to investigate new solutions in 3GPP, a dedicated WID or merge such discussion within FR1 enhancement WI is more preferred.

Proposal 2: For HPUE FDD bands especially band less than 1GHz, it’s suggested to investigate solution targeting at reducing sensitivity degradation in a dedicated WID or merged in FR1 enhancement WID. n28 and n8 are the example bands.

Until now, RAN4 has defined PC1.5 for band n41, n77, n78 and n79. For R18 there is no basket WID to capture other TDD bands for PC1.5. it’s suggested to set up a new basket WID on PC1.5 UE for NR TDD bands. Besides, there is no work item that could cover single UL PC1.5 with x band DL CA. CR on addition PC1.5 single n77 uplink for 3DL combinations are endorsed in TEI agenda in last meeting [2]. This new WID is also suggested to cover such cases as below:

- PC1.5 operation on single TDD UL CC for NR single band operation

- PC1.5 operation on single TDD UL CC for NR inter-band/ intra-band CA/DC combination with X bands DL. X=1,2,3

Proposal 3: a new WID on High power UE (power class 1.5) operation on single TDD UL carrier for NR single band and band combinations with following cases:

- PC1.5 operation on single TDD UL CC for NR single band operation

- PC1.5 operation on single TDD UL CC for NR inter-band/ intra-band CA/DC combination with X band DL. X=1,2,3

A new WID is provided as in [3].

# 3. Conclusions

In this contribution, HPUE basket work items are discussed with following proposals:

Proposal 1: it’s suggested to study SAR issues and solutions in FR1 enhancement WID for NR inter-band CA with 26dBm on FDD band.

Proposal 2: For HPUE FDD bands especially band less than 1GHz, it’s suggested to investigate solution targeting at reducing sensitivity degradation in a dedicated WID or merged in FR1 enhancement WID. n28 and n8 are the example bands.

Proposal 3: a new WID on High power UE (power class 1.5) operation on single TDD UL carrier for NR single band and band combinations with following cases:

- PC1.5 operation on single TDD UL CC for NR single band operation

- PC1.5 operation on single TDD UL CC for NR inter-band/ intra-band CA/DC combination with X band DL. X=1,2,3

# 4. Reference

[1] RP-221822, RAN4 R18 baskets handling, RAN4 chair

[2] R4-2210766, Draft CR for 38.101-1: Addition PC1.5 single uplink for 3DL combinations, Verizon, Samsung

[3] R4-2212322, WID on High power UE (power class 1.5) operation on single TDD UL carrier for NR single band and band combinations, CMCC