3GPP RAN WG4 Meeting #104-e		R4-2212380

Online, August 15th – 26th, 2022

Agenda item:	10.1

Source:	Apple

Title:	On Fallbacks for Basket WIDs

Release:	Rel-18

Document for:	Approval

# 1	Introduction

For Rel. 18 there have been multiple basket WIDs agreed, as it has been for the previous releases. However, experience shows that when requesting band combinations for the basket WIDs many proponents just add the higher order combinations they want, but they forget to add some or all the fallback combinations. We need to clarify in the basket WIDs, that only combinations are added, where all the fallbacks are added as well and have some proposals how this can be ensured.

# Discussion

## 2.1	Fallbacks requirements in 36.101 and 38.101-x specs

It is clearly said in 36.101 that all fallbacks need to be supported by the UE, therefore also all fallbacks need to be specified in 36.101, since otherwise it is not possible for the UE to support these fallback combinations.

Clause 4.3A from 36.101[1]:

A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination.

Similarly 38.101-1[2], 38.101-2[3] and 38.101-3[4] require to support all fallbacks, which also requires all fallbacks to be specified in the 38.101-x specs.

38.101-1, Table 5.3A-5-1, Note 2 (the same sentence is in 38.101-2, Table 5.3A.4-1, Note 2):

It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group.

In 38.101-3 clause 4.2 we can read:

A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

We can see in the specs that there is a requirement to support the fallbacks, but for the UE to be able support these fallbacks, the fallbacks and their requirements also need to be specified in 36.101 and 38.101-x. The constituent E-UTRA or NR parts of an EN-DC or NE-DC combinations need to be specified as well.

To conclude:

Observation 1: In 36.101 and 38.101-x specs it is mandatory to support all the lower order fallbacks of the higher order combination according to the fallback requirements mentioned in the specs.

Observation 2: To support lower order fallback combinations of higher order combinations specified in 36.101 and 38.101-x specs, it is required to specify all the fallback combinations in 36.101 and 38.101-x as well.

Observation 3: For EN-DC or NE-DC configurations the constituent E-UTRA and NR configurations need to be specified as fallbacks as well.

## 2.2	Fallbacks in Basket WIDs

Since all the fallbacks need to be supported and therefore specified in the 36.101 and 38.101-x specs, not yet specified lower order fallbacks of a higher order combination need to be requested together with the higher order combination. Since the fallbacks are iterative, i.e. a higher order combination has fallbacks with one less carrier and these combinations again have lower order fallbacks, all these fallbacks need to be specified down to a dual carrier configuration as well as the constituent E-UTRA and NR configurations.

A proponent of a combination shall check, which fallbacks are not yet specified and needs to request them together with the highest order combination in the basket WIDs.

Already in the first agreed papers on the basket WIDs there was an agreement, that fallbacks need to be specified and a new combination can only be agreed once the fallbacks are specified. For example R4-161491 states for LTE:

b.	A CA configuration can only be considered as completed when ALL fallback mode configurations (which may be in different baskets or different releases) are completed. It is the responsibility of the contact person to verify the status of the fallback modes.  The contact person shall also list all the next level fallback configurations to be completed in the same REL and its status (completed: yes/no) in the status report.

However, it seems this requirement has been forgotten over time.

Also 38.862 states, that a combination can only be added, if all lower order fallbacks have been added:

#6 The contributions will be discussed during RAN4#X meeting. If there are no technical concerns and if all the needed fallbacks are completed, the band combinations can be approved. And the final decision will be made by chairman.

Observation 4: In RAN4 it has been agreed long time ago, that a new combination can only be added, once all the fallbacks have already been specified

Observation 5: Fallbacks do not necessarily need to be in the same basket WID, they may also need to be specified in other lower order basket WIDs

Observation 6: The proponent of the higher order combination is responsible to request all fallbacks as well, otherwise the rapporteur should reject that combination.

## 2.3	Examples of added combinations with missing Fallbacks

There are many cases, where it is quite obvious, that there are missing fallbacks, here is just one example from a CR to add combinations in the last meeting:

We see that CA_n2A-n5A-n261A has been added, then CA_n2A-n5A-n261I, but CA_n2A-n5A-n261G and CA_n2A-n5A-n261H as the fallbacks of CA_n2A-n5A-n261I are missing.

There are also issues, where combinations with a higher number of bands have been added, but the combinations with less bands not. For example DC_1A-7C-20A-28A-32A_n3A has been added, but all fallbacks with 7C have not been added, for example DC_1A-7C-20A-28A_n3A, DC_1A-7C-20A-32A_n3A, DC_1A-7C-28A-32A_n3A, DC_7C-20A-28A-32A_n3A, DC_1A-7C-32A_n3A, DC_7C-20A-28A_n3A, DC_7C-20A-32A_n3A, DC_7C-28A-32A_n3A are missing.

## 2.4	Proposals for the Basket Work Items

It seems many delegates requesting combinations and working on implementing the combinations in the spec do not remember the previous agreements how to work with fallbacks. The result is that we add many combinations without the required fallbacks into the spec and need to have separate CRs to add the missing fallbacks. It would be better to add a small paragraph clarifying the fallback handling in each WID, so that everyone knows how to request combinations with fallbacks and the fallbacks are not forgotten.

Proposal 1: RAN4 to confirm the agreement that all fallbacks down to a dual carrier configuration need to be requested together with a higher order configuration and no configuration shall be added to the specification with a CR without having added all the lower order fallbacks previously

Proposal 2: All rapporteurs of a Basket WID are required to add the following text to each of the Basket WIDs in the Objective in 4.1 of the WID:

- The proponent of a combination needs to check for all fallback configurations down to a dual carrier configuration, if they are already specified. If not, the proponent needs to request the fallbacks and their missing fallbacks to be added to the appropriate basket WIDs. The rapporteur should refrain from adding combinations to the CRs, for which the proponent hasn’t shown that all fallbacks are already in the specification or are specified together with the higher order combination.

Proposal 3: All rapporteurs of a Basket WID shall not add configurations to the big CRs, for which the proponent has not shown that all lower order fallbacks have already been added to the specification or are added together with that configuration to the same specification

# 3	Conclusion

In this contribution, we want to confirm the common understanding in RAN4 of the need to specify all fallback combinations and their fallbacks down to a dual carrier configuration. This should be documented in each WID.

Observation 1: In 36.101 and 38.101-x specs it is mandatory to support all the lower order fallbacks of the higher order combination according to the fallback requirements mentioned in the specs.

Observation 2: To support lower order fallback combinations of higher order combinations specified in 36.101 and 38.101-x specs, it is required to specify all the fallback combinations in 36.101 and 38.101-x as well down to a dual carrier configuration.

Observation 3: For EN-DC or NE-DC configurations the constituent E-UTRA and NR configurations also need to be specified as fallbacks.

Observation 4: In RAN4 it has been agreed long time ago, that a new combination can only be added, once all the fallbacks have already been specified

Observation 5: Fallbacks do not necessarily need to be in the same basket WID, they may also need to be specified in other lower order basket WIDs

Observation 6: The proponent of the higher order combination is responsible to request all fallbacks as well, otherwise the rapporteur should reject that combination.

Proposal 1: RAN4 to confirm the agreement that all fallbacks down to a dual carrier configuration need to be requested together with a higher order configuration and no configuration shall be added to the specification with a CR without having added all the lower order fallbacks previously

Proposal 2: All rapporteurs of a Basket WID are required  to add the following text to each of the Basket WIDs in the Objective in 4.1 of the WID:

- The proponent of a combination needs to check for all fallback configurations down to a dual carrier configuration, if they are already specified. If not, the proponent needs to request the fallbacks and their missing fallbacks to be added to the appropriate basket WIDs. The rapporteur should refrain from adding combinations to the CRs, for which the proponent hasn’t shown that all fallbacks are already in the specification or are specified together with the higher order combination.

Proposal 3: All rapporteurs of a Basket WID shall not add configurations to the big CRs, for which the proponent has not shown that all lower order fallbacks have already been added to the specification or are added together with that configuration to the same specification

# 4	References

3GPP TS 36.101 V17.6.0 (2022-06)

3GPP TS 38.101-1 V17.6.0 (2022-06)

3GPP TS 38.101-2 V17.6.0 (2022-06)

3GPP TS 38.101-3 V17.6.0 (2022-06)