3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212498

Electronic meeting, August 15 – August 26, 2022

Source: 	Huawei, HiSilicon

Title: 	Link level simulation for FR2 UL 256QAM

Agenda Item:	11.7.2

Document for:	Approval

- Introduction

For FR2 NR RF requirements enhancement phase 3 [1], one of the objective is UL 256QAM.

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

In this contribution, we provide a link level simulation to study the gain and operating SNR, taking phase noise and other RF impairment into account.

- Discussion

2.1	Simulation assumption

With the purpose to evaluate the impact of RF impairments, we did link level simulation to check the performance benefit of 256QAM. The following simulation assumptions are used.

Table 2-1 Simulation assumptions for throughput comparison of 256QAM and 64QAM

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz |
| CBW | 50 MHz |
| SCS | 120 kHz; |
| Allocated RBs | Full allocation |
| Propagation | TDL-D 30ns delay spread, 35Hz Doppler frequency |
|  | Static (AWGN) |
| MCS | 64QAM: MCS 23, code rate 719/1024 256QAM: MCS21, code rate 711/1024 |
|  | Baseline: fixed MCSs |
| Precoding | follow PMI |
| Symbol type | CP-OFDM |
| HARQ | None |
| Antenna configuration | Fading channel: 2x2 for Rank1, Low correlation |
|  | Static channel: 2x2 for Rank1 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) Option a): example1 + example1 Option d):example2 (BS) + PN model config1: example1(UE) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 1%, 2%, 3%, 3.5%; rxEVM: 1%, 2%, 3%, 3.5%; |

Proposal 1: it is proposed to adopt the simulation assumption in Table 2-1 in the link level simulation.

The preliminary link lever simulation results are shown in Figure 2-1, Figure 2-2 and Figure 2-3 for CDL-D 30. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 2-1 Throughput comparison of 256QAM and 64QAM for RANK1 with Option a phase noise model

Figure 2-2 Throughput comparison of 256QAM and 64QAM for RANK1 with Option d phase noise model

Figure 2-3 Throughput comparison of 256QAM and 64QAM for RANK2 with Option d phase noise model

- Conclusion

In the contribution, we provide proposals for the simulation assumption and preliminary simulation results to study the gain and operating SNR for UL 256QAM.

Proposal 1: it is proposed to adopt the simulation assumption in Table 2-1 in the link level simulation.

- Reference

- RP-221862, “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi, RAN Meeting #96, June 2022