3GPP TSG-RAN WG4 Meeting#104-e	R4-2212559

Online, 15th Aug – 26th Aug, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on Performance Requirements for Multi-TRP PDCCH Tests

Agenda item:	9.17.4.1.2

Document for:	Discussion

- Introduction

During RAN4#103-e, it was extensively discussed whether to define multi-TRP PDCCH performance requirement as well as what decoding mechanism should be assumed for UE processing. In this paper, we share our views on the remaining open issues.

- Simulation Results for m-TRP PDCCH Repetition Scheme

Issue 1-1-1: Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

Agreements:

- Introduce PDCCH requirements for multi-TRP repetition transmission

  - Option 1: With UE soft-combining

  - Option 2: Without UE soft-combining

The multi-TRP PDCCH repetition scheme allows intra-slot repetition of the same coded bits over two TRPs. The aim is to improve the performance of the PDCCH reception exploiting the repetition. As presented below, as of RAN4#103-e, it is still open as to what decoding assumption UE should assume for RAN4 testing [1].

We note that RAN1 did not make any explicit agreement on how blind decoding (BD) factors 2 and 3 map to decoding assumptions. In particular, the motivation for BD factor 2 was to allow for UE to just decode the two linked candidates individually. Additionally, soft combining is an optional feature based on the UE capability, e.g., for BD factor 3. We also emphasize that RAN4 test cases should not be designed with an optional feature.

Proposal 1: Option 2 (Introduce PDCCH requirements for multi-TRP repetition transmission without UE soft-combining)

Issue 1-2-1: PDCCH requirement setup

Agreements:

- Operating SNR condition for selection AL

  - Option 1: SNR > [-4] dB

  - Other options are not precluded

- Aggregation level

  - Option 1: AL=2

  - Option 2: AL=4

  - Decide aggregation level base on simulation results in next meeting.

As for the test setup, RAN4 also discussed as to what aggregation level (AL) should be assumed for testing purpose. To this end, both AL=2 and AL=4 were kept open and it was decided that AL will be selected assuming the operating SNR is greater than -4dB. Our simulation results suggest that AL= 4 mostly require much lower SNR than -4dB [2].

Proposal 2: Option 1 (Select AL assuming operating SNR > -4 dB)

Proposal 3: Option 1 (Choose AL=2)

- Conclusions

In this paper, we share our views for the m-TRP PDCCH repetition scheme. The following has been proposed.

Proposal 1: Option 2 (Introduce PDCCH requirements for multi-TRP repetition transmission without UE soft-combining)

Proposal 2: Option 1 (Select AL assuming operating SNR > -4 dB)

Proposal 3: Option 1 (Choose AL=2)

# 4. References

[1] R4-2210670, “WF on demodulation requirement for Enhancement on Multi-TRP,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 09 – May 20, 2022.

[2] R4-2212580, “Simulation Results on m-TRP PDCCH Tests,” 3GPP TSG RAN WG4 Meeting #104-e, Online, Aug 15 – Aug 26, 2022.