3GPP TSG-RAN WG4 Meeting #104-e	R4-2212571

Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	9.14.8.2.1

Source:	Ericsson

Title:	Simulation results for PDSCH demodulation in 52.6 – 71 GHz band.

Document for:	Information

# 1	Introduction

In this paper and based on the progress summarized in WF [1], we present new simulation results on the PDSCH performance in the frequency range 52.6 GHz to 71 GHz. We carried out our simulations using the agreed channel models and parameters.

In RAN4 meeting #103-e, WF [1] underlined some issues to consider in the simulation setup. As general aspects, the propagation environments to be considered are given by

Issue 1-2-1/1-2-2: TDL Channel Model and RMS Delay Spread - RAN4 will specify demodulation requirements covering TDL-A10 (baseline) and TDL-D10 (for specific test cases) - FFS whether TDL-A20 needed or not Issue 1-2-3: Max Doppler Frequency - TDL-D with 200Hz Max doppler - TDL-A channel: - Option 1: 200Hz - Option 2: 650Hz - Option 3: both 200Hz, and 650Hz Issue 2-1-11: Phase Noise Model Assumption - PN is considered at UE only

However, several needed related parameters have not been agreed yet, such as

Issue 2-1-7: Whether to define requirements at 30% of peak throughput - Option 1: Yes - Option 2: No Issue 2-1-9: Whether to define requirements with Rank2 - Option 1: Yes - Option 2: No - Option 3: Decide based on simulation results. Issue 2-1-12: Phase Noise Compensation - Option 1: CPE compensation only - Option 2: CPE+ICI compensation - Option 3: CPE or CPE+ICI decision depends on the test case Larger MCS and CBW may benefit with CPE+ICI.

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

As per WF [1], and due to the channel resolution issues, we carried out simulations using a delay spread of 10 ns and 20 ns for both agreed channel models TDLA and TDLD, while considering low and moderate velocities (given by the maximum Doppler frequencies of 200 Hz and 650 Hz).

Furthermore, RAN4 was not able to agree on a single representative phase noise (PN) model during the study item. Therefore, and for the sake of fairness, we performed our simulations considering both PN models in TR 38.808 [2], referring to as Set 1 and Set 2, where Set 1 has been proposed in [3] and Set 2 also known as TR 38.803 Example 2.

We will use as general parameters.

- Numerologies:

- SCS = 120 KHz, CBW = 100 MHz (66 PRB) and 400 MHz (264 PRB).

- SCS = 480 KHz, CBW = 400 MHz (66 PRB) and 800 MHz (124 PRB).

- MCS 4, 13, 17, 20, and 22

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 200 Hz), DMRS 1+1 ((2,11) symbol indexes)

- UE speed at 10 Km/h (f_D = 650 Hz), DMRS 1+1 ((2,11) symbol indexes)

- Two PN models in TR 83.808 [2]:

    - Set 1 in TR 38.808: PN model in [3], referred to as R4_2010176

The design margin (DM) should be set to 0 dB for BS while to be set to 5 dB for UE

    - Set 2 in TR 38.808: PN model TR38.803 Ex2, Set 2 in TR 38.808

- Carrier frequency: 70 GHz

- Antenna configuration: 2x2 ULA Low

- Rank 1

- Rel-15 PTRS configuration, K = 2, L = 1

- Phase noise at the UE receiver only

- Benchmark: No PN and No compensation

- CPE compensation only

- CPE + ICI compensation (de-ICI filter of length 3 (u = 1) and length 5 (u = 2))

We summarize the parameters above in the following table

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 20 ns delay spread (DS)) and TDLD (10 ns DS) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | TR38.803 example 2 BS PN profile and Set 1 [2] |
| UE PN Model | TR38.803 example 2 UE PN profile and Set 1 [2] |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol indexes |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 4, 13, 17, 20, and 22. Note: It is assumed that NohPRB = 0 for MCS calculations. |

Observation 1: Low MCS is less susceptible to phase rotation error. Consequently, only CPE compensation will be applied.

By applying high MCS in the presence of PN effects, ICI compensation filtering could be needed especially for large PRB number allocation.

Observation 2: For moderate/high MCS and large PRB number allocation, the companies are encouraged to consider ICI compensation techniques while keeping same Rel-15 PTRS pattern.

## 2.2 Simulation results

In this section, we will present the normalized PDSCH throughput as function of the signal to noise ratio (SNR). As per RAN4 PDSCH metric, we will refer to SNR1 and SNR2 as the corresponding SNR value to 70% (or 30 %) of the peak throughput value when using PN models Set 1 and Set 2, respectively.

Furthermore, it is worth mentioning that for either low MCS or small PRB number allocation, we only apply CPE compensation. However, if a large PRB number is allocated (such as 124 or 264 PRBs), CPE compensation will be followed by ICI filtering to compensate PN effects when moderate/high MCS has been used. Therein, the ICI compensation filtering of lengths 3 (u = 1) and 5 (u = 2) will apply. In the sequel, we present the simulation setup tables that we have used.

Test cases:

- CBW (MHz)/SCS (KHz) = 100/120, TDLA10-200/650, TDLA20-200/650, TDLD10-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 70 | 100/120 (66) | 4 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | CPE | 2x2 ULA Low |
| Test 1-2 | 70 | 100/ 120 (66) | 13 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | CPE | 2x2 ULA Low |
| Test 1-3 | 70 | 100/ 120 (66) | 17 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 TDLD10-200 | CPE | 2x2 ULA Low |
| Test 1-4 | 70 | 100/ 120 (66) | 20 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 TDLD10-200 | CPE | 2x2 ULA Low |
| Test 1-5 | 70 | 100/ 120 (66) | 22 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 TDLD10-200 | CPE | 2x2 ULA Low |

- CBW (MHz)/SCS (KHz) = 400/120, TDLA10-200/650, TDLA20-200/650, TDLD10-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2-1 | 70 | 400/120 (264) | 17 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 TDLD10-200 | CPE CPE+ICI | 2x2 ULA Low |
| Test 2-2 | 70 | 400/ 120 (264) | 20 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 TDLD10-200 | CPE CPE+ICI | 2x2 ULA Low |
| Test 2-3 | 70 | 400/ 120 (264) | 22 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 TDLD10-200 | CPE CPE+ICI | 2x2 ULA Low |

- CBW (MHz)/SCS (KHz) = 400/480, TDLA10-200, TDLA20-200, TDLD10-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 3-1 | 70 | 400/480 (66) | 4 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE | 2x2 ULA Low |
| Test 3-2 | 70 | 400/480 (66) | 13 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE | 2x2 ULA Low |
| Test 3-3 | 70 | 400/480 (66) | 17 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE | 2x2 ULA Low |
| Test 3-4 | 70 | 400/480 (66) | 20 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE | 2x2 ULA Low |
| Test 3-5 | 70 | 400/480 (66) | 22 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE | 2x2 ULA Low |

- CBW (MHz)/SCS (KHz) = 800/480, TDLA10-200, TDLA20-200, TDLD10-200, Metric: 70% Peak Throughput,

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 70 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| Test 4-1 | 70 | 800/480 (124) | 17 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE CPE+ICI | 2x2 ULA Low | TBD | TBD |
| Test 4-2 | 70 | 800/480 (124) | 20 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE CPE+ICI | 2x2 ULA Low |  |  |
| Test 4-3 | 70 | 800/480 (124) | 22 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE CPE+ICI | 2x2 ULA Low |  |  |

- Metric 30% Peak Throughput, HARQ processes: 16

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 5-1 | 70 | 100/120 (66) | 13 | 1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | CPE | 2x2 ULA Low |
| Test 5-2 | 70 | 400/ 480 (66) | 13 | 1 | TDLA10-200 TDLD10-200 TDLA20-200 | CPE | 2x2 ULA Low |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models and MCSs but limited to 1 MIMO layer (Rank 1). | 1-1, 1-2, 1-3, 1-4, 1-5, 2-1, 2-2, 2-3, 3-1, 3-2, 3-3, 3-4, 3-5, 4-1, 4-2, 4-3, |
| 30% of peak Throughput | Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 5-1, 5-2 |

## 2.2.1 SCS/CBW = 120 KHz/100 MHz results

Considering our benchmark, when no PN applied at both sides (ideal case), the achieved SNRs at 70% of the peak throughput have been referred to as SNR0 and gathered along with SNR1 and SNR2 in Tables 1.1 and 1.2. Furthermore, in the sequel, we will refer to D1 = SNR1 – SNR0 and D2 = SNR2 – SNR0 as the SNR gap (dB) between the PN compensation output, either by applying CPE only or followed by ICI compensation, to the ideal case.

For 120 KHz SCS and 100 MHz CBW, only 66 PRBs are allocated where Table 1.1 gathers results for low MCS (4 and 13) while Table 1.2 considers moderate and high MCS (17, 20 and 22). It is worth that the Reference SNR0 threshold is at [7.7] dB.

Table 1.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for low MCS (4 and 13).

|  | CBW (MHz) / SCS (KHz) = 100/120 (66 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | PN compensa-tion | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-1 | 4 | TDLA10-200 | -3.3788 | No Comp | -3.2522 | - | -3.206 | - |
|  |  |  |  | CPE | -3.1424 | 0.2364 | -3.1289 | 0.2499 |
|  |  | TDLA10-650 | -3.1813 | No Comp | -3.028 | - | -2.9786 | - |
|  |  |  |  | CPE | -2.9126 | 0.2687 | -2.8743 | 0.307 |
|  |  | TDLA20-200 | -3.3421 | No Comp | -3.2319 | - | -3.1826 | - |
|  |  |  |  | CPE | -3.1246 | 0.2175 | -3.0797 | 0.2624 |
|  |  | TDLA20-650 | -3.0976 | No Comp | -2.9745 | - | -2.9579 | - |
|  |  |  |  | CPE | -2.8692 | 0.2284 | -2.851 | 0.2466 |
| Test 1-2 | 13 | TDLA10-200 | 4.4407 | No Comp | 4.7784 | - | 4.9897 | - |
|  |  |  |  | CPE | 4.7578 | 0.3171 | 4.9312 | 0.4905 |
|  |  | TDLA10-650 | 5.0223 | No Comp | 5.3213 | - | 5.5581 | - |
|  |  |  |  | CPE | 5.3199 | 0.2976 | 5.4865 | 0.4642 |
|  |  | TDLA20-200 | 4.4508 | No Comp | 4.8033 | - | 5.0328 | - |
|  |  |  |  | CPE | 4.7688 | 0.318 | 4.9456 | 0.4948 |
|  |  | TDLA20-650 | 4.9883 | No Comp | 5.3312 | - | 5.5643 | - |
|  |  |  |  | CPE | 5.286 | 0.2977 | 5.474 | 0.4857 |

Observation 3: Referring to Table 1.1, when considering SCS/CBW = 120 KHz/100 MHz and low MCS (4 and 13), both D1 and D2 remain below 1 dB showing that the CPE compensation is enough when considering TDLA10/20 – 200/650.

Table 1.2 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for moderate and high MCS (17, 20 and 22).

|  | CBW (MHz) / SCS (KHz) = 100/120 (66 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | PN compensa-tion | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-3 | 17 | TDLA10-200 | 7.7469 | No Comp | 8.4733 | - | 8.9909 | - |
|  |  |  |  | CPE | 8.3185 | 0.5716 | 8.7127 | 0.9658 |
|  |  | TDLA10-650 | 8.3275 | No Comp | 9.0621 | - | 9.5952 | - |
|  |  |  |  | CPE | 8.9399 | 0.6124 | 9.2739 | 0.9464 |
|  |  | TDLA20-200 | 7.7642 | No Comp | 8.5336 | - | 9.1027 | - |
|  |  |  |  | CPE | 8.3566 | 0.5924 | 8.7411 | 0.9769 |
|  |  | TDLA20-650 | 8.2882 | No Comp | 9.0361 | - | 9.6357 | - |
|  |  |  |  | CPE | 8.8699 | 0.5817 | 9.2879 | 0.9997 |
|  |  | TDLD10-200 | 4.0109 | No Comp | 4.6669 | - | 5.2091 | - |
|  |  |  |  | CPE | 4.5089 | 0.498 | 4.8917 | 0.8808 |
| Test 1-4 | 20 | TDLA10-200 | 10.0022 | No Comp | 11.3884 | - | 12.7607 | - |
|  |  |  |  | CPE | 10.9281 | 0.9259 | 11.7925 | 1.79 |
|  |  | TDLA10-650 | 10.7018 | No Comp | 12.0465 | - | 13.4952 | - |
|  |  |  |  | CPE | 11.6043 | 0.9025 | 12.393 | 1.6912 |
|  |  | TDLA20-200 | 10.1098 | No Comp | 11.5219 | - | 12.8943 | - |
|  |  |  |  | CPE | 11.0611 | 0.9513 | 11.8998 | 1.79 |
|  |  | TDLA20-650 | 10.7017 | No Comp | 12.0954 | - | 13.6599 | - |
|  |  |  |  | CPE | 11.6132 | 0.9115 | 12.4859 | 1.7842 |
|  |  | TDLD10-200 | 6.2545 | No Comp | 7.521 | - | 8.6399 | - |
|  |  |  |  | CPE | 7.1234 | 0.8689 | 7.8863 | 1.6318 |
| Test 1-5 | 22 | TDLA10-200 | 12.0276 | No Comp | 14.7793 | - | 20 | - |
|  |  |  |  | CPE | 13.5261 | 1.4985 | 15.5702 | 3.54 |
|  |  | TDLA10-650 | 12.7251 | No Comp | 15.4562 | - | 21.4625 | - |
|  |  |  |  | CPE | 14.1746 | 1.4495 | 16.1848 | 3.4597 |
|  |  | TDLA20-200 | 12.1082 | No Comp | 14.967 | - | 21.5139 | - |
|  |  |  |  | CPE | 13.6848 | 1.5766 | 15.9324 | 3.8242 |
|  |  | TDLA20-650 | 12.7617 | No Comp | 15.6086 | - | NaN | - |
|  |  |  |  | CPE | 14.2801 | 1.5184 | 16.4653 | 3.7036 |
|  |  | TDLD10-200 | 8.1849 | No Comp | 10.6297 | - | 14.0672 | - |
|  |  |  |  | CPE | 9.5798 | 1.3949 | 11.3984 | 3.2135 |

Observation 4: Referring to Table 1.2, when considering SCS/CBW = 120 KHz/100 MHz and moderate/high MCS (17, 20 and 22), D1 remains below 1 dB for MCS = 17 and 20, showing that MCS 20 is achievable under TDLA10/20-200 and TDLD10-200 channel models, while considering the CPE compensation only (considering PN model Set 1).

## 2.2.2 SCS/CBW = 120 KHz/400 MHz results

As per Observation 2, for high PRB allocation (264), we assess CPE+ICI compensation and provide D1 and D2 values when considering moderate and high MCS under TDLA10/20-200/650 and TDLD10-200 channel models. The results have been gathered in Table 2.1.

Table 2.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 400 MHz for moderate and high MCS (17, 20 and 22).

|  | CBW (MHz) / SCS (KHz) = 400/120 (264 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | PN compensa-tion | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 2-1 | 17 | TDLA10-200 | 7.9301 | No Comp | 8.9087 | - | 9.8865 | - |
|  |  |  |  | CPE | 8.5804 | - | 9.2112 | - |
|  |  |  |  | ICI, u = 1 | 8.57 | - | 8.8795 | - |
|  |  |  |  | ICI, u = 2 | 8.5951 | 0.665 | 8.7684 | 0.8383 |
|  |  | TDLA10-650 | 8.4577 | No Comp | 9.3352 | - | 10.4423 | - |
|  |  |  |  | CPE | 9.0403 | - | 9.619 | - |
|  |  |  |  | ICI, u = 1 | 9.0552 | - | 9.3052 | - |
|  |  |  |  | ICI, u = 2 | 9.0435 | 0.5858 | 9.2243 | 0.7666 |
|  |  | TDLA20-200 | 7.906 | No Comp | 8.9752 | - | 9.9191 | - |
|  |  |  |  | CPE | 8.513 | - | 9.2073 | - |
|  |  |  |  | ICI, u = 1 | 8.5102 | - | 8.8428 | - |
|  |  |  |  | ICI, u = 2 | 8.5233 | 0.6173 | 8.733 | 0.827 |
|  |  | TDLA20-650 | 8.3661 | No Comp | 9.3318 | - | 10.4908 | - |
|  |  |  |  | CPE | 9.00 | - | 9.6013 | - |
|  |  |  |  | ICI, u = 1 | 8.9847 | - | 9.2641 | - |
|  |  |  |  | ICI, u = 2 | 9.0095 | 0.6434 | 9.1617 | 0.7956 |
|  |  | TDLD10-200 | 4.0634 | No Comp | 4.9596 | - | 5.7799 | - |
|  |  |  |  | CPE | 4.5691 | - | 5.2208 | - |
|  |  |  |  | ICI, u = 1 | 4.5581 | - | 4.862 | - |
|  |  |  |  | ICI, u = 2 | 4.5932 | 0.5298 | 4.7291 | 0.6657 |
| Test 2-2 | 20 | TDLA10-200 | 10.4857 | No Comp | 12.5289 | - | 16.4167 | - |
|  |  |  |  | CPE | 11.4961 | 1.0104 | 13.2055 | - |
|  |  |  |  | ICI, u = 1 | 11.3993 | - | 12.119 | - |
|  |  |  |  | ICI, u = 2 | 11.3779 | 0.8922 | 11.7061 | 1.2204 |
|  |  | TDLA10-650 | 11.0328 | No Comp | 13.0253 | - | 17.5259 | - |
|  |  |  |  | CPE | 12.0655 | 1.0327 | 13.568 | - |
|  |  |  |  | ICI, u = 1 | 11.9287 | - | 12.5591 | - |
|  |  |  |  | ICI, u = 2 | 11.9099 | 0.8771 | 12.2034 | 1.1706 |
|  |  | TDLA20-200 | 10.5014 | No Comp | 12.6564 | - | 16.9043 | - |
|  |  |  |  | CPE | 11.5499 | 1.0485 | 13.3562 | - |
|  |  |  |  | ICI, u = 1 | 11.4526 | - | 12.1851 | - |
|  |  |  |  | ICI, u = 2 | 11.4352 | 0.9338 | 11.7418 | 1.2404 |
|  |  | TDLA20-650 | 11.0435 | No Comp | 13.0079 | - | NAN | - |
|  |  |  |  | CPE | 12.0593 | 1.0158 | 13.6736 | - |
|  |  |  |  | ICI, u = 1 | 11.9747 | - | 12.5882 | - |
|  |  |  |  | ICI, u = 2 | 11.9332 | 0.8897 | 12.2432 | 1.1997 |
|  |  | TDLD10-200 | 6.3135 | No Comp | 8.1687 | - | 10.6743 | - |
|  |  |  |  | CPE | 7.2806 | 0.9671 | 8.7606 | - |
|  |  |  |  | ICI, u = 1 | 7.2132 | - | 7.8459 | - |
|  |  |  |  | ICI, u = 2 | 7.1985 | 0.885 | 7.5052 | 1.1917 |
| Test 2-3 | 22 | TDLA10-200 | 12.7996 | No Comp | 17.1277 | - | NAN | - |
|  |  |  |  | CPE | 14.4798 | - | 19.0793 | - |
|  |  |  |  | ICI, u = 1 | 14.208 | - | 15.6796 | - |
|  |  |  |  | ICI, u = 2 | 14.106 | 1.3064 | 14.7316 | 1.932 |
|  |  | TDLA10-650 | 13.3778 | No Comp | 17.5385 | - | NAN | - |
|  |  |  |  | CPE | 15.0634 | - | 19.5067 | - |
|  |  |  |  | ICI, u = 1 | 14.7978 | - | 16.0996 | - |
|  |  |  |  | ICI, u = 2 | 14.6798 | 1.302 | 15.2649 | 1.8871 |
|  |  | TDLA20-200 | 12.7935 | No Comp | 17.1544 | - | NAN | - |
|  |  |  |  | CPE | 14.5402 | - | 19.8919 | - |
|  |  |  |  | ICI, u = 1 | 14.2795 | - | 15.8701 | - |
|  |  |  |  | ICI, u = 2 | 14.1842 | 1.3907 | 14.913 | 2.1195 |
|  |  | TDLA20-650 | 13.3122 | No Comp | 17.6844 | - | NAN | - |
|  |  |  |  | CPE | 15.0393 | - | 20.2222 | - |
|  |  |  |  | ICI, u = 1 | 14.7373 | - | 16.202 | - |
|  |  |  |  | ICI, u = 2 | 14.6418 | 1.3296 | 15.2467 | 1.9345 |
|  |  | TDLD10-200 | 8.2215 | No Comp | 11.8651 | - | NAN | - |
|  |  |  |  | CPE | 9.8837 | - | 13.7187 | - |
|  |  |  |  | ICI, u = 1 | 9.6182 | - | 11.0052 | - |
|  |  |  |  | ICI, u = 2 | 9.5342 | 1.3127 | 10.2682 | 2.0467 |

In this case, when 264 RBs have been used, Table 2.1 shows that the ICI compensation boosted the performance by reducing the needed SNR (dB) to achieve 70% of the peak throughput.

Observation 5: Referring to Table 2.1, when considering SCS/CBW = 120 KHz/400 MHz and moderate/high MCS (17, 20 and 22), D1 remains below 1 dB for MCS = 17 and 20, showing that MCS 20 became only achievable under TDLA10/20 – 200/650 channel models when considering the ICI compensation (using PN model Set 1). However, neither of these high MCS are testable using 264 PRBs (since the reference SNR0 threshold is at [-0.6] dB).

## 2.2.3 SCS/CBW = 480 KHz/400 MHz results

Following same arguments as in Section 2.2.1 (since the PRB number is 66), the results have been gathered in Tables 3.1 and 3.2. It is worth mentioning here that we limit the max Doppler frequency to 200 Hz, since 480 KHz SCS is mainly for indoor environments. It is worth that the Reference SNR0 threshold is at [-0.6] dB.

Table 3.1 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for low MCS (4 and 13).

|  | CBW (MHz) / SCS (KHz) = 400/480 (66 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | PN compensa-tion | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-1 | 4 | TDLA10-200 | -3.3065 | No Comp | -3.1847 | - | -3.1177 | - |
|  |  |  |  | CPE | -3.0909 | 0.2156 | -3.0569 | 0.2496 |
|  |  | TDLA20-200 | -3.0668 | No Comp | -2.9629 | - | -2.8863 | - |
|  |  |  |  | CPE | -2.8579 | 0.2089 | -2.8648 | 0.202 |
|  |  | TDLD10-200 | -7.2689 | No Comp | -7.1634. | - | -7.0631 | - |
|  |  |  |  | CPE | -7.0474 | 0.2215 | -7.0448 | 0.2241 |
| Test 1-2 | 13 | TDLA10-200 | 4.4287 | No Comp | 4.8377 | - | 5.2791 | - |
|  |  |  |  | CPE | 4.7345 | 0.3058 | 4.7718 | 0.3431 |
|  |  | TDLA20-200 | 4.6302 | No Comp | 5.0419 | - | 5.4957 | - |
|  |  |  |  | CPE | 4.9283 | 0.2981 | 4.9812 | 0.351 |
|  |  | TDLD10-200 | 0.5299 | No Comp | 0.9066 | - | 1.3013 | - |
|  |  |  |  | CPE | 0.8204 | 0.2905 | 0.8781 | 0.3482 |

Observation 6: Considering SCS 480 KHz/ CBW 400 MHz leads to the same conclusion as per Observation 3, that CPE compensation only is enough under TDLA10/20-200 and TDLD10-200 channel models for low MCS (4 and possibly 9).

Table 3.2 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for moderate and high MCS (17, 20 and 22).

|  | CBW (MHz) / SCS (KHz) = 400/480 (66 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | PN compensa-tion | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-3 | 17 | TDLA10-200 | 7.6773 | No Comp | 8.5722 | - | 9.6883 | - |
|  |  |  |  | CPE | 8.2322 | 0.5549 | 8.3948 | 0.7175 |
|  |  | TDLA20-200 | 7.7596 | No Comp | 8.6748 | - | 9.8495 | - |
|  |  |  |  | CPE | 8.3114 | 0.5518 | 8.4691 | 0.7095 |
|  |  | TDLD10-200 | 3.9846 | No Comp | 4.6926 | - | 5.5218 | - |
|  |  |  |  | CPE | 4.4474 | 0.4628 | 4.5756 | 0.591 |
| Test 1-4 | 20 | TDLA10-200 | 10.0518 | No Comp | 11.8573 | - | 15.6983 | - |
|  |  |  |  | CPE | 10.9436 | 0.8918 | 11.3527 | 1.3009 |
|  |  | TDLA20-200 | 10.0697 | No Comp | 12.0252 | - | NAN | - |
|  |  |  |  | CPE | 11.0418 | 0.9721 | 11.4411 | 1.3714 |
|  |  | TDLD10-200 | 6.2438 | No Comp | 7.7153 | - | 17.8407 | - |
|  |  |  |  | CPE | 7.0882 | 0.8444 | 7.3819 | 1.1381 |
| Test 1-5 | 22 | TDLA10-200 | 12.11 | No Comp | 15.8044 | - | NAN | - |
|  |  |  |  | CPE | 13.6498 | 1.5398 | 14.5723 | 2.4623 |
|  |  | TDLA20-200 | 12.1732 | No Comp | 16.2242 | - | NAN | - |
|  |  |  |  | CPE | 13.7935 | 1.6203 | 14.7736 | 2.6004 |
|  |  | TDLD10-200 | 8.2423 | No Comp | 11.1815 | - | NAN | - |
|  |  |  |  | CPE | 9.6329 | 1.3906 | 10.3107 | 2.0684 |

Observation 7: Referring to Table 3.2, when considering SCS/CBW = 480 KHz/400 MHz and moderate/high MCS (17, 20 and 22), same conclusions as in Observation 4 can be drawn that D1 remains below 1 dB for MCS = 17 and 20, showing that MCS 20 is achievable under TDLA10/20-200 and TDLD10-200 channel models, while considering the CPE compensation only (considering PN model Set 1). However, these reference SNRs are not testable. The PRB allocation should be reduced to test higher MCS (13 and 17).

Observation 8: As a general conclusion, when considering a low PRB number (66), we could achieve MCS 13 and 17 when considering CPE compensation only under TDLD10-200 channel model.

## 2.2.4 SCS/CBW = 480 KHz/800 MHz results

Following same arguments as in Section 2.2.2, for a higher PRB number, we assess the ICI compensation effect on the achievable results when considering moderate or high MCS.

Table 4.1 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 800 MHz for moderate and high MCS (17, 20 and 22).

|  | CBW (MHz) / SCS (KHz) = 800/480 (124 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | PN compensa-tion | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 4-1 | 17 | TDLA10-200 | 7.612 | No Comp | 8.7023 | - | 10.403 | - |
|  |  |  |  | CPE | 8.1701 | - | 8.405 | - |
|  |  |  |  | ICI, u = 1 | 8.194 | - | 8.2179 | - |
|  |  |  |  | ICI, u = 2 | G8.295 | 0.683 | 8.3022 | 0.6902 |
|  |  | TDLA20-200 | 7.8105 | No Comp | 8.9699 | - | 10.7957 | - |
|  |  |  |  | CPE | 8.4173 | - | 8.6705 | - |
|  |  |  |  | ICI, u = 1 | 8.4617 | - | 8.4663 | - |
|  |  |  |  | ICI, u = 2 | 8.5531 | 0.7426 | 8.5559 | 0.7454 |
|  |  | TDLD10-200 | 3.9347 | No Comp | 4.8243 | - | 5.9852 | - |
|  |  |  |  | CPE | 4.4343 | - | 4.6246 | - |
|  |  |  |  | ICI, u = 1 | 4.4718 | - | 4.494 | - |
|  |  |  |  | ICI, u = 2 | 4.5712 | 0.6365 | 4.586 | 0.6513 |
| Test 4-2 | 20 | TDLA10-200 | 9.992 | No Comp | 12.253 | - | NAN | - |
|  |  |  |  | CPE | 10.9991 | 1.0071 | 11.5701 | - |
|  |  |  |  | ICI, u = 1 | 10.8459 | - | 10.8877 | - |
|  |  |  |  | ICI, u = 2 | 10.8845 | 0.8925 | 10.9123 | 0.9203 |
|  |  | TDLA20-200 | 10.1913 | No Comp | 12.6479 | - | NAN | - |
|  |  |  |  | CPE | 11.2364 | 1.0451 | 11.8758 | - |
|  |  |  |  | ICI, u = 1 | 11.0832 | - | 11.1342 | - |
|  |  |  |  | ICI, u = 2 | 11.1084 | 0.9171 | 11.1599 | 0.9686 |
|  |  | TDLD10-200 | 6.2089 | No Comp | 8.0052 | - | NAN | - |
|  |  |  |  | CPE | 7.1008 | - | 7.5273 | - |
|  |  |  |  | ICI, u = 1 | 7.0182 | - | 7.0763 | - |
|  |  |  |  | ICI, u = 2 | 7.0555 | 0.8466 | 7.0871 | 0.8782 |
| Test 4-3 | 22 | TDLA10-200 | 12.2911 | No Comp | 17.2905 | - | NAN | - |
|  |  |  |  | CPE | 14.1182 | - | 15.5237 | - |
|  |  |  |  | ICI, u = 1 | 13.5965 | - | 13.7359 | - |
|  |  |  |  | ICI, u = 2 | 13.5134 | 1.2223 | 13.6124 | 1.3213 |
|  |  | TDLA20-200 | 12.4583 | No Comp | 18.1485 | - | NAN | - |
|  |  |  |  | CPE | 14.3695 | - | 15.9886 | - |
|  |  |  |  | ICI, u = 1 | 13.8517 | - | 14.0058 | - |
|  |  |  |  | ICI, u = 2 | 13.7312 | 1.2729 | 13.8684 | 1.4101 |
|  |  | TDLD10-200 | 8.2942 | No Comp | 12.1936 | - | NAN | - |
|  |  |  |  | CPE | 9.9512 | - | 10.9943 | - |
|  |  |  |  | ICI, u = 1 | 9.5649 | - | 9.6372 | - |
|  |  |  |  | ICI, u = 2 | 9.4974 | 1.2032 | 9.5661 | 1.2719 |

In this case, when 124 RBs have been used, Table 4.1 shows that the ICI compensation boosted the performance by reducing the needed SNR (dB) to achieve 70% of the peak throughput under TDLA10/20-200 channel models.

Observation 9: Referring to Table 4.1, when considering SCS/CBW = 480 KHz/800 MHz and moderate/high MCS (17, 20 and 22), both D1 and D2 remain below 1 dB for MCS = 17 and 20, showing that MCS 20 became only achievable under TDLA10/20 – 200 channel models when considering the ICI compensation. However, all these reference SNRs could not be tested.

## 2.2.5 Reference SNRs at 30% of the peak Throughput

In this section, we evaluate the reference SNRs at 30% of the peak throughput for both 120 KHz and 480 KHz SCS considering low PRB number (66) and MCS 13. For 120 KHz SCS, we consider TDLA10/20-200/650 channel models, while TDLA10/20-200 and TDLD10-200 are considered for 480 KHz SCS. The results have been gathered in Tables 5.1 and 5.2 where CPE compensation only was used.

Table 5.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for MCS 13.

|  | CBW (MHz) / SCS (KHz) = 100/120 (66 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 30% Max Throughput | PN compensa-tion | SNR1 (dB) @ 30% Max Throughput | D1 (dB) | SNR2 (dB) @ 30% Max Throughput | D2 (dB) |
| Test 5-1 | 13 | TDLA10-200 | -1.0809 | No Comp | -0.9667 | - | - | - |
|  |  |  |  | CPE | -0.8869 | 0.194 | -0.8324 | 0.2485 |
|  |  | TDLA10-650 | -1.6556 | No Comp | -1.4974 | - | - | - |
|  |  |  |  | CPE | -1.4115 | 0.2441 | -1.3575 | 0.2981 |
|  |  | TDLA20-200 | -1.1303 | No Comp | -0.9820 | - |  | - |
|  |  |  |  | CPE | -0.9177 | 0.2126 | -0.8309 | 0.2994 |
|  |  | TDLA20-650 | -1.5545 | No Comp | -1.3971 | - | - | - |
|  |  |  |  | CPE | -1.3389 | 0.2156 | -1.2656 | 0.2889 |

Table 5.2 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for MCS 13.

|  | CBW (MHz) / SCS (KHz) = 400/480 (66 PRBs) |  |  |  | PN @ RX (UE) only |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS | Propagation condition | NO PN SNR0 (dB) @ 30% Max Throughput | PN compensa-tion | SNR1 (dB) @ 30% Max Throughput | D1 (dB) | SNR2 (dB) @ 30% Max Throughput | D2 (dB) |
| Test 5-2 | 13 | TDLA10-200 | -0.9102 | No Comp | -0.7603 | - | - | - |
|  |  |  |  | CPE | -0.6890 | 0.2212 | -0.6795 | 0.2307 |
|  |  | TDLA20-200 | -0.8621 | No Comp | -0.6920 | - | - | - |
|  |  |  |  | CPE | -0.6525 | 0.2096 | -0.6265 | 0.2356 |
|  |  | TDLD10-200 | -4.9622 | No Comp | -4.8423 | - | - | - |
|  |  |  |  | CPE | -4.7857 | 0.1765 | -4.7769 | 0.1853 |

Observation 10: Referring to Tables 5.1 and 5.2, when considering performance metric at 30% of the peak throughput, both D1 and D2 remain below 1 dB when considering low PRB number and low MCS under TDLA10/20-200/650 and TDLD10-200 channel models, showing that the CPE compensation is enough.

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Low MCS is less susceptible to phase rotation error. Consequently, only CPE compensation will be applied.

Observation 2: For moderate/high MCS and large PRB number allocation, the companies are encouraged to consider ICI compensation techniques while keeping same Rel-15 PTRS pattern.

Observation 3: Referring to Table 1.1, when considering SCS/CBW = 120 KHz/100 MHz and low MCS (4 and 13), both D1 and D2 remain below 1 dB showing that the CPE compensation is enough when considering TDLA10/20 – 200/650.

Observation 4: Referring to Table 1.2, when considering SCS/CBW = 120 KHz/100 MHz and moderate/high MCS (17, 20 and 22), D1 remains below 1 dB for MCS = 17 and 20, showing that MCS 20 is achievable under TDLA10/20-200 and TDLD10-200 channel models, while considering the CPE compensation only (considering PN model Set 1).

Observation 5: Referring to Table 2.1, when considering SCS/CBW = 120 KHz/400 MHz and moderate/high MCS (17, 20 and 22), D1 remains below 1 dB for MCS = 17 and 20, showing that MCS 20 became only achievable under TDLA10/20 – 200/650 channel models when considering the ICI compensation (using PN model Set 1). However, neither of these high MCS are testable using 264 PRBs (since the reference SNR0 threshold is at [-0.6] dB).

Observation 6: Considering SCS 480 KHz/ CBW 400 MHz leads to the same conclusion as per Observation 3, that CPE compensation only is enough under TDLA10/20-200 and TDLD10-200 channel models for low MCS (4 and possibly 9).

Observation 7: Referring to Table 3.2, when considering SCS/CBW = 480 KHz/400 MHz and moderate/high MCS (17, 20 and 22), same conclusions as in Observation 4 can be drawn that D1 remains below 1 dB for MCS = 17 and 20, showing that MCS 20 is achievable under TDLA10/20-200 and TDLD10-200 channel models, while considering the CPE compensation only (considering PN model Set 1). However, these reference SNRs are not testable. The PRB allocation should be reduced to test higher MCS (13 and 17).

Observation 8: As a general conclusion, when considering a low PRB number (66), we could achieve MCS 13 and 17 when considering CPE compensation only under TDLD10-200 channel model.

Observation 9: Referring to Table 4.1, when considering SCS/CBW = 480 KHz/800 MHz and moderate/high MCS (17, 20 and 22), both D1 and D2 remain below 1 dB for MCS = 17 and 20, showing that MCS 20 became only achievable under TDLA10/20 – 200 channel models when considering the ICI compensation. However, all these reference SNRs could not be tested.

Observation 10: Referring to Tables 5.1 and 5.2, when considering performance metric at 30% of the peak throughput, both D1 and D2 remain below 1 dB when considering low PRB number and low MCS under TDLA10/20-200/650 and TDLD10-200 channel models, showing that the CPE compensation is enough.

# References

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

- R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson.

# Appendix	Simulation assumption

## A.1	Test Parameters as per WF [1]

Issue 2-1-14: Test Cases

Initial list of test cases for simulation results in the next meeting is given below as a starting point. Agree on final list of test cases in the next meeting based on simulation results, considering testability and minimize the number of test cases.

Below list was used for initial simulation purpose to align the simulation assumption.

RAN4 not deicide the final test case list which subject to further discussion and it’s not precluded to specify requirements for the configurations which not captured in current list.

| CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | % of peak thpt |
| --- | --- | --- | --- | --- |
| 100/120 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 100/120 | MCS4/Rank1 | TDLA10-650 | 2x2 ULA Low | 70 |
| 100/120 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 100/120 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low | 70 |
| 100/120 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS4/Rank1 | TDLA10-650 | 2x2 ULA Low | 70 |
| 400/480 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | 70 |
| 400/120 (Note 2) | MCS20/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| Note 1: Interested companies can provide simulation results for other cases not included in above table, as needed. Note 2: This case was added for PN impact and test ability analysis. Note 3: MCS level not decided yet which can be further updated. Note 4: 960kHz SCS not decided yet which can be added pending on further discussion |  |  |  |  |

Issue 2-1-15: Other Parameters

Initial simulation assumptions for parameters not discussed above are provided in the below table, so that companies can bring simulation results in the next meeting. Those assumptions can be revisited in the next meeting, if any issues are found.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| Other Parameters | Same as FR2-1 |