3GPP TSG-RAN WG4 Meeting #104-e	R4-2212573

Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	9.14.8.2.2

Source:	Ericsson

Title:	Simulation results for PDCCH and PBCH demodulation in FR2-2

Document for:	Information

# 1	Introduction

In this paper, we present the simulation results on the PDCCH and PBCH demodulation performance in the frequency range 52.6 GHz to 71 GHz. Following the WF [1], we carried out our simulations using the agreed propagation environments during RAN4 meeting #103-e, given by

Issue 1-2-1/1-2-2: TDL Channel Model and RMS Delay Spread - RAN4 will specify demodulation requirements covering TDL-A10 (baseline) and TDL-D10 (for specific test cases) - FFS whether TDL-A20 needed or not Issue 1-2-3: Max Doppler Frequency - TDL-D with 200Hz Max doppler - TDL-A channel: - Option 1: 200Hz - Option 2: 650Hz - Option 3: both 200Hz, and 650Hz

# 2	PDCCH Simulation Results

## 2.1	Simulation setup

As per WF [1], and for the sake of completeness, we carried out simulations using TDLA10/20-200/650 and TDLD10-200 channel models. The simulation parameters are given in the Table below

Test Parameters (reference from TS 38.101-4 Table 7.3.2.2-1)

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | TBD |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Furthermore, we used the following general parameters:

- Numerologies:

- SCS = 120 KHz, CBW = 100 MHz (66 PRB) and 400 MHz (264 PRB).

- SCS = 480 KHz, CBW = 400 MHz (66 PRB) and 800 MHz (124 PRB).

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 200 Hz), DMRS 1+1

- UE speed at 10 Km/h (f_D = 650 Hz), DMRS 1+1

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%

## 2.2	Simulation results

In this section, we present the PDCCH performance in terms of the probability of miss-detection of the downlink scheduling grant (Pm-dsg) as function of SNR (dB). The PDCCH performance metric is defined as the needed SNR (dB) for a Pm-dsg (BLER) at 1%. In the sequel, we provide the obtained results summarized in the following tables. Moreover, and for the sake of the presentation quality, we moved the figures in the Appendix A.1.

SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | 1x2 ULA Low | 1_0 | 40 | TDLA10-200 | 6.3171 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | 6.2419 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 6.2825 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | 6.3412 |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 3.2834 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | 3.3274 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 2.5653 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | 2.5659 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.4743 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | 1.4583 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 1.1823 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | 1.2222 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | 1_0 | 40 | TDLA10-200 | -1.6591 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | -1.7265 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | -2.7115 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | -2.776 |

Observation 1: For SCS 120 KHZ/CBW 100 MHz, we kept same test setting as in FR2-1 while considering TDLA10/20-200/650 channel models.

SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 240 | 1 | 2 | 1x2 ULA Low | 1_0 | 40 | TDLA10-200 | 4.2353 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | 4.4445 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 4.1656 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | 4.2641 |
| Test 1-2 | 2 | 6 | 240 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.5302 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | 1.5925 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 1.1377 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | 1.2366 |
| Test 1-3 | 3 | 2 | 240 | 1 | 8 | 2x2 ULA Low | 1_1 | 56 | TDLA10-200 | -1.1451 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | -1.2714 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | -1.4157 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | -1.3319 |
| Test 1-4 | 3 | 2 | 240 | 1 | 16 | 2x2 ULA Low | 1_1 | 56 | TDLA10-200 | -3.8889 |
|  |  |  |  |  |  |  |  |  | TDLA10-650 | -3.9608 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | -4.0953 |
|  |  |  |  |  |  |  |  |  | TDLA20-650 | -4.0973 |

Observation 2: For SCS 120 KHz with max CBW such as 400 MHz, we proposed PDCCH modulation performance where 240 PRB have been allocated to the CORESET and all the aggregation levels have been limited to 1 OFDM symbol duration under TDLA10/20-200/650 channel models.

SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | 1x2 ULA Low | 1_0 | 40 | TDLA10-200 | 5.8516 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 6.3931 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | 3.4018 |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.6345 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 2.2419 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -0.5096 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | 1_1 | 56 | TDLA10-200 | 0.7228 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | 0.7353 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | 0.5403 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | 1_0 | 40 | TDLA10-200 | -3.9848 |
|  |  |  |  |  |  |  |  |  | TDLA20-200 | -4.0942 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -5.5854 |

Observation 3: For SCS 480 KHz with min CBW such as 400 MHz, we proposed PDCCH modulation performance where 60 PRB have been allocated to the CORESET and kept same antenna configuration and DCI format as for 120 KHz/100 MHz, under TDLA10/20-200 and TDLD10-200 channel models. Herein,

- The aggregation level 8 has been allocated 1 OFDM symbol duration.

- The aggregation level 16 has been allocated 2 OFDM symbol duration.

# 3	PBCH Simulation Results

## 3.1	Simulation setup

As per WF [1], we carried out simulations under TDLA10-200/650, TDLA20-200 and TDLD10-200 channel models. Furthermore, we considered the following table where it is worth noting that we kept same target coding rate for both numerologies, 120 KHz SCS and 480 KHz SCS.

Table A.3.4.2-1: PBCH Reference Channel

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channels |  | R.PBCH.5 | R.PBCH.6 |
| SS/PBCH block subcarrier spacing | KHz | 120 | 240 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 56/864 | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 | 24 |

## 3.2    Simulation results

In this section, we present the PBCH performance in terms of the probability of miss-detection of the PBCH (Pm-bch), given by

Pm-bch=1-AB

where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs, as function of SNR (dB). The Pm-bch is derived with the assumption that UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms). Therefore, we have 4 transmissions for each MIB as per the following parameters’ table.

Table (TS 38.101-4 7.4.2.2-1): Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 |  |  |

Observation 4: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

The following tables gather the obtained results.

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz | 1x2 Low | TDLA10-200 | -6.3851 | unknown |
|  |  |  | TDLA10-650 | -6.3641 |  |
|  |  |  | TDLA20-200 | -6.7049 |  |
|  |  |  | TDLD10-200 | -9.9231 |  |
| 1-2 |  |  | TDLA10-200 | -9.4371 | known |
|  |  |  | TDLA10-650 | -9.4878 |  |
|  |  |  | TDLA20-200 | -9.353 |  |
|  |  |  | TDLD10-200 | -10.7277 |  |

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz | 1x2 Low | TDLA10-200 | -6.2382 | unknown |
|  |  |  | TDLA10-650 | -6.4688 |  |
|  |  |  | TDLA20-200 | -5.6525 |  |
|  |  |  | TDLD10-200 | -10.039 |  |
| 1-2 |  |  | TDLA10-200 | -9.0228 | known |
|  |  |  | TDLA10-650 | -8.9199 |  |
|  |  |  | TDLA20-200 | -8.7689 |  |
|  |  |  | TDLD10-200 | -10.72 |  |

Observation 5: The reference SNRs for both cases, whether SS/PBCH index is known or not, have been provided. These values need to be confirmed in term of testability.

# 6	Summary

In this paper, we carried out simulations for PDCCH and PBCH in FR2-2. Here we propose some observations that need further discussions:

Observation 1: For SCS 120 KHZ/CBW 100 MHz, we kept same test setting as in FR2-1 while considering TDLA10/20-200/650 channel models.

Observation 2: For SCS 120 KHz with max CBW such as 400 MHz, we proposed PDCCH modulation performance where 240 PRB have been allocated to the CORESET and all the aggregation levels have been limited to 1 OFDM symbol duration under TDLA10/20-200/650 channel models.

Observation 3: For SCS 480 KHz with min CBW such as 400 MHz, we proposed PDCCH modulation performance where 60 PRB have been allocated to the CORESET and kept same antenna configuration and DCI format as for 120 KHz/100 MHz, under TDLA10/20-200 and TDLD10-200 channel models. Herein,

- The aggregation level 8 has been allocated 1 OFDM symbol duration.

- The aggregation level 16 has been allocated 2 OFDM symbol duration.

Observation 4: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

Observation 5: The reference SNRs for both cases, whether SS/PBCH index is known or not, have been provided. These values need to be confirmed in term of testability.

# References

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm.

# Appendix	Simulation assumption

## A.1	PDCCH Performance Results

(a) AL2/1 OFDM Symbol                      (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/2 OFDM Symbols

Fig 1 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (120 KHz, 100 MHz) in {TDLA 10-200, TDLA 10-650, TDLA 20-200, TDLD 20-650} channels, where 60 RBs are allocated for the CORESET.

(a) AL2/1 OFDM Symbol                      (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/1 OFDM Symbol

Fig 2 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (120 KHz, 400 MHz) in {TDLA 10-200, TDLA 10-650, TDLA 10-200, TDLA 20-650} channels, where 240 RBs are allocated for the CORESET. The CORESET duration is 1 OFDM symbol for all aggregation levels.

(a) AL2/1 OFDM Symbols                        (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/2 OFDM Symbols

Fig 3 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (480 KHz, 400 MHz) in {TDLA 10-200, TDLD 10-200, TDLA 20-200} channels, where 60 RBs are allocated for the CORESET.

## A.2	PBCH Performance Results

(a) SS/PBCH Block index unknown                  (b) SS/PBCH Block index Known

Fig 4 – BLER (4 Transmissions) as function of SNR (dB) for PBCH using SCS 120 KHz in {TDLA 10-200, TDLA 10-650, TDLA 20-200, TDLD 10-200} channels.

(a) SS/PBCH Block index unknown                  (b) SS/PBCH Block index Known

Fig 5 – BLER (4 Transmissions) as function of SNR (dB) for PBCH using SCS 480 KHz in {TDLA 10-200, TDLD 10-200, TDLA 10-650, TDLD 10-650} channels.