3GPP TSG-RAN WG4 Meeting #104-e	R4-2212633

Electronic Meeting, August 15 – August 26, 2022

Source:	ZTE Corporation

Title:	Discussion on conformance test of IAB

Agenda Item:	9.15.2.1

Document for:	Approval

- Introduction

In this contribution, we discuss the conformance test issues for simultaneous operation between child link and parent link.

- Discussion

In RAN4#103-e meeting, the timing error between IMT-MT and IMT-DU within an IAB node was discussed[1].

For case6 OTA timing error: Below option as starting point to further discuss the relevant test procedure, test set-up and MU - Measure the time error of IAB-DU and IAB-MT separately and deduce the timing error by two measurement results - Note can be added into the conformance specification to clarify other test methods not precluded if feasible.

The WF above states that the timing of IAB-MT and IAB-DU can be tested separately. While the tested signals need to be clarified.

The DMRS of PUSCH of an IAB-MT and the DMRS of PDSCH of an IAB-DU can be considered for TAE measurement. The position of DMRS for PUSCH and PDSCH can be further discussed.

Proposal 1: To consider DMRS of PUSCH of an IAB-MT and the DMRS of PDSCH of an IAB-DU within an IAB node as the tested signals for TAE tests.

With regard to test models, the following tentative agreement was reached in 103-e meeting.

Tentative agreements: Existing TM including TDD pattern of IAB-DU can be reused for IAB simultaneous operation

The following agreements were reached in RAN1#95 and RAN#104-e meeting[2][3].

# Agreements:[#95] Capture the following conclusions for the IAB SI TR: RAN1 has studied various physical layer aspects for Integrated Access and Backhaul, and from a RAN1 perspective, support for the following features and solutions has been determined to be beneficial and feasible: - Mechanisms for supporting both in-band and out-of-band relaying by multiplexing access and backhaul links in time (TDM), frequency (FDM), or space (SDM) under a per-link half-duplex constraint at the IAB node and across multiple backhaul hops, including: - Semi-static configuration for IAB node DU resources - Dynamic indication to an IAB node of the availability of soft resources for an IAB node DU - Power control/coordination for FDM/SDM of access and backhaul links Agreement[#104-e] For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA Agreement[#104-e] DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources - FFS: If additional enhancements are necessary

It is known from the agreements that both in-band and out-of-band multiplexing are to be supported. To support in-band multiplexing between IAB-MT and IAB-DU, new test models need to be considered to define the new requirement under the situation where IAB-DU and IAB-MT transmit simultaneously within a carrier.

Observation 1: Both in-band and out-of-band multiplexing within an IAB node need to be supported.

With regard to the conformance test of modulation quality and unwanted emissions, the current version of TS38.176-1/2 didn’t consider the cases when IAB-DU and IAB-MT transmit simultaneously with unbalanced power declared by manufacturer, therefore we think that applicable test model should be further discussed and clarified.

In the legacy test mode, the testing signal for DL or UL should occupy the whole channel for either IAB-DU or IAB-MT transmission, however for simultaneous transmission tests at IAB-DU and IAB-MT, frequency allocation for IAB-DU and IAB-MT should be further clarified. In figure 1, we provide a proposal on how to allocate the frequency resource for IAB-DU and IAB-MT.

Figure 1, Depiction of simultaneous transmission at IAB-DU and IAB-MT with or without power imbalance

To be more specific, for IAB-DU DL transmission, the PDCCH is transmitted in the first 2 OFDM symbols of a slot and other symbols are for IAB-DU PDSCH with RNTI = 2, PDSCH with RNTI = 0 are transmitted in all OFDM symbols, and the PUSCH for IAB-MT is transmitted in all the OFDM symbols. Power imbalance between IAB-DU and IAB-MT could be left up the declarations and detailed test model could be found as Table 1.

Table 1, Test models for the case when IAB-DU and IAB-MT transmit simultaneously with power imbalance

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs for IAB-DU | x=50% |
| # of QPSK PDSCH RBGs for IAB-DU which are boosted | , where P is determined from table 5.1.2.2.1-1 from TS 38.214 [24], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs for IAB-DU which are boosted |  |
| Level of boosting (dB) | Up to declaration |
| # of QPSK PUSCH PRBsfor IAB-MT which are deboosted | NRB-3-PNRBG |
| Locations of PUSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) |  |
| Modulation of PDSCH PRBs with | QPSK |
| Modulation of PUSCH PRBs with | QPSK |

Proposal 2: To consider the following power allocation and frequency resource allocation for ACLR testing when IAB-DU and IAB-MT of the same IAB-Node transmit simultaneously.

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs for IAB-DU | x=50% |
| # of QPSK PDSCH RBGs for IAB-DU which are boosted | , where P is determined from table 5.1.2.2.1-1 from TS 38.214 [24], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs for IAB-DU which are boosted |  |
| Level of boosting (dB) | Up to declaration |
| # of QPSK PUSCH PRBsfor IAB-MT which are deboosted | NRB-3-PNRBG |
| Locations of PUSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) |  |
| Modulation of PDSCH PRBs with | QPSK |
| Modulation of PUSCH PRBs with | QPSK |

It is should be explicitly described in the test model that IAB-DU and IAB-MT transmit simultaneously when measuring EVM and ACLR.

For EVM conformance testing when IAB-DU and IAB-MT of the same IAB-Node transmit simultaneously, the modulation order in the above table could be updated correspondingly.

- Conclusion

Based on the analysis, the following observation and proposals are given:

Observation 1: Both in-band and out-of-band multiplexing within an IAB node need to be supported.

Proposal 1: To consider DMRS of PUSCH of an IAB-MT and the DMRS of PDSCH of an IAB-DU within an IAB node as the tested signals for TAE tests.

Proposal 2: To consider the following power allocation and frequency resource allocation for ACLR testing when IAB-DU and IAB-MT of the same IAB-Node transmit simultaneously.

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs for IAB-DU | x=50% |
| # of QPSK PDSCH RBGs for IAB-DU which are boosted | , where P is determined from table 5.1.2.2.1-1 from TS 38.214 [24], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs for IAB-DU which are boosted |  |
| Level of boosting (dB) | Up to declaration |
| # of QPSK PUSCH PRBsfor IAB-MT which are deboosted | NRB-3-PNRBG |
| Locations of PUSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) |  |
| Modulation of PDSCH PRBs with | QPSK |
| Modulation of PUSCH PRBs with | QPSK |

- Reference

R4-2210643

Chairman's Notes RAN1 95 final

Chairman's Notes RAN1#104-e final