3GPP TSG-RAN WG4 Meeting #104-e	R4-2212842

Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.115-2 clause 6.7 OTA Input intermodulation

Agenda item:	9.5.5.3.2	Emission requirements

Document for:	Approval

# Introduction

In this contribution we provide text proposal for radiated repeater conformance specification TS 38.115-2 for clause 6.7 OTA Input intermodulation according work split agreed during RAN4#103-e meeting in [1].

# References

[1]  R4-2211153, WF on NR FR1/FR2 repeater test specs draft rules and TP splits, CATT

<Start of TP to 38.115-1>

## 6.7	Input Intermodulation

### 6.7.1	Definition and applicability

#### 6.7.1.1	General

The input intermodulation is a measure of the capability of the repeater to inhibit the generation of interference in the passband, in the presence of interfering signals on frequencies other than the passband.

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the repeater to maintain the wanted frequency free of internally created interference.

The measurements shall apply to both uplink and downlink paths of the repeater, at maximum gain.

#### 6.7.1.2	Minimum requirements

The minimum requirement is in TS 38.106 [x] clause 7.7.2.

#### 6.7.1.3	Test purpose

The purpose of this test is to verify that the repeater meets the intermodulation characteristics requirements as specified by the minimum requirements.

#### 6.7.1.4	Method of test

##### 6.7.1.4.1	Initial conditions

Test environment: normal; see Annex B

A measurement system set-up is shown in Annex E.

##### 6.7.1.4.2	Procedure

1)	Set the repeater to maximum gain.

2)	Place the repeater with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex E.

3)  Align the test antennas and repeater both in input and output directions of the repeater.

4)  Align the repeater and test antennas so that repeater antennas are polarization matched with the test antenna(s)

5)	Configure the beam peak direction of the repeater according to declared reference beam direction pair for the appropriate beam identifier.

6)	Adjust the frequency of the input signals, either below or above the passband, so that one carrier, f1, is 1 MHz outside the channel edge frequency of the first or last channel in the passband, and the lowest order intermodulation product from the two carriers is positioned in the centre of the passband.

7)	Take the measurement of the rise of the output signal.

8)	Repeat the measurement for all supported polarizations and all specified measurement directions.

9)  Repeat the measurement for the opposite path of the repeater.

#### 6.7.1.5	Test requirements

For the parameters specified in table 6.7.1.5-1, the power in the passband shall not increase with more than [10+TT] dB at the output of the repeater as measured with 1 MHz measurement bandwidth, compared to the level obtained without interfering signals applied.

Table 6.7.1.5-1 specifies the parameters for two interfering signals, where:

-	f1 offset is the offset from the channel edge frequency of the first or last channel in the passband of the closer carrier.

-	GRX_ANT is the gain of the receive side antennas and is calculated from EIRP and TRP declaration.

Table 6.7.1.5-1: Input intermodulation requirement

| f1 offset | Interfering signal levels | Type of signals | Measurement bandwidth |
| --- | --- | --- | --- |
| 1 MHz | -53dBm – G_RX_ANT | 2 CW carriers | 1 MHz |

### 6.8	Output intermodulation

<End of TP to 38.115-1>