3GPP TSG-RAN WG4 Meeting #104-e													R4-2212847

Electronic meeting, August 15 –26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Adjacent channel coexistence in Sub Band non-overlapping Full Duplex operation

Agenda item:	11.13.2.1

Document for:	Approval

- Introduction

The new RAN1/RAN4 study item on evolution of duplex operation for NR TDD systems in unpaired spectrum was adopted [1]. The assumptions are listed as follows:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

This document concentrates on the detailed objective on studying “subband non-overlapping full duplex” and specifically the impact on adjacent channel co-existence.

# Use cases and deployment scenarios

Studying the feasibility and potential enhancements for enabling sub-band non-overlapping full duplex (SBFD) operation is one of the main objectives of the agreed RAN1/RAN4 study item in [1]. SBFD refers to the possibility of having simultaneous UL and DL transmission/reception on separate physical resource blocks (PRBs) of a same unpaired radio carrier, used today for TDD. SBFD has the promise of improving both coverage and latency as compared to what is currently achieved with traditional DL-heavy TDD in today’s deployments. The coverage improvement comes from allowing more frequent UL transmission opportunities in time-domain which is especially beneficial for cell-edge UEs as these can only transmit over a limited set of PRBs at a time due to the limited UL transmit power. Similarly, a latency improvement is also obtained not only for UL data transmissions, but also for UL signaling (e.g., HARQ feedback) which may translate into a DL latency improvement as well. Based on discussions in 3GPP RAN, the main focus of the work is in improving uplink performance.

With regards to deployment scenarios, urban macro deployments (UMa) are of the most interest due to market penetration but also the fact that UMa deployments may benefit the most from the expected coverage improvements. Additionally, SBFD may also be relevant for small cell deployments as it allows more symmetric UL-DL capacity and also lower latency e.g., required for industrial applications.

Nevertheless, the envisioned benefits of SBFD do not come for free. From RF perspective, SBFD introduces many challenges related to self, co-channel and adjacent channel interference handling as illustrated in Figure 1 and listed below reusing similar formulation from the agreement reached in RAN1#109e:

- gNB self-interference (SI) (#1 in Figure 1): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency.

- (inter-cell and intra-cell) UE-UE co-channel intra-subband CLI (#2 and #3 in Figure 1): CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI (#4 in Figure 1): CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier.

- (inter-cell) co-site inter-sector co-channel intra-subband CLI (not illustrated, for simplicity): CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier.

- (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI (#5 in Figure 1): CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (inter-cell) co-site inter-sector co-channel inter-subband CLI (not illustrated, for simplicity): CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency.

- (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI (#6 in Figure 1): CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency.

- gNB-gNB adjacent-channel CLI (#7 in Figure 1): CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier.

- UE-UE adjacent-channel CLI (#8 in Figure 1): CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier.

Figure 1: Types of interference in SBFD deployment

Certainly, not all the interference types are equally relevant for all the deployment scenarios. The handling of the gNB self-interference (SI) can be regarded as one of the biggest challenges to be addressed in order to make SBFD feasible. SI is a problem regardless of the scenario/deployment type, but it is certainly an even larger problem for UMa deployments due to high gNB DL transmit power and the use of active antenna systems (AAS) with beamforming and MU-MIMO capabilities as further elaborated in the upcoming sections. If evaluations in the SI are not handled correctly with appropriate modeling of feasible RF implementation and interference cancellation techniques, SBFD deployed based on incorrect assumptions may in fact deteriorate the performance as compared to TDD. As this is envisioned to increase the gNB complexity, size and cost, a fair and thorough study is needed to see whether the benefits of SBFD versus static and dynamic TDD justify the increased complexity.

For small cell deployments, the simpler gNB architecture and lower RF transmit power may make the self-interference more manageable, while challenges related to UE-to-UE and inter-cell interference may also not be a such a huge problem due to similar transmit powers in UL and DL directions. Moreover, dynamic TDD was also found feasible for small cells in the Rel-16 coexistence study in [TR 38.828], therefore it should be considered as part of the baseline when doing the corresponding feasibility evaluation.

Proposal 1: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to fixed and dynamic TDD to justify the complexity of introducing support for SBFD.

In Table 1, we summarize the main scenarios of relevance for SBFD as well as possible benefits and corresponding challenges.

Table 1: Scenarios for SBFD

| Scenario | Benefits of SBFD | Challenges to be addressed |
| --- | --- | --- |
| Urban Macro (UMa) or Dense Urban Macro (DU) Applicable frequency ranges: FR1 (UMa and DU), FR2 (DU) ISD 200-700 meters Number of TxRUs: 64 (FR1), 2-4 (FR2) Tx power per TxRU: ~37 dBm (FR1), ~23-30 dBm (FR2), i.e. 26-36 dBm TRP | +++ Coverage + Latency ++ UL throughput/capacity | – – – gNB self-interference – – – gNB inter-sector co-channel interference – – gNB inter-site co-channel interference – – – gNB inter-cell adjacent-channel interference – UE-to-UE co-channel and adjacent-channel interference |
| Indoor Hotspot (InH) Applicable frequency ranges: FR1-FR2 ISD: 20 – 50 meter Number of TxRUs: 2-4 Tx power per TxRU: ~26 dBm | + Latency (vs dynamic TDD) +++ Latency (vs static TDD) ++ UL throughput/capacity (vs static TDD) | – – gNB self-interference – gNB inter-cell co-channel interference – UE-to-UE co-channel and adjacent-channel interference |

# Adjacent channel co-existence

In SBFD operation, non-overlapping PRBs are configured for DL and UL transmission during a given slot. This means that a gNB operating SBFD needs to be able to receive UL transmission at the same time as other operators, which may or may not adopt SBFD, are transmitting DL to the end-users in the cell. This results in gNB-to-gNB adjacent channel cross-link interference. Similarly, a gNB operating SBFD transmits DL signals to a SBFD UE at the same time as UEs in another cell are transmitting UL. This results in UE-to-UE adjacent channel cross-link interference. Therefore, both UE-to-UE and gNB-to-gNB adjacent channel CLI are present in SBFD operation.

Observation 1: Both gNB-to-gNB and UE-to-UE adjacent channel CLI are present in SBFD operation.

RAN4 has previously analyzed adjacent channel CLI in a co-existence study performed during the Rel-16 work item for cross-link interference and remote interference management, and the co-existence study results are captured in TR 38.828 [2]. In that study, the adjacent channel coexistence analysis was carried out for networks adopting different TDD configurations. Specifically, scenarios of fully aligned, partly aligned, and fully mis-aligned transmission directions between different networks were analyzed. This covers the cases in which there is no adjacent CLI experienced as well as cases where the BS-to-BS adjacent channel CLI and the UE-to-UE adjacent channel CLI are present. It can also be noted from the TR that the analysis was done for macro, and macro-indoor deployments as well as indoor small cells deployments. Therefore, it appears that the same deployment scenarios of interest as well as the interference types of interest for SBFD have been already covered in the Rel-16 dynamic TDD coexistence study. Any difference coming from SBFD, e.g., allocation of only 80% of the channel bandwidth for DL during SBFD slots, is minor from a co-existence perspective.

As SBFD would require new HW implementations as analyzed in companion Tdoc [3], it can be assumed that SBFD operation would be more permanent way of operating. Due to the complexities coming from the loss of reciprocity and potentially at least halving the antenna panel size for SBFD, it may be that SBFD needs to be utilized all the time to enable bridging the losses inherent from architectural changes.

Dynamic TDD, as the name says, is something that can be more easily used only when needed. Therefore, it can be expected that networks using dynamic TDD could operate also using fixed TDD, and therefore cause less interference. Therefore, it can be expected that the adjacent channel studies in TR 38.828 are at least not too pessimistic.

Given the existing analyses, meaningful differences to the deployment scenarios and/or used simulation parameters compared to TR 38.828 would need to be introduced to justify further studies for adjacent channel co-existence. In case the baseline assumptions for the SBFD coexistence studies are significantly modified as compared to Rel-16, new coexistence studies may be also required for dynamic TDD as conclusions might be different from the concluded in TR 38.828.

Observation 2: From a co-existence perspective, differences between SBFD and dynamic TDD operation are minor during non-aligned slots.

Observation 3: Due to the dynamics in dynamic TDD, it is expected that there are less mis-aligned slots when dynamic TDD is used as compared to SBFD.

Proposal 2: Unless there are meaningful differences in the deployment scenarios and/or used parameters, the conclusions done for dynamic TDD in TR 38.828 apply also for SBFD operation.

# Conclusion

In this contribution Adjacent channel co-existence in SBFD operation was discussed. Following observations and proposals were made.

Proposal 1: Sufficiently large gain under realistic assumptions should be observed from SBFD as compared to fixed and dynamic TDD to justify the complexity of introducing support for SBFD.

Observation 1: Both gNB-to-gNB and UE-to-UE adjacent channel CLI are present in SBFD operation.

Observation 2: From a co-existence perspective, differences between SBFD and dynamic TDD operation are minor during non-aligned slots.

Observation 3: Due to the dynamics in dynamic TDD, it is expected that there are less mis-aligned slots when dynamic TDD is used as compared to SBFD.

Proposal 2: Unless there are meaningful differences in the deployment scenarios and/or used parameters, the conclusions done for dynamic TDD in TR 38.828 apply also for SBFD operation.

# References

[1] RP-221352, “Study on Evolution of NR Duplex Operation”, CMCC

[2] 3GPP TR 38.828 Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR