3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212897

# Electronic Meeting, 15 August - 26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | draftCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: channel quality reporting requirements for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case of CQI reporting defined for Rel-17 NB-IoT is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of CQI reporting test for NB-IoT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CQI reporting functionality is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.14 (new), A.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## 9.14	CSI reporting of Narrowband IoT

The requirements in this sub-clause are defined based on the simulation results with UE DL Category NB2 unless otherwise stated.

### 9.14.1	CQI reporting definition under AWGN conditions

#### 9.14.1.1	Half-duplex FDD

The following requirements apply to UE Category NB2 capable of npdsch-16QAM-r17. For the parameters specified in Table 9.14.1.1-1, and using the downlink physical channels specified in C.3.6, the reported candidateRep value according to RC.33 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value - 1) shall be less than or equal to 0.1.

Table 9.14.1.1-1: Half-duplex FDD

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Bandwidth | kHz | 200 |  |
| Operation mode |  | Stand-alone |  |
| Carrier type |  | Non-anchor |  |
| Number of NRS ports |  | 1 |  |
| Propagation condition and antenna configuration |  | AWGN (1 x 1) |  |
| SNR (Note 2) | dB | 10 | 11 |
| Ior(j) | dB[mW/15kHz] | -88 | -87 |
| Noc(j) | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  | 1 |  |
| NPDCCH repetition number | subframe | 1 |  |
| Rmax (npdcch-NumRepetitions-r13) | subframe | 4 |  |
| G (nPDCCH-startSF-USS-r13) | subframe | 2 |  |
| Scheduling delay field (IDelay) |  | 0 |  |
| NRepAN (ack-NACK-NumRepetitions-r13) |  | 1 |  |
| ACK/NACK resource field |  | 0 |  |
| NPDCCH reference channel |  | R.NB.3 FDD |  |
| αoffset (npdcch-Offset-USS-r13) |  | 0 |  |
| CQI reporting period | ms | 40 |  |
| NPDSCH repetition number | subframe | 1 |  |
| Note 1: Reference measurement channel RC.33 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern NB.OP.1 FDD as described in Annex A.5.3.1. For the subframes the reference measurement channel is not scheduled, NRS is transmitted with OCNG pattern NB.OP.1 FDD. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: MAC CE Downlink Channel Quality Report Command is triggered every 40 subframes, where NPDCCH scheduling MAC CE (NPDSCH) is transmitted in subframe n. UL grant is transmitted in subframe n+24. |  |  |  |

#### 9.14.1.2	TDD

The following requirements apply to UE Category NB2 capable of npdsch-16QAM-r17. For the parameters specified in Table 9.14.1.2-1, and using the downlink physical channels specified in C.3.6, the reported candidateRep value according to RC.33 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value - 1) shall be less than or equal to 0.1.

Table 9.14.1.2-1: TDD

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| Bandwidth | kHz | 200 |  |
| Operation mode |  | Stand-alone |  |
| Uplink downlink configuration (Note 4) |  | 4 |  |
| Special subframe configuration (Note 5) |  | 4 |  |
| Carrier type |  | Non-anchor |  |
| Number of NRS ports |  | 1 |  |
| Propagation condition and antenna configuration |  | AWGN (1 x 1) |  |
| SNR (Note 2) | dB | 10 | 11 |
| Ior(j) | dB[mW/15kHz] | -88 | -87 |
| Noc(j) | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  | 1 |  |
| NPDCCH repetition number | subframe | 1 |  |
| Rmax (npdcch-NumRepetitions-r13) | subframe | 4 |  |
| G (nPDCCH-startSF-USS-r13) | subframe | 2 |  |
| Scheduling delay field (IDelay) |  | 0 |  |
| NRepAN (ack-NACK-NumRepetitions-r13) |  | 1 |  |
| ACK/NACK resource field |  | 0 |  |
| NPDCCH reference channel |  | R.NB.3 TDD |  |
| αoffset (npdcch-Offset-USS-r13) |  | 0 |  |
| CQI reporting period | ms | 80 |  |
| NPDSCH repetition number | subframe | 1 |  |
| Note 1: Reference measurement channel RC.33 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern NB.OP.1 TDD as described in Annex A.5.3.1. For the subframes the reference measurement channel is not scheduled, NRS is transmitted with OCNG pattern NB.OP.1 TDD. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: MAC CE Downlink Channel Quality Report Command is triggered every 80 subframes, where NPDCCH scheduling MAC CE (NPDSCH) is transmitted in subframe n. UL grant is transmitted in subframe n+40. Note 4: As specified in Table 4.2-2 in TS 36.211 [4]. Note 5: As specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

# A.4	CSI reference measurement channels

This section defines the DL signal applicable to the reporting of channel status information (Clause 9.2, 9.3 and 9.5).

In Table A.4-1 are specified the reference channels. Table A.4-13 specifies the mapping of CQI index to modulation coding scheme, which complies with the CQI definition specified in Section 7.2.3 of [6].

Table A.4-0: Void

Table A.4-1: CSI reference measurement channels

| RMC Name | Duplex |  | CH-BW |  | Alloc. RB-s |  | UL/DL Config |  | Alloc. SF-s |  |  | MCS Scheme |  | Nr. HARQ Proc. |  | Max. nr HARQ Trans. |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 CRS Port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.1 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.1 |  | 8 |  | 1 |  |  |
| RC.1A FDD | FDD |  | 10 |  | 50 |  |  |  |  |  |  | MCS.1A |  | 8 |  | 1 |  |  |
| RC.1 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.1 |  | 10 |  | 1 |  |  |
| RC.1A TDD | TDD |  | 20 |  | 100 |  | Note 3 |  |  |  |  | MCS.1B |  | 10 |  | 1 |  |  |
| RC.3 FDD | FDD |  | 10 |  | 6 |  | - |  |  |  |  | MCS.10 |  | 8 |  | 1 |  |  |
| RC.3 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  |  |  |  | MCS.10 |  | 10 or 7 (Note 9) |  | 1 |  |  |
| RC.4 FDD | FDD |  | 10 |  | 15 |  | - |  |  |  |  | MCS.15 |  | 8 |  | 1 |  | Note 6 |
| RC.4 TDD | TDD |  | 10 |  | 15 |  | Note 3 |  |  |  |  | MCS.15 |  | 10 |  | 1 |  | Note 6 |
| RC.5 FDD | FDD |  | 10 |  | 3 |  | - |  |  |  |  | MCS.17 |  | 8 |  | 1 |  |  |
| RC.5 TDD | TDD |  | 10 |  | 3 |  | Note 3 |  |  |  |  | MCS.17 |  | 10 |  | 1 |  |  |
| RC.14 FDD | FDD |  | 5 |  | 25 |  | - |  |  |  |  | MCS.14 |  | 8 |  | 1 |  |  |
| RC.15 FDD | FDD |  | 5 |  | 15 |  | - |  |  |  |  | MCS.15 |  | 8 |  | 1 |  | Note 6 |
| RC.16 FDD | FDD/HD-FDD |  | 10 |  | 2 |  |  |  |  |  |  | MCS.20 |  | 8 |  | 1 |  | Note 8,10 |
| RC.16 TDD | TDD |  | 10 |  | 2 |  | Note 3 |  |  |  |  | MCS.20 |  | 10 |  | 1 |  | Note 8 |
| RC.23FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 8 |  | 1 |  | Note 12, 13 |
| RC.23 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 10 |  | 1 |  | Note 12 |
| RC.25 FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 8 |  | 1 |  | Note 14, 20 |
| RC.25 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 10 |  | 1 |  | Note 12, 15 |
| RC.31 FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.40 |  | 8 |  | 1 |  | Note 12, 13 |
| RC.31 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.40 |  | 10 |  | 1 |  | Note 12 |
| RC.32 FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.41 |  | 8 |  | 1 |  | Note 12, 13 |
| RC.32 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.41 |  | 10 |  | 1 |  | Note 12 |
| RC.30 FDD | FDD |  | 10 |  | 50 |  |  |  |  |  |  | MCS.38 |  | 8 |  | 1 |  |  |
| RC.30 TDD | TDD |  | 20 |  | 100 |  |  |  |  |  |  | MCS.39 |  | 10 |  | 1 |  |  |
| 2 CRS Ports |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.2 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.2 |  | 8 |  | 1 |  |  |
| RC.2A FDD | FDD |  | 20 |  | 100 |  |  |  |  |  |  | MCS.2A |  | 8 |  | 1 |  |  |
| RC.2 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.2 |  | 10 or 7 (Note 9) |  | 1 |  |  |
| RC.4A FDD | FDD |  | 20 |  | 15 |  | - |  |  |  |  | MCS.16 |  | 8 |  | 1 |  | Note 6 |
| RC.6 FDD | FDD |  | 10 |  | 15 |  | - |  |  |  |  | MCS.16 |  | 8 |  | 1 |  | Note 6 |
| RC.6 TDD | TDD |  | 10 |  | 15 |  | Note 3 |  |  |  |  | MCS.16 |  | 7 |  | 1 |  | Note 6 |
| 4 CRS Ports |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.17 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.18 |  | 8 |  | 1 |  |  |
| RC.17 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.18 |  | 7 |  | 1 |  |  |
| RC.21 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.26 |  | 8 |  | 1 |  |  |
| RC.21 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.26 |  | 7 |  | 1 |  |  |
| 1 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.8 FDD | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS |  |  | MCS.11 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12 |  |  |  |  |  |  |
| RC.8A FDD | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS |  |  | MCS.11A |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12A |  |  |  |  |  |  |
| RC.8 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | Non CSI-RS |  |  | MCS.11 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12 |  |  |  |  |  |  |
| RC.8A TDD | TDD |  | 20 |  | 8 |  | Note 3 |  | Non CSI-RS |  |  | MCS.11B |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12B |  |  |  |  |  |  |
| RC.9 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.3 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.4 |  |  |  |  |  |  |
| RC.9A FDD | FDD |  | 20 |  | 100 |  | - |  | Non CSI-RS |  |  | MCS.3A |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.4A |  |  |  |  |  |  |
| RC.9B FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS, rank 1/2 |  |  | MCS.3 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | Non CSI-RS, rank 3/4 |  |  | MCS.30 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 1/2 |  |  | MCS.29 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 3/4 |  |  | MCS.31 |  |  |  |  |  |  |
| RC.9 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.3 |  | 7 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.4 |  |  |  |  |  |  |
| RC.9B TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS, rank 1/2 |  |  | MCS.3 |  | 7 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | Non CSI-RS, rank 3/4 |  |  | MCS.30 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 1/2 |  |  | MCS.29 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 3/4 |  |  | MCS.31 |  |  |  |  |  |  |
| 2 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.7 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.5 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.7 |  |  |  |  |  |  |
| RC.7 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.5 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 8 CSI-RS |  |  | MCS.8 |  |  |  |  |  |  |
| RC.11 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.5 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.6 |  |  |  |  |  |  |
| RC.11 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.5 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.6 |  |  |  |  |  |  |
| RC.18 FDD | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS |  |  | MCS.13 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.19 |  |  |  |  |  |  |
| RC.18 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | Non CSI-RS |  |  | MCS.13 |  | 7 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.19 |  |  |  |  |  |  |
| RC.17 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | 4 ZP-CSI-RS |  |  | MCS.21 |  | 10 |  | 1 |  |  |
| RC.18 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | 4 ZP-CSI-RS |  |  | MCS.22 |  | 10 |  | 1 |  |  |
| RC.19 TDD | TDD |  | 10 |  | 41 |  | Note3 |  | 4 ZP-CSI-RS |  |  | MCS.23 |  | 10 |  | 1 |  | Note 11 |
| RC.20 TDD | TDD |  | 10 |  | 50 |  | Note3 |  | Non CSI-RS |  |  | MCS.24 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS, 4 ZP-CSI-RS |  |  | MCS.25 |  |  |  |  |  |  |
| RC.22 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.5 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.27 |  |  |  |  |  |  |
| RC.22 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.5 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.27 |  |  |  |  |  |  |
| RC.23 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.9 |  | 10 |  | 1 |  | Rank 4 |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.32 |  |  |  |  |  |  |
| 1 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.13 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS/IM | MCS.3 |  | 8 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| RC.13 TDD |  | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS/IM | MCS.3 |  | 10 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| 2 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.10 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.5 |  | 8 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 4 CSI-RS, 1 CSI process | MCS.8 |  |  |  |  |  |  |  |
| RC.10 TDD |  | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS | MCS.5 |  | 10 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 8 CSI-RS, 1 CSI process | MCS.9 |  |  |  |  |  |  |  |
| RC.12 FDD |  | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS/IM | MCS.13 |  | 8 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| RC.12 TDD |  | TDD |  | 10 |  | 6 |  | Note 3 |  | Non CSI-RS/IM | MCS.13 |  | 10 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| Short TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.26 FDD |  | FDD |  | 10 |  | 50 |  | - |  | - | MCS.32-1 MCS.32-2 |  | 8 |  | 1 |  | Note 2 |  |
| RC.27 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.33-1 MCS.33-2 |  | 8 |  | 1 |  | Note 2 |  |
|  |  | FDD |  | 10 |  | 50 |  | - |  | 2 CSI-RS | MCS.34-1 MCS.34-2 |  | 8 |  | 1 |  | Note 2 |  |
| RC.28 FDD |  | FDD |  | 10 |  | 50 |  | - |  | - | MCS.35-1 MCS.35-2 MCS.35-3 MCS.35-4 MCS.35-5 |  | 16 |  | 1 |  | Note 2 Note 17 Note 18 |  |
| RC.29 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.36-1 MCS.36-2 MCS.36-3 MCS.36-4 MCS.36-5 |  | 16 |  | 1 |  | Note 2 Note 17 Note 19 |  |
|  |  | FDD |  | 10 |  | 50 |  | - |  | 2 CSI-RS | MCS.37-1 MCS.37-2 MCS.37-3 MCS.37-4 MCS.37-5 |  | 16 |  | 1 |  | Note 2 Note 17 Note 19 |  |
| RC.26 TDD |  | TDD |  | 10 |  | 50 |  | - |  | - | MCS.32-1 MCS.32-2 |  | 10 |  | 1 |  | Note 5 |  |
| RC.27 TDD |  | TDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.33-1 MCS.33-2 |  | 16 |  | 1 |  | Note 5 |  |
|  |  | TDD |  | 10 |  | 50 |  | - |  | 2 CSI-RS | MCS.34-1 MCS.34-2 |  | 16 |  | 1 |  | Note 5 |  |
| Narrowband IoT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.33 FDD |  | HD-FDD |  | 200kHz |  | 1 |  | - |  | - | MCS.42 |  | 1 |  | 1 |  |  |  |
| RC.33 TDD |  | TDD |  | 200kHz |  | 1 |  | Note 3 |  | - | MCS.42 |  | 1 |  | 1 |  |  |  |
| Note 1: 3 symbols allocated to PDCCH. Note 2: For FDD only subframes 1, 2, 3, 4, 6, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 3: TDD UL-DL configuration as specified in the individual tests. Note 4: For TDD when UL-DL configuration 1 is used only subframes 4 and 9 are allocated to avoide PBCH and synchronizaiton signal overhead. Note 5: For TDD when UL-DL configuration 2 is used only subframes 3, 4, 8, and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 6: Centered within the Transmission Bandwidth Configuration (Figure 5.6-1). Note 7: Only subframes 2, 3, 4, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 8: Allocate PDSCH on 5th and 6th PRBs within a subband. Note 9: The number of HARQ processes is 10 for TDD UL/DL configuration 2 and 7 for TDD UL/DL configuration 1. Note 10: The downlink subframes are scheduled at the 1st, 2nd, 8th, 9th, 16th, 17th, 18th, 24th, 26th, 32nd, 33rd, 34th subframes every 40ms. Information bit payload is available if downlink subframe is scheduled.(starting from 0th subframe) Note 11: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in subframe 0 and 5 in RC.19 TDD. Note 12: Allocate PDSCH on 3th, 4th and 5th PRBs within a narrowband. Allocate MPDCCH on the 0th and 1st PRBs within a narrowband. Note 13: The PDSCH subframes are scheduled at the 0th and 1st subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe). MPDCCH subframes are scheduled at the 8th and 9th subframes every 10ms. Note 14: The downlink subframes are scheduled at the 0th to 4th subframes every 20ms. Information bit payload is scheduled at the 4th subframe (starting from 0th subframe). MPDCCH and Information bit payload are not scheduled in the radio frames where systemInformation1-BR is scheduled and = 4 with the set of frames and subframes for SIB1-BR defined in TS 36.211 [16] Table 6.4.1-2. Note 15: Information bit payload is scheduled at the 8th subframe every 20ms (starting from 0th subframe). Note 16: 2 symbols allocated for PDCCH. Note 17: No PDSCH is scheduled in subslot index 0. Note 18: Subslot-PDSCH is scheduled in subslots 2, 3, and 4. Note 19: Subslot-PDSCH is scheduled in subslots 1 and 5. Note 20: Allocate PDSCH on 3th, 4th and 5th PRBs within a narrowband. Allocate MPDCCH on the 0th, 1st, 2nd and 3rd PRBs within a narrowband. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-1a: Void

Table A.4-1b: Void

Table A.4-1c: Void

Table A.4-1d: Void

Table A.4-1e: Void

Table A.4-2: Void

Table A.4-2a: Void

Table A.4-2b: Void

Table A.4-2c: Void

Table A.4-2d: Void

Table A.4-2e: Void

Table A.4-3: Void

Table A.4-3a: Void

Table A.4-3b: Void

Table A.4-3c: Void

Table A.4-3d: Void

Table A.4-3e: Void

Table A.4-3f: Void

Table A.4-3g: Void

Table A.4-3h: Void

Table A.4-3i: Void

Table A.4-3j: Void

Table A.4-3k: Void

Table A.4-3l: Void

Table A.4-3m: Void

Table A.4-4: Void

Table A.4-4a: Void

Table A.4-4b: Void

Table A.4-5: Void

Table A.4-5a: Void

Table A.4-5b: Void

Table A.4-6: Void

Table A.4-6a: Void

Table A.4-6b: Void

Table A.4-6c: Void

Table A.4-6d: Void

Table A.4-6e: Void

Table A.4-6f: Void

Table A.4-7: Void

Table A.4-8: Void

Table A.4-9: Void

Table A.4-10: Void

Table A.4-11: Void

Table A.4-12: Void

Table A.4-13: Mapping of CQI Index to Modulation coding scheme (MCS)

| CQI Index |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  |  | OOR | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6016 | 0.4551 | 0.5537 | 0.6504 | 0.7539 | 0.8525 | 0.9258 |  |
| Modulation |  |  |  | OOR | QPSK |  |  |  |  |  | 16QAM |  |  | 64QAM |  |  |  |  |  |  |
| MCS Scheme |  | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.1 |  | 50 | 6300 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.2 |  | 50 | 6000 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.2A |  | 100 | 12000 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.3 |  | 50 | 5700 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 10 | 13 | 15 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.3A |  | 100 | 11400 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.4 |  | 50 | 5600 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.4A |  | 100 | 11200 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.5 |  | 50 | 5400 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.6 |  | 50 | 5300 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 22 | 24 | 25 |  |
| MCS.7 |  | 50 | 5200 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.8 |  | 50 | 5000 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.9 |  | 50 | 4800 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.10 |  | 6 | 756 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 19 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.11 |  | 6 | 684 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 14 | 17 | 20 | 21 | 23 | 25 | 27 |  |
| MCS.12 |  | 6 | 672 | DTX | 0 | 0 | 1 | 4 | 6 | 8 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.13 |  | 6 | 648 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 22 | 24 | 25 |  |
| MCS.14 |  | 25 | 3150 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.15 |  | 15 | 1890 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.16 |  | 15 | 1800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.17 |  | 3 | 378 | DTX | 0 | 1 | 2 | 5 | 7 | 9 | 12 | 13 | 16 | 19 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.18 |  | 50 | 5800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 17 | 20 | 22 | 23 | 26 | 27 |  |
| MCS.19 |  | 6 | 624 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.20 |  | 2 | 252 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 19 | 21 | 23 | 23 | 23 | 23 |  |
| MCS.21 |  | 6 | 696 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 21 | 24 | 25 | 27 |  |
| MCS.22 |  | 6 | 624 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 20 | 22 | 24 | 24 |  |
| MCS.23 |  | 41 | 4264 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 18 | 20 | 22 | 24 | 24 |  |
| MCS.24 |  | 50 | 5400 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.25 |  | 50 | 5100 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 12 | 13 | 15 | 18 | 20 | 22 | 23 | 24 |  |
| MCS 26 |  | 50 | 5800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.27 | CW0 | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 23 | 23 |  |
|  | CW1 | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 22 | 23 |  |
| MCS 29 |  | 50 | 5500 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.30 |  | 50 | 10200 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 12 | 14 | 15 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.31 |  | 50 | 9800 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 11 | 13 | 14 | 18 | 20 | 21 | 23 | 23 |  |
| MCS.32 |  | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 22 | 23 | 2Layer1CW |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement except for [MCS.23]. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-14: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.1A | 50 | 6300 | DTX | 0 | 1 | 3 | 5 | 7 | 10 | 11 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 26 |  |
| MCS.1B | 100 | 12600 | DTX | 0 | 1 | 3 | 5 | 7 | 10 | 11 | 14 | 15 | 18 | 20 | 22 | 24 | 26 | 26 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-15: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.11A | 6 | 684 | DTX | 0 | 1 | 3 | 5 | 7 | 8 | 10 | 13 | 14 | 16 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.12A | 6 | 672 | DTX | 0 | 1 | 3 | 5 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.11B | 8 | 912 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 10 | 13 | 14 | 16 | 18 | 19 | 22 | 24 | 26 |  |
| MCS.12B | 8 | 896 | DTX | 0 | 1 | 3 | 5 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 19 | 22 | 24 | 25 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-16: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS indx Table 3)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0391 | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6015 | Reserved | Reserved | Reserved | Reserved | Reserved |  |
| Modulation |  |  | OOR | QPSK |  |  |  |  |  |  | 16QAM |  |  |  |  |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.28 | 3 | 378 | DTX | 0 | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | N/A | N/A | N/A | N/A | N/A |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-17: Mapping of CQI Index to Modulation coding scheme (Slot-PDSCH)

| CQI Index |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  |  | OOR | 0.1523 | 0.2344 | 0.3770 | 0.6016 | 0.8770 | 1.1758 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 |  |
| MCS Scheme | PRB | Available RE-s | Slot number | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.32-1 | 50 | 2800 | 0 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 | Slot 0 |
| MCS.32-2 | 50 | 3672 | 1 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 13 | 15 | 16 | 20 | 23 | 25 | 27 | 28 | 28 | Slot 1 |
| MCS.33-1 | 50 | 2600 | 0 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 | Slot 0 |
| MCS.33-2 | 50 | 3348 | 1 | DTX | 0 | 0 | 2 | 4 | 7 | 9 | 12 | 14 | 16 | 19 | 21 | 23 | 26 | 27 | 28 | Slot 1 |
| MCS.34-1 | 50 | 2500 | 0 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 | Slot 0 |
| MCS.34-2 | 50 | 3348 | 1 | DTX | 0 | 0 | 2 | 4 | 7 | 9 | 12 | 14 | 16 | 19 | 21 | 23 | 26 | 27 | 28 | Slot 1 |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1 in TS 36.213 [6]. Note 2: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-18: Mapping of CQI Index to Modulation coding scheme (Subslot-PDSCH)

| CQI Index |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  |  | OOR | 0.1523 | 0.2344 | 0.3770 | 0.6016 | 0.8770 | 1.1758 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 |  |
| MCS Scheme | PRB | Available RE-s | Subslot number | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.35-1 | 50 | 1408 | 1 | DTX | 0 | 1 | 3 | 6 | 8 | 9 | 14 | 16 | 16 | 22 | 24 | 27 | 28 | 28 | 28 | Subslot 1 |
| MCS.35-2 | 50 | 1008 | 2 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 | Subslot 2 |
| MCS.35-3 | 50 | 872 | 3 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 | Subslot 3 |
| MCS.35-4 | 50 | 1008 | 4 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 | Subslot 4 |
| MCS.35-5 | 50 | 1472 | 5 | DTX | 0 | 1 | 3 | 6 | 9 | 9 | 14 | 16 | 16 | 22 | 25 | 27 | 28 | 28 | 28 | Subslot 5 |
| MCS.36-1 | 50 | 1180 | 1 | DTX | 0 | 0 | 2 | 5 | 7 | 9 | 12 | 14 | 16 | 19 | 22 | 24 | 27 | 28 | 28 | Subslot 1 |
| MCS.36-2 | 50 | 680 | 2 | DTX | 0 | 0 | 0 | 2 | 4 | 5 | 10 | 10 | 12 | 17 | 17 | 17 | 19 | 20 | 22 | Subslot 2 |
| MCS.36-3 | 50 | 612 | 3 | DTX | 0 | 0 | 0 | 2 | 3 | 5 | 10 | 10 | 11 | 17 | 17 | 17 | 18 | 19 | 20 | Subslot 3 |
| MCS.36-4 | 50 | 680 | 4 | DTX | 0 | 0 | 0 | 2 | 4 | 5 | 10 | 10 | 12 | 17 | 17 | 17 | 19 | 20 | 20 | Subslot 4 |
| MCS.36-5 | 50 | 1212 | 5 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 12 | 15 | 16 | 20 | 22 | 25 | 27 | 28 | 28 | Subslot 5 |
| MCS.37-1 | 50 | 1180 | 1 | DTX | 0 | 0 | 2 | 5 | 7 | 9 | 12 | 14 | 16 | 19 | 22 | 24 | 27 | 28 | 28 | Subslot 1 |
| MCS.37-2 | 50 | 612 | 2 | DTX | 0 | 0 | 0 | 2 | 3 | 5 | 10 | 10 | 11 | 17 | 17 | 17 | 18 | 19 | 20 | Subslot 2 |
| MCS.37-3 | 50 | 612 | 3 | DTX | 0 | 0 | 0 | 2 | 3 | 5 | 10 | 10 | 11 | 17 | 17 | 17 | 18 | 19 | 20 | Subslot 3 |
| MCS.37-4 | 50 | 680 | 4 | DTX | 0 | 0 | 0 | 2 | 4 | 5 | 10 | 10 | 12 | 17 | 17 | 17 | 19 | 20 | 22 | Subslot 4 |
| MCS.37-5 | 50 | 1212 | 5 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 12 | 15 | 16 | 20 | 22 | 25 | 27 | 28 | 28 | Subslot 5 |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1 in TS 36.213 [6]. Note 2: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-19: Mapping of CQI Index to Modulation coding scheme (4-bit CQI Table 5)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0391 | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6016 | 0.4551 | 0.5337 | 0.6504 | 0.7439 | 0.8525 |  |
| Modulation |  |  | OOR |  |  |  | QPSK |  |  |  | 16QAM |  |  |  | 64QAM |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.40 | 1 | 126 | DTX | 0 | 0 | 0 | 1 | 4 | 5 | 8 | 12 | 14 | 16 | 18 | 21 | 23 | 25 | 27 |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-20: Mapping of CQI Index to Modulation coding scheme (4-bit CQI Table 6)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0547 | 0.2021 | 0.2598 | 0.1904 | 0.1387 | 0.2598 | 0.4424 | 0.6221 | 0.4131 | 0.5439 | 0.6797 | 0.8252 | 0.6357 | 0.7617 | 0.8672 |  |
| Modulation |  |  | OOR |  |  |  | QPSK |  |  |  | 16QAM |  |  |  | 64QAM |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.41 | 3 | 378 | DTX | 0 | 2 | 3 | 2 | 1 | 3 | 7 | 9 | 12 | 15 | 16 | 16 | 22 | 25 | 27 |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-21: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 3 and 4-bit CQI Table 4)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 2.4063 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 | 8.3321 | 9.2578 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.38 | 50 | 6300 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 22 | 23 | 25 |  |
| MCS.39 | 100 | 12600 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 22 | 23 | 25 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1B, 7.1.7.2.1-1 and 7.2.3-4 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-22: Mapping of channel quality reported value to Modulation coding scheme

| Channel quality reported value |  |  | No measurement | A | B | C | D | E | F | G | H | I | J | K | L | M | N | O | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.4316 | 0.2737 | 0,1579 | 0.0789 | 0.0395 | 0.0198 | 0.6579 | 0.8860 | 1.1316 | 1.4825 | 1.9035 | 2.1140 | 2.5702 | 3.0614 | 3.2719 |  |
| NPDSCH repetition |  |  |  | 1 | 1 | 1 | 2 | 4 | 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |
| MCS Scheme | PRB | Available RE-s per subframe | ITBS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.42 | 1 | 160 | DTX | 3 | 0 | 0 | 0 | 0 | 0 | 6 | 8 | 10 | 12 | 15 | 16 | 18 | 21 | 21 |  |
| Note 1: Mapping between ITBS and channel quality reported value according to Table 9.1.22.17-1 in TS 36.133 and Table 16.4.1.5.1-1 in TS 36.213. Note 2: ISF=1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------