3GPP TSG-RAN WG4 Meeting #104-e     	R4-2212913

E-meeting, August 15th – 26th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM requirements for UE power saving for RedCap

Agenda item:	9.18.3.1.4 Signaling characteristics	[NR_redcap-Core]

Document for:	Discussion

- Introduction

- In the RedCap WID in [1] considers that power saving enhancements solutions specififed in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) are assumed to be available also to RedCap UEs by default. During the RAN4 discussions on the definition of requirements for different Rel-16 and Rel-17 features an LS was sent to RAN2 with the following agreement [3] (RAN4 #102):

RAN4 would like to thank RAN2 for the incoming LS in R2-2109218. RAN4 has discussed the RAN2 agreements on UE capabilities from a RRM perspective and further discussed the Rel-16 UE features and their applicability for RedCap in Rel-17. Based on the discussions, following conclusion is reached: Rel-15/Rel-16 features in TS 38.133; RedCap RRM requirements applicability in R17. Dual connectivity and carrier aggregation; Not applicable. 2-step RA; Applicable. NR measurements with autonomous gaps; Applicable. Rel-17 features in TS 38.133. SDT; Applicable. Note: RAN4 will not define any RRM requirements for RedCap UE for other release 16/release 17 features which are not listed above in release 17..

RAN4 has agreed that no RRM requirements will be defined for RedCap UE for release 17 features other than Small Data Transmission.

However, in RAN4 #103, RAN4 has discussed whether requirements for UE power saving would be defined for RedCap UEs. While there was no consensus, different companies supported the proposal of defining requirements for this combination of features still in Rel-17, in the maintenance phase, as in the “other notes” section in the agreed WF in [2] copied below.

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17 Candidate options: - Option 1 (QC): RAN4 to define further relaxations (e.g., larger K values) to RLM/BFD measurements for a Rel-17 RedCap UE that is configured with and satisfies the stationary criterion. - Option 2 (HW, Apple, CMCC, vivo, OPPO): Relaxed RLM/BFD based on Rel-17 UE power saving WI is out of RedCap WI scope. - Option 3 (E///, MTK, Xiaomi, MTK): FFS under maintenance

- In RAN4 #103, it was proposed to define further relaxation (RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

In this contribution, we present Nokia’s views on the definition of relaxed RLM/BFD based on Rel-17 UE power saving for RedCap UEs in Rel-17.

- Discussion

Both work items NR_redcap and NR_power_sav_enh have defined new RRM requirements for radio link monitoring (RLM) and beam failure detection (BFD). The baseline for the requirements in both features is the RRM requirements defined in NR. Up to Rel-16, the NR RRM requirements have been defined under the assumption that the UEs have 2 Rx branches. This assumption changed in the RedCap work item, which also considers UEs with 1 Rx branch. Furthermore, requirements were defined for Half-duplex FDD UEs (HD-FDD).

In general, the requirements defined for 2 Rx RedCap UEs are the same as the requirements for NR UEs.

The RLM/BFD relaxation factors in the NR_power_sav_enh were defined based on the assumption of 2 Rx UEs.

For 1 Rx RedCap UEs, there is an accuracy degradation caused by reducing the number of Rx branches, when compared to the accuracy for 2 Rx RedCap UEs, as shown in our contributions in [4] and [5]. Therefore, in the RedCap WI, the following modifications were done when comparing to the baseline requirements:

- The OOS evaluation period and the lower bound of the OOS evaluation period were doubled.

- The PDCCH transmission parameters for in-sync and out-of-sync evaluation were changed. For in sync and out-of-sync, the bandwidth and the aggregation level were doubled.

- It is not straightforward to assume RLM/BFD relaxation for 1 Rx RedCap UEs, considering the modifications above and considering also that the relaxation is based on different criteria which are based on RSRP thresholds. There is still a discussion about whether to apply an offset to RSRP thresholds for 1 Rx RedCap UEs, and to which thresholds this offset will be applied.

It is not straightforward to apply the RLM/BFD requirements to 1 Rx RedCap UEs without simulations, since the evaluation periods were modified in comparison to those defined for 2 Rx RedCap.

Considering that for 2 Rx RedCap UEs, the assumptions under which the requirements have been defined are the same as for NR UEs, we consider that there is not an extensive RAN4 effort required to define requirements for RLM/BFD relaxation in this case. Therefore, we propose the following:

RAN4 to prioritize the definition of RLM/BFD relaxation requirements to 2 Rx RedCap UEs in Rel-17.

The RLM/BFD relaxation factors defined in the NR_power_sav_enh WI are applicable to 2 Rx RedCap UEs.

- Conclusions

In this document, Nokia’s view regarding Rel-17 power saving applicability to RedCap UEs is shared. The following observations and conclusions are made:

- RAN4 has agreed that no RRM requirements will be defined for RedCap UE for release 17 features other than Small Data Transmission.

- In RAN4 #103, it was proposed to define further relaxation (RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

- In general, the requirements defined for 2 Rx RedCap UEs are the same as the requirements for NR UEs.

- The RLM/BFD relaxation factors in the NR_power_sav_enh were defined based on the assumption of 2 Rx UEs.

- It is not straightforward to apply the RLM/BFD requirements to 1 Rx RedCap UEs without simulations, since the evaluation periods were modified in comparison to those defined for 2 Rx RedCap.

- RAN4 to prioritize the definition of RLM/BFD relaxation requirements to 2 Rx RedCap UEs in Rel-17.

The RLM/BFD relaxation factors defined in the NR_power_sav_enh WI are applicable to 2 Rx RedCap UEs.

References

- RP-220966, “Revised WID on support of reduced capability NR devices”

- R4-2210592, “WF on RedCap RRM Requirements”, Ericsson (RAN4 #103)

- R4-2206977, “Reply LS on UE capabilities for RedCap from RRM perspective” , Ericsson (RAN4 #102)

- R4-2200867, “Simulation Results: RLM for RedCap”, Nokia, Nokia Shanghai Bell, (RAN4 #101-bis)

- R4-2200868, “Discussion on hypothetical PDCCH parameters for RedCap UEs”, Nokia, Nokia Shanghai Bell (RAN4 #101-bis)