3GPP TSG-RAN WG4 Meeting #104-e	R4-2212920

Electronic Meeting August 15 – August 26, 2022

Source	: Nokia, Nokia Shanghai Bell

Title	: Remaining issues on unified TCI switching delay requirement

Agenda item	: 9.17.2.1

Document for	: Discussion

Introduction

RAN4#103 has discussed remaining issues of Rel-17 feMIMO WI with RRM core requirements. The requirements include RRM core requirements of items below :

- TCI switching delay requirements under Rel-17 unified TCI framework

- L1 measurement requirements for inter-cell beam management

- Others : BFD/CBD/BFRQ  and QCL

It still remains significant issues; we see statements from incomplete discussions with brackets in the CRs regarding Rel-17 TCI switching. In this contribution, we discussed remaining issues on TCI switching delay requirements.

Discussion

RAN4 has discussed that path-loss maintenance is the condition of ‘active’ TCI state for UL. In fact, we cannot find any definition of ‘active’ TCI for UL or related behaviors in RAN1 and RAN2 spec yet, while active DL TCI definition and behaviors are found in TS38.133, TS38.214 and TS38.306. So, we assume ‘active’ TCI state is new concept for UL in Rel-17. Especially, we see still unclarity on how to maintain the target UL TCI state as FFS bullets in email discussions and multiple CRs, we further discuss on UL TCI switching in this contribution.

Issue in ‘active’ TCI state for UL

RAN4#103 has discussed about UE behaviors of active TCI states.

RAN4#103 WF R4-2211203 Issue 1-1-1 Active TCI state for UL - Further discuss whether LS is needed for clarifying ‘active UL TCI state’ the following issues: - i: If the UL (or joint) TCI state is activated, should a UE track UL TCI state timing and/or frequency derived from DL-RS associated with UL TCI state (or joint) TCI as a UE does for active DL-TCI? - i-1: What DL-RS can be used to track timing and/or frequency for active UL TCI for non-serving cell? Specifically, how can a UE track timing and/or frequency, if SRS is indicated as source RS in the active UL TCI? - ii: If a UE maintains the PL-RS of the activated UL TCI state (or joint) TCI state by the RAN1 agreement, the UE should maintain all of PL-RSs in the activated UL TCI (or joint) TCIs to support inter-cell or mTRP scenarios? - ii-1: What is a UE capability to measure pathloss to support the active UL TCI list in inter-cell and mTRP? (Note : there is no UE capability indication on pathloss measurement in TS38.306 capability spec )

For DL MAC-CE and DCI switching, active TCI state list has been used to trigger TCI switching with data scheduling. If a TCI state is active in DL, the UE is supposed to conduct time and frequency tracking on the source RS. Such DL MAC-CE active state has been defined in 133. We capture below to compare DL and UL switching statements as currently captured in latest Rel-17 version of 38.133.

CR R4-2206945 TS38.133 8.15.3 MAC-CE based downlink TCI state switch delay If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. … TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise. 8.16.3 MAC-CE based uplink TCI state switch delay For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell, - If target TCI state is known, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms). - If target TCI state is unknown, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms. NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise

A problem of the current spec is found related to ‘active’ TCI state for UL. The UE behavior is not clear regarding how a UE manages active UL TCI comparing to active DL TCI. For DL case, if target TCI state is in the active TCI state list for PDSCH/PDCCH, the UE should track time and frequency sync on the resource RS. When DL TCI is indicated, the UE can immediately demodulate the PDCCH/PDSCH based on the sync information as captured below. In RAN1/2 spec, DL case clearly specifies maxNumberActiveTCI-PerBWP in tci-statePDSCH, and a UE should track time and frequency sync.

tci-StatePDSCH TS38.306 - maxNumberActiveTCI-PerBWP indicates the maximum number of activated TCI-states per BWP per CC, including control and data. If a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. The UE shall include this field. Note the UE is required to track only the active TCI states.

For UL TCI transmission, RAN4 has mainly discussed path-loss measurement latency. However, we note firstly that fundamental condition for active UL TCI has been missing, a UE should keep time and frequency sync to the source RS in the UL TCI for UL transmission like the DL case. A UE cannot transmit PUCCH/PUSCH/SRS independently from DL sync. Putting TA adjustment discussion aside, maxNumberActiveTCI-PerBWP under tci-StatePDSCH must be about activated TCI-states per BWP per CC with UE synchronization for both DL and UL.

Observation 1 : Fundamental conditions for active TCI for UL are missing. There is no definition of active TCI for UL in RAN1/2 specs. Also, a UE should keep time and frequency sync tracking on the source RS in UL TCI to be capable of UL transmission.

Observation 2 : maxNumberActiveTCI-PerBWP under tci-StatePDSCH should be about activated TCI-states with UE synchronization for both DL and UL. The current spec addresses about DL only.

Proposal 1 : Rel-17 active UL TCI state should be under time and frequency tracking. This means that active UL TCI list belongs to active DL TCI state list.

- Add the time and frequency tracking condition to the active TCI state for UL.

If Proposal-1 is agreeable, there are two ways to correct it in the spec. One is to add the time and frequency tracking condition to the known UL TCI state (CR R4-2206945), and another way is to consider actual tracking delay in 8.16.3 MAC-CE based uplink TCI state switch delay (CR R4-2206945).

- 2.2 Issue in PL-RS maintenance for active TCI

As mentioned, active UL TCI state has an additional condition of PL-RS maintenance than active DL TCI state.  In fact, the active UL TCI state with PL-RS measurement maintenance is not written yet in any of RAN1/2 spec, now RAN4 has directly to discuss it in this meeting. RAN1/2 has only active TCI state for tci-statePDSCH, and it is up to 8 active TCI state can be configured up to UE capability.

Also, a UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions.

TS38.213 Chapter 7. Uplink Power control A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in subcaluses 7.1.1, 7.2.1, and 7.3.1.

In results, we assume that some of DL active TCI state will have UL active TCI state as the left diagram, although it is unclear whether RAN1/2 have assumed the right diagram is also possible.

The configuration and operation seem like RAN1/2 issues. RAN4 may want to send LS to clarify in Rel-17. Anyhow, if assuming maintained PL-RS measurement is the active UL TCI condition, we found the issues below in RAN4 perspective,

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

In Rel-16, PL-RS maintenance relies on DL and UL channel reciprocity. If DL reception is triggered using a beam selection, UL beam can be selected by the channel reciprocity. However, in Rel-17, the unified TCI framework is designed without DL and UL channel reciprocity to support non-collocated multiple cell scenarios. Therefore, the path loss measurement for active UL TCI in the UE side should be aligned separately by network scheduling intention.

In addition, the number of PL-RS samples are not clear to say always requiring 5 sample measurement. It is obvious that accurate PL-RS measurement can be a pre-condition to make UL transmission, but it is not clear if the 5 sample PL-RS measurement period is essential for ‘active’ UL TCI. RAN4 does not have accuracy requirement for PL-RS measurement, but the spec addresses unconditionally 5 samples are required, that is problematic. For an example of DCI-based UL TCI switching, this activation of UL TCI should be quicker than MAC-CE based switching. In DCI-based TCI switching requirements, we are not sure if the ‘active TCI list’ in the box is the same TCI list equal to ‘active TCI list’ for UL. The UE behaviors regarding the active UL TCI state should be clear to address in 8.16.4. At least the ‘active’ TCI list seems different between DL and UL cases because of PL-RS maintenance in UL.

Issue 1-1-4 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 FFS: - Option-1 : Longer delay is expected when SSB is indicated as PL-RS in UL TCI state for FR2 - Option-2 : If a UE has measured and reported L1-RSRP within [Y] msec on the SSB indicated as PL-RS in UL TCI state, the PL-RS is regarded to be maintained. (i.e. a filtered L1-RSRP measurement process is equivalent to PL measurement process based on = referenceSignalPower – higher layer filtered RSRP in TS38.213)

Observation 3 : We found main issues regarding ‘active UL TCI list’ in inter-cell or mTRP scenarios

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

Proposal 2 : Based on Observation-2,3 regarding ‘active TCI list’ for UL, we propose a LS draft in appendix.

- Send LS to RAN1/2 to share the issue statements on active TCI list for UL. Possible solution is up to RAN1/2 in Rel-17 or Rel-18 enhancement.

- Improve PL-RS measurement period for TCI switching using active UL TCI list by reducing strict 5 PL-RS sample measurement period.

Here is the insight to reduce the 5 PL-RS sample measurement period. In the spec, the sample usage boundary between L1-RSRP measurement and PL-RS measurement is not clearly written regarding UE behavior. Regarding the PL-RS measurement, TS38.213 spec specifies as

= referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by higher layer parameter QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

RAN4 just takes a baseline UL behavior assuming two different steps of pathloss measurement: step-(i) PL-RS measurement beam selection through L1-RSRP, and then step-(ii) derives ‘higher layer filtered RSRP’ from PL-RS (5 samples)’. However, if the two steps are performed on the same RS, that needs not to make two separate steps. The process measuring higher layer filtered RSRP can be combined to obtain the filtered received power.

As stated above, we find two problems. If a UE needs to maintain multiple PL-RSs in multiple UL TCI, which PL-RS will be maintained or not maintained and how network knows about UL-TCI with maintained PL-RS. There is no separate indication to make such activation of PL-RS measurement in the current specs.

Also, the number of PL-RS samples (M) won’t always be fixed at 5 samples. If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, then we think it can be less than 5. In Rel-17, assuming PL-RS with different QCL-D cannot be measured simultaneously, a scaling factor will be applied, but the number of PL-RS under pathloss measurement should be scheduled and managed.

This issue assumes when SSB is indicated as PL-RS in UL TCI state, so TSSB is referred in this issue discussion. However, the PL-RS configuration is possible with both SSB and CSI-RS in case, the notation in the spec should be ‘TPL_RS’ that makes more general and generic requirements.

Proposal 3 : when SSB is indicated as PL-RS in UL TCI state for FR2,

-  The number of sample M will not always be fixed as 5 samples.

-  If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, the number of samples used for L1-RSRP is counted for pathloss measurement.

-  If a UE has reported L1-RSRP measurement on a PL-RS within a time window, the PL-RS is regarded as maintained.

  - Joint TCI state switching requirement

RAN4#103 WF R4-2211203 Issue 1-1-3a Joint TCI state switching requirement - RAN4 #101bis GTW Agreements - No extra requirement needed for Joint TCI mode, DL and UL requirements can be applicable independently - Note: it is not expected that UE will be required to make DL reception or UL transmission before UE completes the DL or UL TCI state switching, respectively - Joint TCI switching delay requirement - Option 1: In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch - Option 2: Joint TCI switching delay is regarded as same as a pair of separate DL/UL TCI switching. - In case of joint TCI state switch, UE is expected to receive on DL, when UE completes the DL state switch. - In case of joint TCI state switch, UE is expected to transmit on UL, when UE completes the UL state switch. - Other options are not precluded.

Based on the agreements in RAN4#101-bis, it is clear that option-2 is the obvious UE behaviors. No more discussion is required on this. Somehow, the previous CR in RAN4#103 captured statements reverting the agreement, that should not happen.

Proposed 4 : Regarding Issue 1-1-3a Joint TCI state switching requirement,  it is clear that option-2 is the obvious UE behaviors based on the agreements in RAN4#101-bis. No more technical discussion is required. A CR should capture the option-2 statement as the agreement.

  - Known condition on shared RS in CA scenario

Issue 1-3-1 Known condition on shared RS in CA scenario Agreement: - Reuse the existing known condition. If the associated RS in common TCI state provides QCL-TypeD, the known condition can only consider whether the associated RS in the reference CC is known or not. - FFS: the associated RS in common TCI state provides QCL-Type C Issue 1-3-3a Common TCI state switching delay requirement for shared RS - FFS: - The delay requirement is defined per CC for the common TCI indicated by simultaneousU-TCI-UpdateList1/2/3/4-r17 - The delay requirement is defined for multiple CCs for the common TCI indicated by RefUnifiedTCIStateList-r17

Regarding issue 1-3-1, it is up to the definition of ‘known condition’. So far, the main intention of the known condition is about beam correspondence. The QCL type-C has {average delay, doppler shift }, then it is related to time and frequency synchronization. We wonder what new UE behaviors are additionally defined with QCL type-C in CA scenario.

If network aims to request to monitor PDCCH or schedule PDSCH in CA scenarios, the UE should be prepared with sync with QCL-Type C information. This is included as ‘active TCI’ state condition, but we wonder if it needs to be ‘known conditions’ for ‘known’ TCI state.

Proposal 5 : Clarify what new UE behaviors are additionally defined with QCL type-C.

- QCL-type C information is required to be ‘active TCI’ state, but wonder if QCL-type C information is needed to be ‘known conditions’ for ‘known’ TCI state in CA scenario.

Basically, we have the same view as the FFS points in Issue 1-3-3a. The TCI switching delay requirement is defined per CC that refers to the common TCI indicated by simultaneous-TCI-UpdateList. Note that the TCI switching mechanism by simultaneous-TCI-UpdateList-rel17 has been existed since Rel-16, so it has been introduced to switch TCI over multiple CC in the legacy manner. Rel-17 still supports the legacy mechanism under unified TCI framework.

Also Rel-17 has introduced RefUnifiedTCIStateList-r17 as a common reference TCI for CA TCI switching mechanism over multiple CCs, the requirements should be applied for this case as well.

Proposal 6 : Regarding common TCI state switching delay requirement for shared RS,

    - The delay requirement is defined based on the common TCI indicated by simultaneousU-TCI-UpdateList1/2/3/4-r17, and the requirements are applied to all CCs referring to the common TCI.

    - The delay requirement is defined based on the common TCI indicated by RefUnifiedTCIStateList-r17, and the requirements are applied to all CCs referring to the common TCI.

Conclusion

In this contribution we provide our observations and proposals on remaining issues on unified TCI switching requirements.

Observation 1 : Fundamental conditions for active TCI for UL are missing. There is no definition of active TCI for UL in RAN1/2 specs. Also, a UE should keep time and frequency sync tracking on the source RS in UL TCI to be capable of UL transmission.

Observation 2 : maxNumberActiveTCI-PerBWP under tci-StatePDSCH should be about activated TCI-states with UE synchronization for both DL and UL. The current spec addresses about DL only.

Proposal 1 : Rel-17 active UL TCI state should be under time and frequency tracking. This means that active UL TCI list belongs to active DL TCI state list.

- Add the time and frequency tracking condition to the active TCI state for UL.

Observation 3 : We found main issues regarding ‘active UL TCI list’ in inter-cell or mTRP scenarios

(i) Network does not know which PL-RS is maintained by UE, so it does not know which path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

Proposal 2 : Based on Observation-2,3 regarding ‘active TCI list’ for UL, we propose a LS draft in appendix.

- Send LS to RAN1/2 to share the issue statements on active TCI list for UL. Possible solution is up to RAN1/2 in Rel-17 or Rel-18 enhancement.

- Improve PL-RS measurement period for TCI switching using active UL TCI list by reducing strict 5 PL-RS sample measurement period.

Proposal 3 : when SSB is indicated as PL-RS in UL TCI state for FR2,

-  The number of sample M will not always be fixed as 5 samples.

-  If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, the number of samples used for L1-RSRP is counted for pathloss measurement.

-  If a UE has reported L1-RSRP measurement on a PL-RS within a time window, the PL-RS is regarded as maintained.

Proposed 4 : Regarding Issue 1-1-3a Joint TCI state switching requirement,  it is clear that option-2 is the obvious UE behaviors based on the agreements in RAN4#101-bis. No more technical discussion is required. A CR should capture the option-2 statement as the agreement.

Proposal 5 : Clarify what new UE behaviors are additionally defined with QCL type-C.

- QCL-type C information is required to be ‘active TCI’ state, but wonder if QCL-type C information is needed to be ‘known conditions’ for ‘known’ TCI state in CA scenario.

Proposal 6: Regarding common TCI state switching delay requirement for shared RS,

    - The delay requirement is defined based on the common TCI indicated by simultaneousU-TCI-UpdateList1/2/3/4-r17, and the requirements are applied to all CCs referring to the common TCI.

    - The delay requirement is defined based on the common TCI indicated by RefUnifiedTCIStateList-r17, and the requirements are applied to all CCs referring to the common TCI.

- 4. References

[1] R4-2211203, WF on FeMIMO RRM impact for unified TCI state, RAN4#103-e