GPP TSG-RAN WG4 Meeting # 104-e	R4-2212973

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	11.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion RRM requirements for FR1-FR1 NR-DC

Document for:	Discussion

# Introduction

RRM requirements for FR1+FR1 NR-DC are missing in TS 38.133. The objective of defining corresponding requirements is included in Rel-18 WI [1] and further revised [2]. In this paper, we provide our views of RRM impact of the objective.

# Discussion

In current TS 38.133, there is applicability rules about NR-DC as follows:

- NR-NR dual connectivity which involves two gNB acting as Master gNB and Secondary gNB is referred to as “NR-NR dual connectivity” or “NR-DC”. NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2.

In WID, the scope of the RRM requirements for FR1-FR1 is described as follows:

- Define RRM requirements for FR1-FR1 NR-DC scenarios [RAN4] - RRM requirements include the number of serving carriers, PSCell addition/release delay requirement, PSCell change and conditional PSCell change delay, scheduling availability, and CSSF. Other Rel-15 requirements are not precluded and are subject to WI stage discussion. - For R16 and R17 features, RRM requirements for FR1-FR1 NR-DC including HO with PSCell, SCG activation/deactivation and CPAC. - Note: no other R16/17 features are considered

Before going through the details of each requirements, it is important to clarify the scope and impact of the requirements of the objective. From our understanding, some requirements mentioned in the WI needs to be updated for FR1+FR1 NR-DC. For instance, for PSCell addition requirements, the time for cell searching and Tprocessing can be different when the PSCell is in FR1 or FR2. For other requirements, for instance, the timing requirements/SCell activation requirements which may not have difference between FR1+FR1/FR1+F2 NR-DC, and it is not mentioned in the WID.

Observation 1: There are requirements which may not have difference between FR1+FR1/FR1+F2 NR-DC.

It is worthy to be clarified that how to treat the requirements which don’t have difference between FR1+FR1/FR1+F2 NR-DC. For instance, as is clearly stated in the WID, no other R16/R17 feature are considered. For some requirements introduced in R16/R17, e.g. BWP switch on multiple CCs, there may be not spec updating to support FR1+FR1 NR-DC, does it means it is supported by default or it only applies to FR1+FR2 NR-DC?

Observation 2: For R16/R17 requirements, there are requirements need to be updated to support FR1+FR1 NR-DC, while for others there may be no impact on existing requirements.

Proposal 1: Apart from the requirements in R16/R17 mentioned in the WID, RAN4 to discuss the applicability rules of other R16/R17 requirements which may not have impact on existing requirements for FR1+FR1 NR-DC.

Based on the analysis above, if it is concluded that some requirements can apply to both FR1+FR1/FR1+FR2 NR-DC, while others can only apply to FR1+FR2 NR-DC, then some necessary descriptions about applicability rules needs to be added in the spec.

Apart from the generic consideration of the applicability rules of requirements for FR1+FR1 NR-DC, we further provide detailed analysis for requirements which needs to be updated as mentioned in the WID.

Serving carriers

In current spec, the applicability of number of serving cells for NR-DC is defined as follows where only FR1+FR2 NR-DC is considered. It is straightforward to add the corresponding requirements for FR1+FR1 NR-DC based on the BC accordingly.

#### 3.6.2.4 Number of serving carriers for NR-DC Requirements for NR-DC are applicable for the UE configured with the following number of serving NR CCs: - up to 2 NR DL CCs in total in FR1, up to 8 NR DL CCs in total in FR2, with 1 UL in PCell, 1 UL in PSCell, and up to 1 UL in each SCell.

PSCell addition/release delay requirement

For PSCell addition requirements, Tprocessing and Tsearch need to be updated to be updated for FR1+FR1 NR-DC, which only considers the processing and cell search time of a FR2 PSCell, which is shown as follows.

### 8.9.2 PSCell Addition Delay Requirement The requirements in this clause shall apply for the UE configured with only PCell in FR1. Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in subframe n + Tconfig_PSCell Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 40 ms. Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 24* Trs ms.

PSCell change and conditional PSCell change delay

For PSCell change and conditional PSCell change delay, the requirements seems can apply to FR1+FR1 NR-DC without changing the requirements.

Scheduling availability

For scheduling availability, for instance the scheduling availability in RLM measurement, as inter-band FR1+FR1 NR-DC is considered, the requirements seem can apply to FR1+FR1 NR-DC without changing the requirements.

CSSF

For CSSF, in the current spec, only FR2 PSCC is considered in that table for measurement outside gaps. It straightforward that CSSF can be updated accordingly based on the same searcher assumptions. But one thing needs to be considered is that CSSF includes both SSB and CSI-RS based measurement. However, whether to consider CSI-RS based measurement which is introduced in R16 needs to be clarified.

HO with PSCell

For HO with PSCell which is introduced in R17, similar as PSCell addition, Tprosessing, Tsearch needs to be reconsidered.

SCG activation/deactivation

For SCG activation/deactivation requirements which is introduced in R17, similar as PSCell addition, Tprosessing, Tsearch needs to be reconsidered for FR1 PSCell. Besides, the known conditions also need to be updated which only include FR2 currently.

CPAC

Similar as conditional PSCell addition, the requirements seems can apply to FR1+FR1 NR-DC without changing the requirements.

# Conclusions

Observation 1: There are requirements which may not have difference between FR1+FR1/FR1+F2 NR-DC.

Observation 2: For R16/R17 requirements, there are requirements need to be updated to support FR1+FR1 NR-DC, while for others there may be no impact on existing requirements.

Proposal 1: Apart from the requirements in R16/R17 mentioned in the WID, RAN4 to discuss the applicability rules of other R16/R17 requirements for FR1+FR1 NR-DC with/without changing on existing requirements.

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC