3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213069

# Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for Redcap PBCH

Agenda item:	9.18.5.1.2

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF for UE demodulation and CSI requirements for RedCap [1] are captured in the summary [2] and WF [3].

In this contribution, we present a summary of our simulation results for PBCH based on the agreed configurations in [3].

PBCH simulation results summary

The agreed configurations for PBCH demodulation performance were revised in the last meeting and are depicted in [3]. Results are presented both for alignment and for specification of requirements.

PBCH simulation results for alignment

Simulation results for alignment are reported in this section for agreed simulation assumptions contained in the WF [3].

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New | -5.8 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New | -4.7 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse | -8.4 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse | -9.6 |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) | -8.8 |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) | -10.7 |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse | -8.2 |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse | -9.2 |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse | -8.0 |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse | -8.9 |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |  |

PBCH simulation results with impairments

Simulation results including impairments are reported in this section for agreed simulation assumptions contained in the WF [3].

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New | -3.3 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New | -2.2 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse | -5.9 |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse | -7.1 |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) | -6.3 |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) | -8.2 |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse | -5.1 |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse | -6.1 |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse | -4.9 |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse | -5.8 |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |  |

Conclusion

In this contribution we have provided a summary of our PBCH simulation results for agreed configurations in the WF [3] both for alignment and for requirements specification.

The following proposal is made.

Include PBCH results for alignment and for requirements specification into the simulation summary.

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2210532,"Email discussion summary for [103-e][329] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2210672, "WF on RedCap demodulation and CQI reporting requirements", Ericsson