3GPP TSG-RAN WG4 Meeting #104-e                              R4-2213130

Electronic Meeting, 15 August– 26 August, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on CADC band combinations including n7/7 and n38/38

Agenda Item:	5.1.8

Document for:	Approval

Introduction

In current TS 38.101-3 specification, some band combinations include both FDD 2.5GHz band 7(n7) and European TDD 2.6GHz band 38(n38). Since band 7(n7) UL signals will have a severe impact on band 38(n38) DL slot and band 38(n38) UL slot will have a severe impact on 7(n7) DL band, band 7(n7) and band 38(n38) can only be configured as DL Scell when these two bands are included in one CA/DC combination.

However, the following band combinations were still introduced into specification with a clarification that “NOTE 17:	The combination is not used alone as fall back mode of other band combinations.”

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n38A17,18 | N/A |
| DC_1A-38A_n7A17,18 | N/A |
| DC_3A-7A_n38A17,18 | N/A |
| DC_3A-38A_n7A17,18 | N/A |
| DC_7A-20A_n38A17,18 | N/A |
| DC_7A-38A_n3A17,18 | N/A |
| DC_7A-38A_n78A17,18 | N/A |
| DC_7A_n38A-n78A17,18 | N/A |
| DC_7A-38A_n3A-n78A10 | N/A |
| DC_1A-3A-7A-20A_n38A7,10 | N/A |

In this paper, we’d like to discuss whether we need to introduce some band combinations which are not used alone and further clarify the fallback mode of band combinations.

Discussion

Figure 1 Illustration on general fall back paths of ENDC band combination

Referring to figure 1, it’s shown what the typical fall back paths of ENDC band combination look like. For the highest order band combination which is targeted to be configured by network, there are multiple paths to reach the final goal. For example, path1 can be DC_1A_n3A => DC_1A-2A_n3A => DC_1A-2A_n3A-n4A, and path2 can be DC_1A_n4A => DC_1A-2A_n4A => DC_1A-2A_n3A-n4A. Generally, RAN4 need to traverse and introduce all the possible fall back band combinations before toughing the final goal.

Observation 1: Based on the current principle and procedure, RAN4 need to traverse and introduce all the possible fall back band combinations before studying the highest order band combination.

But some special band combinations can’t follow this principle due to technical issues. As we all know, the UL frequency range (2500~2570MHz) of band n7/7 is adjacent to DL frequency range (2570~2620MHz) of TDD band n38/38 and UL frequency range (2570~2620MHz) of TDD band n38/38 is adjacent to the DL frequency range (2620~2690MHz) of band n7/7. Thus, it’s very hard to address the UL interference from in-device coexistence perspective when band n7/7 and band n38/38 are combined with one UL band. In order to solve this problem, band n7/7 and band n38/38 are restricted as DL Scells from standard perspective once band n7/7 and band n38/38 are combined into one band combination.

Observation 2: band n7/7 and band n38/38 are restricted as DL Scells from standard perspective once band n7/7 and band n38/38 are combined into one band combination in order to solve UL-DL interference issue.

Figure 2 Illustration on fall back paths of ENDC DC_1A-7A_n38A-n78A

For example, the fall back paths of DC_1A-7A_n38A-n78A are shown in figure 2. Since band n7/7 and band n38/38 are restricted as DL Scells, it’s impossible to configure the red paths to achieve the final goal DC_1A-7A_n38A-n78A and DC_1A-7A_n38A can’t work in the real deployment, e.g. DC_1A_n38A => DC_1A-7A_n38A => DC_1A-7A_n38A-n78A. And the green paths are technically reasonable configuration paths, e.g. DC_1A_n78A => DC_1A_n38A-n78A => DC_1A-7A_n38A-n78A.

Figure 3 Illustration on reasonable fall back paths of ENDC DC_1A-7A_n38A-n78A

Thus, we can get the reasonable fall back paths of DC_1A-7A_n38A-n78A from technical perspective as figure 3.

Proposal 1: those band combinations including band n7/7 and band n38/38 which can’t be deployed in reality can’t be considered as fall back band combinations.

In addition, it’s proposed not to introduce those band combinations including band n7/7 and band n38/38 which can’t be deployed in reality due to the technical issues.

Proposal 2: not to introduce those band combinations including band n7/7 and band n38/38 which can’t be deployed in reality due to the technical issues.

In total, since it’s clarified that the following band combinations in TS 38.101-3 are not used alone, it’s proposed to remove them from specification.

Table 1 Band combinations including band n7/7 and band n38/38 which can’t be deployed in reality

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n38A17,18 | N/A |
| DC_1A-38A_n7A17,18 | N/A |
| DC_3A-7A_n38A17,18 | N/A |
| DC_3A-38A_n7A17,18 | N/A |
| DC_7A-20A_n38A17,18 | N/A |
| DC_7A-38A_n3A17,18 | N/A |
| DC_7A-38A_n78A17,18 | N/A |
| DC_7A_n38A-n78A17,18 | N/A |
| DC_7A-38A_n3A-n78A10 | N/A |
| DC_1A-3A-7A-20A_n38A7,10 | N/A |

Proposal 3: it’s proposed to remove band combinations in Table 1 from specification.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Observation 1: Based on the current principle and procedure, RAN4 need to traverse and introduce all the possible fall back band combinations before studying the highest order band combination.

Observation 2: band n7/7 and band n38/38 are restricted as DL Scells from standard perspective once band n7/7 and band n38/38 are combined into one band combination in order to solve UL-DL interference issue.

Proposal 1: those band combinations including band n7/7 and band n38/38 which can’t be deployed in reality can’t be considered as fall back band combinations.

Proposal 2: not to introduce those band combinations including band n7/7 and band n38/38 which can’t be deployed in reality due to the technical issues.

Proposal 3: it’s proposed to remove band combinations in Table 1 from specification.

Table 1 Band combinations including band n7/7 and band n38/38 which can’t be deployed in reality

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n38A17,18 | N/A |
| DC_1A-38A_n7A17,18 | N/A |
| DC_3A-7A_n38A17,18 | N/A |
| DC_3A-38A_n7A17,18 | N/A |
| DC_7A-20A_n38A17,18 | N/A |
| DC_7A-38A_n3A17,18 | N/A |
| DC_7A-38A_n78A17,18 | N/A |
| DC_7A_n38A-n78A17,18 | N/A |
| DC_7A-38A_n3A-n78A10 | N/A |
| DC_1A-3A-7A-20A_n38A7,10 | N/A |

- References

[1]