# 3GPP TSG-RAN WG4 Meeting # 104-e						R4-220xyz

# Electronic Meeting, 15th  –26th August, 2022

Source:		Ericsson

Title:			IoT NTN coexisting simulation overview

Agenda item:		12.5.2

Document for:		Approval

# Introduction

In [1], the RF part objective is stated as below:

- Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

- Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

In this paper, we present our view on the IoT NTN coexisting overview.

# Discussion

In WID[1], the verification of co-existence of IoT NTN and TN is stated as one investigation:

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

To evaluate the re-using the existing coexistence results from NR NTN, the system parameter comparison between IoT NTN to NR NTN should be discussed firstly.

It is the NB-IoT NTN and Cat-M1 NTN to be speficied according to the WID [1]. The Cat-M1 is 1.4MHz LTE channel BW and thus difference between Cat-M1 NTN and NR NTN is the system bandwidth. In TR 38.863 [5], the system parameter for NTN is according to Table 6.2.2.1-4 as quoted below:

Table 6.2.2.1-4 [5]: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO1 | NOTE |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6.2.6.2 |  |
| NTN satellite Noise Figure in dB | See Table 2.3-3-1 |  |
| Handover margin | 3dB |  |

In [6], the NB-IoT and eMTC related parameter for link budget calculation is agreed and quoted as below:

Agreement:

The following assumptions are agreed for a common set of link budget parameters:

- UE power class (PC5=20 dBm)

- UE Noise Figure (NF=9 dB)

- Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e

  - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz

  - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL)

- Other losses

| Other Losses | GEO (35786 km) | LEO (1200 km) | LEO (600 km) |
| --- | --- | --- | --- |
| Scintillation losses | 2.2 | 2.2 | 2.2 |
| Atmospheric losses | 0.2 | 0.1 | 0.1 |
| Polarization loss | 3 | 3 | 3 |
| Shadow margin | 3 | 3 | 3 |

For eMTC, the number of the cat-M1 NTN UE is 6 if the 1 RB is allotted per UE and 24 if 3 tones to be allocated per UE.

For NB-IoT NTN UE, the same assumption could be reused for the TN NB-IoT for number of UE:

| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone |
| --- | --- |

For antenna configuration parameter, as the Cat-M1 and NB-IoT only have 1 antenna so the UE parameter needs to be changed:

Table 1: IoT NTN UE characteristics for system level simulations

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | omni-directional antenna |
| Rx Antenna gain | 0 dBi |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx transmit power | 200 mW (23 dBm) or PC5 (to be discussed) |
| Tx antenna gain | 0 dBi |

For the UE output power, RAN4 should discuss whether to specify PC3 or PC5.

Table 2: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) | For CAT-M1, 6 UEs and 1RBs per UE 24 UEs and 3 tones per UE For NB-IoT, 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | NOTE |

As the Cat-M1 has 1.4MHz channel and NB-IoT has 200kHz, the satellite parameter for channel bandwidth is to be updated as below, considering the EIRP density is the same and the max Tx power only scaled down from 5MHz NTN satellite power.

Table 3 : Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  | 1200 km |  | 600 km |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  | 40 dBW/MHz |  | 34 dBW/MHz |  |
| Satellite max TX power in dBm | BW (MHz) |  | 1.4MHz | 200kHz | 1.4MHz | 200kHz | 1.4MHz | 200kHz |
|  | SCS 15kHz |  | 44.53 + l0log(6/25) | 44.53+10log(1/25)0) | 46.53+ l0log(6/25) | 46.53++10log(1/25)) | 40.53+ l0log(6/25) | 40.53+ +10log(1/25) |
|  | SCS 30kHz |  | N/A | N/A | N/A | N/A | N/A | N/A |
| Channel bandwidth |  |  | 200kHz/1.4MHz |  | 200kHz/1.4MHz |  | 200kHz/1.4MHz |  |  |

Consider system parameter difference between IoT NTN and NR NTN for coexisting simulation in above tables.

For DL, though the Tx power is scaled down compared to 5MHz channel bandwidth, the PSD is not changed and thus the scenario involving the NTN DL coexisting simulation could be reused for IoT NTN. For UL, the number of IoT NTN UE will be different comparing to NR NTN. Thus, the scenario involving the NTN UL should be verified. In [5], the scenario is listed in Table 6.3.2-1 and the highlighted scenario could be further verified. This is to verify the UE ACLR and SAN ACS according to table below.

Scenario involving the NR NTN DL could be reused for IoT NTN

Verify the scenario involving the NR NTN UL, that is scenario 2, 4, 5 and 6 in TR 38.863.

Table 6.3.2-1 [5]: Selected option for each scenario

| Scenario | Aggressor system | Victim system | Environment | Contributing |
| --- | --- | --- | --- | --- |
| 1 | TN DL | NTN GEO DL | Urban | NTN UE ACS |
| 2 | TN UL | NTN GEO UL | Urban | NTN SAN ACS |
| 3 | NTN LEO-600 DL | TN DL | Rural | NTN SAN LEO ACLR |
|  | NTN GEO DL | TN DL | Rural | NTN SAN GEO ACLR |
| 4 | NTN GEO UL | TN UL | Urban | NTN UE ACLR |
| 5 | NTN GEO UL | TN DL | Rural | NTN UE ACLR |
| 61 | NR-TN DL | NTN LEO-600 UL | Rural 2 | NTN SAN ACS |
|  | NR-TN DL | NTN GEO UL | Rural 2 | NTN SAN ACS |
| 7 | HAPS DL | TN DL | Rural | HAPS ACLR |
| 8 | TN UL | HAPS UL | Rural | HAPS ACS |
| NOTE 1: Agreed representative case for Scenario 6. NOTE 2： The initial results suggested that the NR-NTN SAN would suffer more interference in urban deployment scenario. It is agreed that a more relevant environment for case 6 is a mixture of Urban and Rural environment (e.g., urban area with a 50km diameter inside a GEO beam with a 250km diameter). Further studies based on the mixed urban environment could be considered. As compromise, rural only scenario was then selected. |  |  |  |  |

# Conclusions

In this contribution, we present our view on the coeixsiting overview for IoT NTN with below observations and proposal:

- Consider system parameter difference between IoT NTN and NR NTN for coexisting simulation in above tables.

- Scenario involving the NR NTN DL could be reused for IoT NTN

Verify the scenario involving the NR NTN UL, that is scenario 2, 4, 5 and 6 in TR 38.863.

# References

- RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc

- 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)

- TS 38.101-5, NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- TS 36.101,Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

- TR 38.863, Non-terrestrial networks (NTN) related RF and co-existence aspects

- R1-2103962, Summary #3 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC