3GPP TSG-RAN WG4 Meeting #104-e	R4-2213311

Electronic Meeting, 15 – 26 Aug, 2022

Title:	R18 Discussion on 4Tx FWA

Source:	OPPO

Agenda item:	11.6.2

Document for:	Approval

- Introduction

In Rel-18, 4Tx for FWA/CPE/vehicle/industrial devices was one of the objectives for FR1 enhancement as below [1]. This paper will discuss these aspects.

Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4] - Investigate framework and architecture Example bands: - TDD bands: n41, n77/n78 - FDD bands: n1 - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 4Tx - First priority: 4x4 UL MIMO - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port - PA configuration assumption: - First priority: 4x23dBm - Second priority: 2x23dBm + 2x26dBm, 4x26dBm - UE power class - First priority: PC 1.5 - Second priority: PC2/PC3, and/or new power class if needed - Note 1: PC1.5 is only applicable for TDD bands - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97 - NOTE1: Requirements are specified with phase approach. Objectives with 1st priority are considered first. - NOTE2: It is assumed the devices are equipped with 4Rx antennas for band n1 requirement definition.

- Discussion

  - 4Tx architecture

From WID it can be seen that the 1st priority conditions are UE with 4x23 PAs to support PC1.5 at bands n41/n77/n78 and works under 4x4 UL MIMO mode as shown in figure 1 below.

Figure 1 Combination of 1st priority conditions in the WID

Observation 1:    1st priority of Rel-18 4Tx is to focus on UE with 4x23 PAs to support PC1.5 at bands n41/n77/n78 and works under 4x4 UL MIMO mode.

And 2nd priority includes many combinations of conditions including different PA configurations (23+23+26+26 and 4x26), different power classes (PC2 and PC3), different bands (n1), and TxD conditions. The detailed architecture can be seen in figure 2 for 4x23 PA configurations, and in figure 3 for 23+23+26+26 architecture, and in figure 4 for 4x26 architecture below. Each of the PA configurations may further support TxD.

Apparently to support all the 2nd priority conditions would make the discussion very complex, therefore, the phased approach has been adopted which in the 1st phase focus on Figure 1 architecture.

Observation 2:    2nd priority of Rel-18 4Tx includes many possible architectures which are quite complex and need to be further down selected.

Figure 2 2nd priority architecture with 4x23 architecture in the WID

Figure 3 2nd priority architecture with 23+23+26+26 architecture in the WID

Figure 4 2nd priority architecture with 4x26 architecture in the WID

It should be noticed that in the WID it has mentioned that UE with TxD condition to support same power class when fallback to single antenna port is not been considered in 1st priority. Then the TxD related discussion should leave for 2nd phase.

Observation 3:    TxD is necessary to keep 4x23 UE to achieve same power class in UL MIMO and single antenna port, however, it is not included in the 1st priority.

Then regarding the fallback behavior for 4x23 UE architecture, it might be much more complex than the 2Layer UL MIMO fallback since it may be configured with 4port, 2port or 1port (with different TPMI configurations in the spec), and supporting different Layers in each antenna port configuration for example:

- 3Layer with 4port configuration

- 2Layer with 4port configuration

- 1Layer with 4port configuration

- 2Layer with 2port configuration

- 1Layer with 2port configuration

- 1Layer with 1port configuration …

And if we further consider the TxD feature in each configuration, then it will become even more complex. Therefore, it would be better to consider the fallback behavior in the 2nd phase.

Proposal 1:         In 1st phase only discuss PC1.5 UE with 4x23 PA configuration at bands n41/n77/n78 and focus on 4Layer UL MIMO requirements.

Proposal 2:        Leave TxD and fall back behaviour in 2nd phase.

  - 4Layer UL MIMO requirements

Current RAN4 requirements only support 2Layer UL MIMO, and when it extends to 4Layer UL MIMO with single CC the necessary requirement updates (in red) are shown in Table 1 on a high level (including 2nd priority requirements in grey).

It can be seen some of the requirements only need small updates like OFF power, ON/OFF time mask, Frequency error, EVM, Carrier leakage, IBE and Tx intermodulation which are defined under single antenna connector or per layer. While some requirements need to SUM the power from 4 antenna connectors which may need more studies like MOP, MPR, Pcmax, etc.

Observation 4:    4Tx mainly impact the requirements which are defined under SUM of all Tx connectors, while requirements are defined under single Tx connector or per layer only need small updates.

Proposal 3:         Propose to review 4Layer UL MIMO requirements according to table 1.

Table 1 Summary of UL MIMO requirements from 2Layer to 4Layer

|  | 2 UL MIMO | 4 UL MIMO 1st priority |
| --- | --- | --- |
| Max power | - SUM from both connectors - 2Layer UL MIMO codebook - PC1.5, PC2, PC3 - ULFPTx with 2 ports - Fall back 1 port (1T or TxD) | - SUM from 4 connectors - 4Layer UL MIMO codebook - PC1.5, PC2, PC3 - ULFPTx with 4 ports - Fall back to 2 port and 1 port and combine with TxD |
| MPR | PC2/PC1.5 with 2T | PC1.5/PC2/PC3 with 4T |
| AMPR | Reuse single port AMPR | Reuse single port AMPR |
| Pcmax | Sum from both connectors | SUM from 4 connectors |
| Minimum power | Sum from both connectors | SUM from 4 connectors |
| OFF power | Each connector | Each connector |
| ON/OFF time mask | Each connector | Each connector |
| Power control | Sum from both connectors | SUM from 4 connectors |
| Frequency error | Each connector | Each connector |
| EVM | Per layer | Per layer |
| Carrier leakage | Each connector | Each connector |
| In band emission | Each connector | Each connector |
| Time alignment error | Frame timing difference between any two antenna connectors | Frame timing difference between any two antenna connectors |
| Coherent UL MIMO | Relative power and phase errors between different antenna connectors | Relative power and phase errors between any two different antenna connectors |
| OBW | Sum from both connectors | SUM from 4 connectors |
| OOBE | Sum from both connectors | SUM from 4 connectors |
| Spurious emission | Sum from both connectors | SUM from 4 connectors |
| Transmit intermodulation | Each connector | Each connector |

One point that isn’t mentioned in the WID is about the ULFPTx feature which was introduced in Rel-16 and is part of UL MIMO enhancement. In Rel-16 the enhanced ULFPTx requirements for 2Tx have been defined, and when 4Tx is introduced the ULFPTx also need to be enhanced with mode 0, mode1 and mode2 which already been supported by RAN1 and RAN2 specs.

Observation 5:    ULFPTx also need to be defined for 4Tx, but it is unclear whether it should be introduced in 1st priority or not.

Considering the complexity of ULFPTx experienced in Rel-16/Rel-17 together with TxD, it probably would be better to be included in 2nd priority and focus on the basic UL MIMO requirements in 1st phase.

Proposal 4:         Propose to define ULFPTx mode0/1/2 requirements for 4Tx in 2nd phase.

About the TPMI used for 4Layer MIMO, it can be as following table defined in 38.211, i.e. 121000010000100001.

Proposal 5:       Use    121000010000100001 as the codebook for 4Layer UL MIMO.

- Conclusion

In this paper, we discussed the 4Tx enhancements for CPE/FWA devices, and focus on UE architecture selection, UE requirement impacts comparing to 2Layer UL MIMO, and also the clarification of some features like TxD, ULFPTx, fallback behavior and codebooks used.

Observation 1:    1st priority of Rel-18 4Tx is to focus on UE with 4x23 PAs to support PC1.5 at bands n41/n77/n78 and works under 4x4 UL MIMO mode.

Observation 2:    2nd priority of Rel-18 4Tx includes many possible architectures which are quite complex and need to be further down selected.

Observation 3:    TxD is necessary to keep 4x23 UE to achieve same power class in UL MIMO and single antenna port, however, it is not included in the 1st priority.

Proposal 1:         In 1st phase only discuss PC1.5 UE with 4x23 PA configuration at bands n41/n77/n78 and focus on 4Layer UL MIMO requirements.

Proposal 2:        Leave TxD and fall back behaviour in 2nd phase.

Observation 4:    4Tx mainly impact the requirements which are defined under SUM of all Tx connectors, while requirements are defined under single Tx connector or per layer only need small updates.

Proposal 3:         Propose to review 4Layer UL MIMO requirements according to table 1.

Observation 5:    ULFPTx also need to be defined for 4Tx, but it is unclear whether it should be introduced in 1st priority or not.

Proposal 4:         Propose to define ULFPTx mode0/1/2 requirements for 4Tx in 2nd phase.

Proposal 5:       Use    121000010000100001 as the codebook for 4Layer UL MIMO.

- References

[1] RP-221496, New WID on Further RF requirements enhancement for NR frequency range 1 (FR1)