3GPP TSG-RAN WG4 Meeting #104-e 	       R4-2213337

# Electronic Meeting, 15 – 26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NTN SAN demodulation requirements

Agenda item: 	9.11.7.2.1

Document for:	Discussion

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present discussion of some open issues, proposals for simulation parameters of PUSCH demodulation requirements for NTN.

# Discussion

# PUSCH performance requirements

# NTN normal PUSCH requirements

Agreed way forward [2]:

Issue 2.1.1: SCS/CBW set for PUSCH requirements

NTN targets to provide the most critical services where few PRBs configuration is most commonly used for all SCS. A few PRBs allocation is also usually used in coverage limited scenario. Hence, a wide/full PRB allocation is not expected for PUSCH in NTN (e.g. voice, IoT, ...) as it is the case in current RAN1 discussion.

Observation 1 : A few PRBs allocation is the most commonly used in the considered NTN applications and it is considered in VoIP RAN1 discussion.

Observation 2 : Considering different CBW with the same PRB allocation have similar results.

Hence, we support 4 PRBs for all SCS allocation.

Proposal 1 : Consider 4 PRBs allocation for all SCS. (Support option 2)

# NTN UL timing adjustment requirements

Issue 2.1.2: Transform precoding

NTN is a coverage limited scenario and the PUSCH waveform is usually DFT-s-OFDM. Hence, it is preferable to follow in NTN UL timing adjustment same waveform as normal PUSCH.

Proposal 2: Support option 1 (Transform precoding for NTN UL timing adjustment requirements same as normal PUSCH), and we are open to discuss CP-OFDM only as the performance is quite similar.

# NTN PUSCH repetition type A requirements

Transform precoding for PUSCH repetition type A

Candidate options

- Option 1: Same as normal PUSCH

- Option 2: CP-OFDM only

Issue 2.1.3: Transform precoding for PUSCH repetition type A
NTN is a coverage limited scenario and the PUSCH waveform is usually DFT-s-OFDM. Hence, it is preferable to follow in NTN UL timing adjustment same waveform as normal PUSCH.

Proposal 3: Support option 1 (Transform precoding for PUSCH repetition type A same as normal PUSCH), and we are open to discuss CP-OFDM only as the performance is quite similar.

# PUCCH requirements

Issue PUCCH format 3/4, DMRS configuration:

Proposal 4: For PUCCH format 3/4, select the DMRS 1+0 configuration.

# Conclusion

This contribution discusses aspects related to NTN SAN demodulation requirements and has the following proposal and observations:

Observation 1 : A few PRBs allocation is the most commonly used in the considered NTN applications and it is considered in VoIP RAN1 discussion.

Observation 2 : Considering different CBW with the same PRB allocation have similar results.

Proposal 1 : Consider 4 PRBs allocation for all SCS. (Support option 2)

Proposal 2: Support option 1 (Transform precoding for NTN UL timing adjustment requirements same as normal PUSCH), and we are open to discuss CP-OFDM only as the performance is quite similar.

Proposal 3: Support option 1 (Transform precoding for PUSCH repetition type A same as normal PUSCH), and we are open to discuss CP-OFDM only as the performance is quite similar.

Proposal 4: For PUCCH format 3/4, select the DMRS 1+0 configuration.

# References

- RP-220208, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2210662 WF on NTN SAN demodulation requirements