3GPP TSG-RAN WG4 #104-e	R4-2213341

Electronic meeting, August 15 – August 26, 2022

Source: 	Ericsson

Title:  	Other test cases for HST FR2

Agenda Item:	9.7.3.6

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#103-e meeting, with outcome in terms of agreements and open issues on performance requirements captured in WFs [2].

In this contribution, we present our viewpoints and proposals on remaining issues in WFs [2].

# Disucssion

## Sub-Topic 1: Scope of RRM Performance Requirements

Issue 1-3: Necessity of Test Cases for Timing Related Requirement

Way Forward: - TC for Timing Related Requirement: - TC for gradual timing adjustment requirement for FR2 PC6 UE: - Option 1: Need new test case (similar as A.7.4.1.1) - Option 2: Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case. - Option 3: No new test case needed - TC for one shot large UL timing adjustment for FR2 PC6 UE - Define test case to verify one shot large UL timing adjustment requirement. - Option 1: New test defined in clause A.7.4.1 - Option-2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test. (The test shall verify the one shot large UL timing adjustment requirement and MAC-CE based TCI switch delay requirement.)

Proposal 1: Regarding TC for gradual timing adjustment requirement for FR2 PC6 UE, support Option 2: Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case.

Proposal 2: Regarding TC for one shot large UL timing adjustment for FR2 PC6 UE, support Option 2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test.

## Sub-Topic 2: RRM Test Configuration and Applicability

Issue 2-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements

Way Forward: - Applicability of Set-1 and Set-2 of enhanced RRM requirements: TC category (if confirmed in Sub-Topic 1); Applicability of set1 and set2 (either set1 with 2RX beams or set2 with 6RX beams, or both). TC for Cell Re-selection Requirement; TBD. TC for Re-establishment Delay Requirement; TBD. TC for Timing Related Requirement; N/A (not differentiated by set1 and set2). TC for MAC-CE based TCI State Switch Delay Requirement; TBD (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference). TC for L3 measurement; TBD. TC for L1-RSRP/SINR Measurement Requirement; TBD.

Proposal 3: General rule is adopting set2 in L3 relevant tests and adopting set1 in L1 relevant tests.

# Conclustion

Proposal 1: Regarding TC for gradual timing adjustment requirement for FR2 PC6 UE, support Option 2: Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case.

Proposal 2: Regarding TC for one shot large UL timing adjustment for FR2 PC6 UE, support Option 2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test.

Proposal 3: General rule is adopting set2 in L3 relevant tests and adopting set1 in L1 relevant tests.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2210609 “WF on RRM performance requirement for FR2 HST”, Samsung