3GPP TSG-RAN WG4 Meeting #104-e	R4-2213518

Electronic Meeting, 15 – 26 August, 2022

Title: 	Discussion on other issues for NTN RRM

Source: 	Huawei, HiSilicon

Agenda item:	9.11.5.2

Document for:	Discussion

Introduction

RRM requirements for NTN are discussed in RAN4#103-e, and the outcomes are captured in WF [1]. Based on [1], besides measurement the following issues need to be further discussed.

- Paging interruption

- Re-establishment

- Re-direction

In this paper we will provide our views on above open issues for NTN RRM.

Discussion

Paging interruption

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. Where, If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2; otherwise, if the target cell is unknown, then K = 5. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2; otherwise, if the target cell is unknown, then K = 5. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5.

Based on the requirements in clause 4.2C.2.5 for maximum interruption in paging reception, UE is required to re-select and synchronize to the new serving cell within K SMTC periods. In particular K=5 is defined for unknown case, based on the assumption of 3 SMTC periods for cell search and 2 SMTC periods for sync.

The assumption of 3 SMTC periods for cell search is re-used from HO requirements, where Tsearch is 3 SMTC period for unknown case.

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

However, the conditions for cell reselection and HO are different. One difference is that 3 SMTC periods for cell search in unknown HO is based on >= -2dB Es/Iot, but that is not defined for unknown cell reselection. Most importantly, for unknown HO, the target cell is still configured by the source cell, i.e. UE knows which cell and carrier frequency it should search for, but for unknown reselection UE has no idea which carrier and cell to search, and in case of NTN, UE may also have no ephemeris information for the neighbour cells.

Based on above, we think it could be difficult for UE to ensure that it can find a suitable cell for reselection within 3 SMTC periods, which was unknown before it starts the cell search, e.g. UE may need to search over multiple carrier frequencies or with multiple ephemeris information. Therefore, we suggest to remove the requirements for unknown case, which anyway happens rarely in the field.

Proposal 1: Remove the requirements for unknown case for paging interruption.

Re-establishment

In the latest 38.133 the exact requirements for RRC re-establishment is TBD. RAN4 needs to discuss and agree on the values to complete the requirements.

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | TBD | TBD |
| < -8 | FR1 | N/A | TBD |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

For known cell case, we understand UE will measure only one target cell based on the ephemeris information, so TN requirements can be re-used, i.e. the scaling factor for multiple SMTC and multiple LEO satellites are not applicable.

For unknown case and serving cell Es/Iot is ≥ -8dB, UE will search for the target frequency based on the SMTC configured by the serving cell and the ephemeris information provided by the serving cell. Since there are multiple SMTCs and satellites to be searched and measured, the scaling factor for multiple SMTC and multiple LEO satellites are applicable. The exact number of SMTC depends on the exact configuration for the MO. For simplicity, we suggest to apply the scaling factor Kmulti_SMTC on top of the current TN requirements.

For unknown case and serving cell Es/Iot is < -8dB, UE will search the target frequency ignoring the SMTC configured by the serving cell. The question is whether the ephemeris information provided by the serving cell is valid or not. We suggest to assume the ephemeris are still valid, as otherwise UE has to search for the carrier in the similar way as initial cell selection, and RAN4 has not defined any requirement for cell selection. Valid ephemeris information can be defined as condition for the requirements. Similar as the above case with serving cell Es/Iot is ≥ -8dB, the exact time needed depends on how many candidate satellites UE will search, and we suggest to assume UE would search for at most [8] satellites per carrier, so the time required will be [8] times of the TN requirements.

Proposal 2: Define NTN re-establishment requirements as in Table 1 and Table 2.

Table 1: NTN re-establishment requirements for intra-frequency

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | Kmulti_SMTC * MAX (800 ms, 10 x TSMTC) |
| < -8 | FR1 | N/A | [6400]Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. |  |  |  |

Table 2: NTN re-establishment requirements for inter-frequency

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | [6400]Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4. |  |  |  |

Re-direction

In the latest 38.133 the exact requirements for RRC re-direction is TBD. RAN4 needs to discuss and agree on the values to complete the requirements.

Table 6.2C.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | TBD |
| Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

For re-direction, UE will search the target frequency based on the SMTC configured by the serving cell in RRCRelease message. Similar as the re-establishment, we suggest to apply the scaling factor Kmulti_SMTC on top of the current TN requirements.

Proposal 3: Define NTN re-direction requirements as in Table 3.

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | K_satellite * MAX (680 ms, 11 x Trs) |
| Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: K_satellite is defined in clause 9.3C.4. |  |

Conclusions

In this paper we provided our views on open issues for NTN RRM.

Proposal 1: Remove the requirements for unknown case for paging interruption.

Proposal 2: Define NTN re-establishment requirements as in Table 1 and Table 2.

Table 1: NTN re-establishment requirements for intra-frequency

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | Kmulti_SMTC * MAX (800 ms, 10 x TSMTC) |
| < -8 | FR1 | N/A | [6400]Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. |  |  |  |

Table 2: NTN re-establishment requirements for inter-frequency

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | [6400]Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4. |  |  |  |

Proposal 3: Define NTN re-direction requirements as in Table 3.

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | K_satellite * MAX (680 ms, 11 x Trs) |
| Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: K_satellite is defined in clause 9.3C.4. |  |

Reference

- R4-2210610, WF on NR NTN RRM requirements, Moderator (Qualcomm Incorporated)