3GPP TSG-RAN WG4 Meeting #104-e                 	R4- 2213776

Electronic Meeting, August 15 – August 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	General Discussion for PDC Measurement Period and Accuracy Performance Requirements

Agenda item:	9.21.2.1

Document for:	Discussion

Introduction

Agreement:

RAN4 will introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17

From RAN4#103e WF [1]: we have the following agreements

In this paper we will further discuss the PDC measurement period and accuracy performance requirements

Discussion

SRS type for usagePDC-r17

Consider the POS-SRS.1/2 are used for the positioning PRS UE Rx-Tx measurement test case, we think the new SRS configuration for usagePDC-r17 can reuse the current configutation for POS-SRS.1/2 with only changing the usage and set b-SRS, b-hop to zero.  Table 1 and table 2 shows the proposed SRS configurations for SCS 15kHz and 30kHz.

|  | PDC-SRS.1 |
| --- | --- |
| Field | Value |
| c-SRS | Same as NRB,c in the test case |
| b-SRS | 0 |
| b-hop | 0 |
| groupOrSequenceHopping | neither |
| freqDomainPosition | 0 |
| freqDomainShift | 0 |
| pathlossReferenceRS ssb-Index | 0 |
| usage | usagePDC-r17 |
| startPosition | 0 |
| nrofSymbols | 4 |
| repetitionFactor | n1 |
| transmissionComb | n4 |
| combOffset-n2 | 0 |
| cyclicShift-n2 | 0 |
| nrofSRS-Ports | port1 |
| resourceType | Periodic |
| periodicityAndOffset-p | Sl160, 2 |

Table 1 usagePDC-r17 SRS Configuration for SCS=15kHz

|  | PDC-SRS.2 |
| --- | --- |
| Field | Value |
| c-SRS | Same as NRB,c in the test case |
| b-SRS | 0 |
| b-hop | 0 |
| groupOrSequenceHopping | neither |
| freqDomainPosition | 0 |
| freqDomainShift | 0 |
| pathlossReferenceRS ssb-Index | 0 |
| usage | usagePDC-r17 |
| startPosition | 0 |
| nrofSymbols | 4 |
| repetitionFactor | n1 |
| transmissionComb | n4 |
| combOffset-n2 | 0 |
| cyclicShift-n2 | 0 |
| nrofSRS-Ports | port1 |
| resourceType | Periodic |
| periodicityAndOffset-p | Sl320, 40 |

Table 2 usagePDC-r17 SRS Configuration for SCS=30kHz

Consider table 1 and table 2 for usagePDC-r17 SRS Configurations.

Conclusion

We have the following proposal:

- Consider table 1 and table 2 for usagePDC-r17 SRS Configurations.

- References

- R4-2210618 WF on NR_IIOT_URLLC_enh, Nokia, Nokia Shanghai Bell