3GPP TSG-RAN WG4 Meeting # 104e	R4-2213809

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Discussions on open issues for SDR requirements for FR2-2

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.2.3

Document for:	Discussion

Background

As per [1], RAN 4 agreed to define FR2-2 SDR requirements and many issues are discussed based on the simulation results. In this paper, we provide our simulation results.

Discussions

It is noted that we capture our simulation results in appendix.

Scenarios

In last meeting, we reached following initial agreements for SDR test scenarios:

- Define SDR requirements for following scenarios: - SA FR2-2 - FR1 (CA or single CC) CA with FR2-2 (CA or single CC) - Further discuss which cases can be covered considering limited number of HARQ processes. For example, one such case could be FR1 FDD + FR2-2 CA. - FR1 (CA or single CC) DC with FR2-2 (CA or single CC)

Based on the latest version of TS 38.101-1, RF team didn’t specify any bandwidth combination for FR1+FR2-2 DC and any bandwidth combination for FR2-2 CA. Hence we propose to define SDR requirements for following scenarios:

  - SA FR2-2

  - FR1 (CA or single CC) CA with FR2-2 single CC

Proposal 1: Define FR2-2 SDR requirements for following scenarios:

  - SA FR2-2

  - FR1 (CA or single CC) CA with FR2-2 single CC

Schedule and feedback patterns for FR1+FR2-2 CA scenario

PDSCH CA requirements are defined based on the assumptions all ACK/NACK are transmitted on PCell CC. However, for SDR requirements, the data rate will be lower if ACK/NACK are transmitted on PCell CC since UE should wait for UL slot of Pcell to transmit ACK/NACK. Hence we suggest that all ACK/NCK are transmitted on separately rather than on Pcell.

Proposal 2: For FR1+FR2-2 CA scenario, all ACK/NACK should be transmitted separately on each CC

Rank

- Define SDR requirements for Rank1. - Decide based on simulation results for Rank2.

One issue is number of MIMO layers. Based on our understanding, rank1 and rank2 are typical configurations when UE are operating in FR2-2. Also, RAN 4 defined rank 1 and rank 2 for FR2 SDR requirements, we proposed to cover both rank 1 and rank 2.

Proposal 3: Define FR2-2 SDR requirements for both rank 1 and rank 2.

SCS

- Option 1: Only 120kHz - Option 2: 120kHz and 480kHz

We propose to only consider 120kHz SCS with following reasons:

- 480kHz and 960kHz are optional for UE and 120kHz is mandatory for UE

- 120kHz SCS is more sensitive to ICI caused by phase noise. RAN 4 should consider minimum requirements

- Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz, it is meaningless to test SDR requirements with such low maximum DL testable SNR.

Observation 1: 480kHz and 960kHz SCS are optional for UE and 120kHz is mandatory for UE

Observation 2: 120 kHz SCS is more sensitive to ICI caused by phase noise which is related to minimum requirements.

Observation 3: Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz. SDR is untestable under such quite low SNR.

Proposal 4: Only consider 120kHz SCS for SDR test

PN Compensation

- Option 1: Same as PDSCH test cases based on outcome of Issue 2-1-12 - Option 2: CPE compensation only

CPE compensation is basic algorithm when UE is operating on FR2-2 band which is corresponding to minimum performance requirements. Since RAN4 always consider minimum requirements, we propose to only consider PN compensation.

Proposal 5: Only consider CPE compensation.

Whether to define separate MCS to SNR mapping tables for different CBWs

- Option 1: Yes - Option 2: No - Option 3: Decide based on simulation results.

Based on our simulation results, when MCS is larger than 20, there is obvious performance difference between cases with 100MHz and 400MHz. We think the reason is that large bandwidth will cause more phase noise compared to small bandwidth. Hence, we propose to define separate MCS to SNR mapping tables for different CBWs.

Observation 4: Due to the more phase noise impact, performance for cases with 400MHz CBW is worse than 100MHz CBW when MCS is larger than 20.

Proposal 6: Define separate MCS to SNR mapping tables for different CBWs

Changes to Maximum MCS for FR2-2 64QAM, Rank1

Decide based on simulation results in the next meeting - Option 1: MCS24 for Scaling factor 1. - Option 2: MCS27 for Scaling factor 1. - Other Options not precluded.

Based on our simulation results, 85% of peak throughput can’t be reached when MCS is larger than 25 and we find that target SNR for MCS25 is quite high (26.5dB), we propose to configure maximum MCS to MCS24.

Proposal 7: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 1.

Changes to Maximum MCS for FR2-2 64QAM, Rank2 (if defined)

Decide based on simulation results in the next meeting - Option 1: MCS22 for Scaling factors 1 and 0.8. - Other Options not precluded.

Based on our simulation results, 85% of peak throughput can’t be reached when MCS is larger than 25 and we find that target SNR for MCS25 is quite high (25.6dB), we propose to configure maximum MCS to MCS24.

Proposal 8: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 2.

Minimum MCS for FR2-2 MCS to SNR mapping

Decide based on simulation results in the next meeting - Option 1: MCS 6 - Other Options not precluded.

Minimum MCS is MCS13 in MCS to SNR mapping table in FR2 SDR requirements definition. However, in FR2-2, minimum MCS should be lower since the maximum DL testable SNR is quite low. In FR2-2, maximum DL testable SNR is 7.7dB for 100MHz CBW and -0.6dB for 400MHz CBW.  To consider the worse case -0.6dB, we propose to set minimum MCS for FR2-2 MCS to SNR mapping to 0.

Proposal 9: Set MCS0 for minimum MCS for FR2-2 MCS to SNR mapping.

Conclusion

In this paper we provide our views on SDR requirements definition. The observations and proposals are:

Proposal 1: Define FR2-2 SDR requirements for following scenarios:

  - SA FR2-2

  - FR1 (CA or single CC) CA with FR2-2 single CC

Proposal 2: For FR1+FR2-2 CA scenario, all ACK/NACK should be transmitted separately on each CC

Proposal 3: Define FR2-2 SDR requirements for both rank 1 and rank 2.

Observation 1: 480kHz and 960kHz SCS are optional for UE and 120kHz is mandatory for UE

Observation 2: 120 kHz SCS is more sensitive to ICI caused by phase noise which is related to minimum requirements.

Observation 3: Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz. SDR is untestable under such quite low SNR.

Proposal 4: Only consider 120kHz SCS for SDR test

Proposal 5: Only consider CPE compensation.

Observation 4: Due to the more phase noise impact, performance for cases with 400MHz CBW is worse than 100MHz CBW when MCS is larger than 20.

Proposal 6: Define separate MCS to SNR mapping tables for different CBWs

Proposal 7: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 1.

Proposal 8: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 2.

Proposal 9: Set MCS0 for minimum MCS for FR2-2 MCS to SNR mapping.

Appendix

Table 2-2: Simulation results for SNR@85% of max TP

| MCS | 120kHz/100MHz |  | MCS | 120kHz/400MHz |  |
| --- | --- | --- | --- | --- | --- |
|  | Rank 1 | Rank 2 |  | Rank 1 | Rank 2 |
| 0 | -7.3 | -4.2 | 0 | -7.0 | -4.3 |
| 1 | -6.3 | -3.2 | 1 | -6.3 | -3.3 |
| 2 | -5.3 | -2.3 | 2 | -5.3 | -2.3 |
| 3 | -4.3 | -1.2 | 3 | -4.3 | -1.3 |
| 4 | -3.3 | -0.3 | 4 | -3.3 | -0.3 |
| 5 | -2.3 | 0.7 | 5 | -2.3 | 0.7 |
| 6 | -1.3 | 1.7 | 6 | -1.3 | 1.7 |
| 7 | -0.3 | 2.7 | 7 | -0.3 | 2.7 |
| 8 | 0.7 | 3.7 | 8 | 0.7 | 3.7 |
| 9 | 1.6 | 4.7 | 9 | 1.6 | 4.7 |
| 10 | 1.7 | 4.7 | 10 | 1.7 | 4.7 |
| 11 | 2.7 | 5.7 | 11 | 2.7 | 5.7 |
| 12 | 3.7 | 6.7 | 12 | 3.7 | 6.7 |
| 13 | 4.7 | 7.7 | 13 | 4.7 | 7.7 |
| 14 | 5.7 | 8.7 | 14 | 5.7 | 8.7 |
| 15 | 6.7 | 9.7 | 15 | 6.7 | 9.7 |
| 16 | 7.7 | 10.7 | 16 | 7.7 | 10.7 |
| 17 | 7.8 | 11.3 | 17 | 8.0 | 11.2 |
| 18 | 8.7 | 11.7 | 18 | 8.7 | 11.7 |
| 19 | 9.7 | 12.9 | 19 | 9.8 | 13.0 |
| 20 | 10.7 | 14.4 | 20 | 11.4 | 14.6 |
| 21 | 11.8 | 15.6 | 21 | 12.5 | 15.7 |
| 22 | 13.7 | 16.8 | 22 | 13.9 | 17.2 |
| 23 | 15.6 | 19.0 | 23 | 16.4 | 19.8 |
| 24 | 17.2 | 21.3 | 24 | 18.5 | 22.1 |
| 25 | 20.9 | 26.8 | 25 | 25.6 | Inf |
| 26 | Inf | Inf | 26 | Inf | Inf |
| 27 | Inf | Inf | 27 | Inf | Inf |
| 28 | Inf | Inf | 28 | Inf | Inf |

Reference

[1]  R4-2210665 WF on UE demodulation performance requirements definition for 52.6 - 71 GHz