3GPP TSG-RAN WG4 Meeting #104-e	R4-2213841

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Simulation results on UE FeMIMO demod HST-SFN

Source: 	Huawei, HiSilicon

Agenda item:	9.17.4.1.1

Document for:	Information

Background

During RAN4#103-e meeting, WF [1] on demodulation requirement for Enhancement on HST-SFN deployment was approved. In this contribution, we provide our updated simulation results about the FeMIMO HST-SFN deployment demodulation requirements.

Simulation

PDSCH requirement for SFN scheme A with Single Carrier

Here we provide the updated simulation results for PDSCH requirement for SFN scheme A with Single Carrier for alignment, as shown in Table 2.1-2 and Figure 2.1-1, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PDSCH requirement for SFN scheme A with Single Carrier

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2, 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot patterns |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Table 2.1-2 Simulation results for PDSCH requirement for SFN scheme A with Single Carrier

| Case Number | Duplex | Maximum Doppler (Hz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | FDD | 870 | 2x2 | 14.86 |
| 2 | FDD | 870 | 2x4 | 12.35 |
| 3 | FDD | 972 | 2x2 | N/A |
| 4 | FDD | 972 | 2x4 | N/A |
| 5 | TDD | 1667 | 2x2 | 14.80 |
| 6 | TDD | 1667 | 2x4 | 12.32 |

Figure 2.1-1 Simulation results for PDSCH requirement for SFN scheme A with Single Carrier

Maximum Doppler 972Hz for 15kHz SCS cannot achieve the maximum throughput for HST SFN scheme A.

PDSCH requirement for SFN scheme B with Single Carrier

Here we provide the initial simulation results for PDSCH requirement for SFN scheme B with Single Carrier for alignment, as shown in Table 2.2-2 and Figure 2.2-1, based on simulation assumption as shown in Table 2.2-1.

Table 2.2-1 Simulation assumption for PDSCH requirement for SFN scheme B with Single Carrier

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2, 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot patterns |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Table 2.2-2 Simulation results for PDSCH requirement for SFN scheme B with Single Carrier

| Case Number | Duplex | Maximum Doppler (Hz) | Pre-compensation error at the UE side (Hz) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2x2 | 2x4 |
| 1 | FDD | 870 | 0 (0 ppm) | 13.22 | 10.42 |
| 2 | FDD | 870 | 188 (0.1 ppm) | 14.33 | 11.82 |
| 3 | FDD | 870 | 376 (0.2 ppm) | 15.34 | 12.81 |
| 4 | TDD | 1667 | 0 (0 ppm) | 13.07 | 10.30 |
| 5 | TDD | 1667 | 360 (0.1 ppm) | 14.57 | 12.08 |
| 6 | TDD | 1667 | 720 (0.2 ppm) | 15.88 | 13.35 |

Figure 2.2-1 Simulation results for PDSCH requirement for SFN scheme B with Single Carrier

Conclusion

In this contribution, we provide our updated simulation results on FeMIMO HST-SFN demodulation requirements. Our observations are:

- Maximum Doppler 972Hz for 15kHz SCS cannot achieve the maximum throughput for HST SFN scheme A.

Reference

R4-2210670, WF on demodulation requirement for Enhancement on HST-SFN deployment, RAN4#103-e, Intel Corporation