3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214016

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR |  | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113: updates reflecting modifications in IEC 61000-4-3:2020 for the upper frequency range of the RI test, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Recently, the IEC 61000-4-3 specification has been updated (IEC 61000-4-3:2020), where one of the modifications introduced was the removal of the upper frequency limit of the RI requirements. RAN4 EMC specifications are referring to the 61000-4-3 and reusing the upper frequency limit for the RI testing up to 6 GHz, so far. Therefore based on the IEC 61000-4-3:2020 content, the related 6GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications. Related discussion paper was provided in R4-2214014, where feedback from previous RAN4 meeting was addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of the upper frequency limit of 6 GHz for the Radiated Immunity testing, with consideration of potantial capability limitations of the test labs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect upper frequency limit of 6 GHz and misleading testing limitation would be kept for the RI test, not aligned with the external IEC 61000-4-3:2020 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 9.2, 9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.175, TS 38.114, TS 38.124 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified sections ------------------------------

## 7.2	Immunity

Table 7.2-1: Immunity requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | BS equipment | Ancillary equipment | clause in the present document | standard |
| RF electromagnetic field | Enclosure | applicable | applicable | 9.2 | IEC 61000-4-3 [18] |
| Electrostatic discharge | Enclosure | applicable | applicable | 9.3 | IEC 61000-4-2 [17] |
| Fast transients common mode | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.4 | IEC 61000-4-4 [19] |
| RF common mode 0.15 - 80 MHz | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.5 | IEC 61000-4-6 [21] |
| Voltage dips and interruptions | AC mains power input ports | applicable | applicable | 9.6 | IEC 61000-4-11 [22] |
| Surges, common and differential mode | AC power input ports and telecommunications port | applicable | applicable | 9.7 | IEC 61000-4-5 [20] |

------------------------------ Next modified section ------------------------------

## 9.2	RF electromagnetic field

The test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

------------------------------ Next modified section ------------------------------

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [18], which specified test methodology based on anechoic chamber. The use of reverberation chamber test method according to IEC 61000-4-21 [29], clause 6.1 and Annex D as alternative method is allowed.

For transmitters, receivers and transceivers the following requirements shall apply:

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed at fixed frequency test steps starting from 80 MHz, as defined in IEC 61000-4-3 [18]; with the exception of the exclusion band for receivers (see clause 4.4);

NOTE: 	In case of BS type 2-O testing and test equipement limitations not allowing to test above 6 GHz, such test capability limitation shall be recorded in the test report, including information on the highest frequency tested.

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

-	For the test method in accordance with IEC 61000-4-3 [18], the spatial exclusion zone can be chosen to protect the base station receiver. For the frequency arrange above 690 MHz (according to the test method in ETSI EN 301 489-50 [28]), the EMC RF electromagnetic field immunity requirement applies on the non-radiating faces of the BS type 1-O or BS type 2-O, as depicted on figure 9.2.2-1.

NOTE:	Depending on the BS implementation, application of the spatial exclusion to all radiating faces of the BS may not allow proper execution of the RI testing. In such cases, to protect the BS type 1-O or BS type 2-O receiver(s), exclusion bands shall be considered, as in table 4.4.2-1.

Figure 9.2.2-1: EMC RF electromagnetic field immunity requirement testing directions for BS type 1-O and BS type 2-O (horizontal plane depicted) with the spatial exclusion zone applied

------------------------------ End of modified section ------------------------------